3GPP TSG-RAN WG4 Meeting #88-bis	R4-1812331

Chengdu, China, 8 – 12 October 2018

Title: 	Discussion on the differences between LTE MIMO OTA and NR MIMO OTA

Source: 	Huawei, HiSilicon

Agenda item:	10.4.1

Document for:	Discussion

# Background

The new study item (SI) on NR MIMO OTA was approved in 3GPP RAN plenary meeting in La Jolla, USA, in June 2018 [1]. The LTE MIMO OTA is specified in [2], and this contribution discusses the main differences between NR MIMO OTA and LTE MIMO OTA.

# Discussion

There are several challenges in “extrapolating” the MIMO OTA to cover NR requirements. The NR MIMO OTA differs from LTE MIMO OTA especially when using higher frequencies (FR2) due to the shorter wavelength, more directive antennas at Tx and Rx, as well as wider bandwidth. NR frequency range is very wide: from 0.5 to 100 GHz. Prioritization of the frequency bands would help the development of the MIMO OTA test methods

Channel model is one of the most important things in the very beginning of the test system design and requirement specification. Since NR UE will utilize multiple antenna reception, spatial channel model is necessary. It should be discussed within this SI whether 2D model is sufficient. 3D model might be needed, e.g., for 2D array testing. These models could be, e.g., the clustered delay line models (CDL) from TR38.901. These models allow antenna independent testing, i.e. the antenna under test together with the spatial channel provides the complete radio channel (similar to LTE MIMO OTA). There may be more antennas in NR than in LTE. The higher number of UE antennas leads to higher spatial resolution of the receiver. Thus higher number of probe antennas is needed. The electrical volume of a DUT depends on the physical size and the carrier frequency. Physical handset size is not changed according to the frequency. Therefore, the higher the frequency the larger the electrical size of the device. Since the electrical volume is larger, the required quiet zone is larger as well, which means that higher number of probe antennas are needed. Fortunately, the narrow antenna beamwidth at gNB will reduce the angle spread and thus allow simplification of the test system.

Oxygen absorption within the anechoic chamber is insignificant. However, oxygen absorption may affect channel model. The delayed echoes at around 60 GHz are attenuated due to the oxygen absorption. This is taken into account in the additional feature called “oxygen absorption” in TR38.901, but it is not usually included in the CDL model.

Link budget of the test system may be challenging in higher frequencies due to the limited availability of mmW power amplifiers and higher path loss. Directive probe antennas may be used to compensate this problem, but they should not be too directive. Impact of BS antenna beamforming on the channel seen by the UE is significant. For example rich scattering may become sparse when Tx allocated a single narrow beam towards a strong cluster of the propagation channel.

Measurement uncertainty will most probably increase according to frequency due to the smaller wavelength (physical uncertainty of device or antenna location causes higher error, the phase noise may be higher, and SNR worse due to the link budget.

Joint Tx-Rx beamtracking and beamforming would require bi-directional test, but it might be out of scope in the early phase of NR MIMO OTA. Moreover, according to the SID [1], for FR2, a study of the extension of Rel-15 RRM tests [TS38.133] to include dynamic geometry is needed. Dynamic geometry means dynamic channel model (i.e. drifting AoA). This should be done only if a feasible solution is found.

The complexity of the test system should be kept reasonable. Simplifications on channel model and test configuration are welcome, but the impact of the simplifications should be understood well.

# Comparison of the LTE and NR MIMO OTA

Table below compares different aspects for LTE, NR FR1, and NR FR2. Note: the figures are not exact. They are rough numbers for clarifying the differences.

|  | LTE MIMO OTA | NR FR1 MIMO OTA | NR FR2 MIMO OTA |
| --- | --- | --- | --- |
| Frequency range | 0.7 – 6 GHz | 0.45 – 6 GHz | 24.25 – 52.6 GHz |
| Bandwidth | 20 MHz | 100 MHz | 400 MHz |
| Rank | 2 | 4 | 2 |
| Number of antenna elements in DUT | 2 – 4 | 2 – 4 | >>2 |
| Typical electrical size of a handheld DUT (required quiet zone) | ~1 | ~1 | ~10 |
| Number of significant paths seen by the UE | 9 – 12 | 12 – 24 | 3 – 12 |
| Oxygen absorption | no impact | no impact | minor impact |
| Spatial channel | 2D | 2D/3D | 3D |
| Suitable channel model for MIMO OTA static geometry | SCME | TR38.901 CDL | TR38.901 CDL |
| Dynamic model | N/A | N/A | Necessary for MIMO throughput under dynamic geometry |
| FoM | MIMO throughput under static geometry | MIMO throughput under static geometry | MIMO throughput under static geometry 1st priority, MIMO throughput under dynamic geometry 2nd priority |

# Conclusion / Proposals

In this contribution, we discussed the differences between LTE and NR MIMO OTA. The main differences are coming from the frequency band, bandwidth, and possible antenna solutions of BS and UE.

# Reference

- CATR, OPPO, Samsung, “New SID on Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs”, RP-181402, 3GPP TSG-RAN Meeting #80, La Jolla, USA, June 11 - 14, 2018.

- “Verification of radiated multi-antenna reception performance of User Equipment (UE)”, 3GPP TR37.977, V14.6.0 (2018-06).