3GPP TSG-RAN WG4 Meeting #88-Bis                                  R4-1812339

Chengdu, China, 08 - 12 October 2018

Agenda item:	7.13.2.2

Source: 	ZTE

Title: 	On NR BS PUSCH demodulation performance requirements

Document for:	Discussion

Introduction

During RAN4#88 meeting in August two WFs [1] [2] relevant for NR BS PUSCH demodulation performance requirements and simulation assumptions for alignment purpose have been approved. This contribution presents simulation results.

Simulation assumptions and simulation results

- Simulation assumptions

The simulation assumptions considering [2] are summarized in the table below.

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1, 2 | 1 |
| Number of Rx | 2, 4, 8 | 1, 2 |
| Number of layers | 1, 2 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | - |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1, 1+1 | 1 front-loaded |
| symbols length | 14 | 7 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type B |
| Frequency domain resource | full RB allocation of the applicable BW | full RB allocation of the applicable BW |
| MCS index | 2, 16, 20 | 2, 16, 20 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | TDL-A 30ns 10Hz | AWGN |
| SCS and BW | 15kHz: 10 30kHz: 40 | 60kHz: 100 120kHz: 100 |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS =1 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |
| Phase noise | Not modelled | Not modelled |

- Initial simulation results

The simulation results for CP-OFDM (TX: 1, RX: 2) are summarized in the table below, and FR2 symbols length is set to 7.

| Part of simulation assumptions |  |  |  |  | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
| CP-OFDM (TX: 1, RX: 2) | FR1 (TDL-A 30ns 10Hz) | 15kHz, 10MHz | MCS2 | DMRS 1 | -4.6 |
|  |  |  |  | DMRS 1+1 | -4.6 |
|  |  |  | MCS16 | DMRS 1 | 6.9 |
|  |  |  |  | DMRS 1+1 | 6.9 |
|  |  |  | MCS20 | DMRS 1 | 9.6 |
|  |  |  |  | DMRS 1+1 | 9.6 |
|  |  | 30kHz, 40MHz | MCS2 | DMRS 1 | -4.7 |
|  |  |  |  | DMRS 1+1 | -4.7 |
|  |  |  | MCS16 | DMRS 1 | 7.64 |
|  |  |  |  | DMRS 1+1 | 7.64 |
|  |  |  | MCS20 | DMRS 1 | 9.7 |
|  |  |  |  | DMRS 1+1 | 9.7 |
|  | FR2 (AWGN, symbols length = 7) | 60kHz, 100MHz | MCS2 | DMRS 1 | TBD |
|  |  |  | MCS16 | DMRS 1 | TBD |
|  |  |  | MCS20 | DMRS 1 | TBD |
|  |  | 120kHz, 100MHz | MCS2 | DMRS 1 | -2.4 |
|  |  |  | MCS16 | DMRS 1 | 9.4 |
|  |  |  | MCS20 | DMRS 1 | 12.5 |

Conclusions

The PUSCH simulation results for CP-OFDM (TX: 1, RX: 2) with symbol length 7 are summarized in the table.

# References

- R4-1811726 Way forward on General part of NR BS demodulation performance, August 2018.

- R4-1811694 WF on NR PUSCH demodulation requirements, August 2018.