3GPP RAN WG4 Meeting #88bis 	R4-1812395

# Chengdu, China, October 8 - 12, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | DRAFT | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-2: Introducing EIS spherical coverage requirements for PC3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The REFSENS power level requirement for PC3 in FR2, as captured in Version 15.2.0 of the specification, does not include the EIS spherical coverage. Additionally, the same requiement doesnot consider the impact of supporting multiple FR2 bands on PC3 UEs. For operators who own spectrum which spans multiple 3GPP FR2 bands and for device vendors who target global device SKUs the lack of 3GPP requirements for PC3 UE supporting multiple FR2 bands may pose a serious risk from the certification perspective. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the technical analysis provided in the accompanying discussion paper (R4-1810065), this CR proposes to define relaxations for peak EIS and EIS spherical coverage limits, labeled ΔTEMBP and ΔTEMBS, respectively. These relaxations are indexed by the bands which the device supports and are defined in a band-specific manner. It is noted that the EIS spherical coverage requirement is only applicable to UEs which do not support beam correspondence as defined in Section 6.2.5 of TS 38.101-2. For consistency, the CR also clarifies that the existing peak EIS limits are applicable to UEs which support a single band in FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EIS spherical coverage requirements are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-2 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<<start of change>>>

#### 7.3.2.3	Reference sensitivity power level for power class 3

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal) with peak reference sensitivity specified in Table 7.3.2.3-1. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link Angle).

Table 7.3.2.3-1: Reference sensitivity

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.3 | -85.3 | -82.3 | -79.3 |
| n258 | -88.3 | -85.3 | -82.3 | -79.3 |
| n260 | -85.7 | -82.7 | -79.7 | -76.7 |
| n261 | -88.3 | -85.3 | -82.3 | -79.3 |
| NOTE 1: The requirements in this table are only applicable for UE which supports single band in FR2. |  |  |  |  |

For UEs which do not support beam correspondence as defined in Section 6.2.5, the minimum requirement for EIS at the 50th percentile of the distribution of received power measured over the full sphere around the UE is defined as the EIS spherical coverage requirement and is found in Table 7.3.2.3-2 below. The requirement is verified with the test metric of EIS (Link=[TBD search grid], Meas=Link angle).

Table 7.3.2.3-2: UE EIS spherical coverage for power class 3

| Operating band | EIS at 50t%-tile CDF (dBm)/ Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | [-74.9] | [-71.9] | [-68.9] | [-65.9] |
| n258 | [-74.9] | [-71.9] | [-68.9] | [-65.9] |
| n260 | [-70.6] | [-67.6] | [-64.6] | [-61.6] |
| n261 | [-74.9] | [-71.9] | [-68.9] | [-65.9] |
| NOTE 1: Minimum requirement for EIS at 50 %-tile CDF is defined as the upper limit without tolerance. NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2. NOTE 3: The EIS spherical coverage requirement is only applicable to UEs which do not support beam correspondence as defined in Section 6.2.5. |  |  |  |  |

The minimum requirement for reference sensitivity and EIS spherical coverage for UEs which support multiple bands is expected to be optimized over the entire supported range of frequencies and is relaxed from the single-band requirements in Table 7.3.2.3-1 by the factor ΔTEMBP (multi-band relaxation of peak EIS) and in Table 7.3.2.3-2 by the factor ΔTEMBS (multi-band relaxation of EIS spherical coverage), which are defined in Table 7.3.2.3-3 below.

Table 7.3.2.3-3: UE multi-band relaxations for power class 3

| Supported bands |  | Applicable to band |  | ΔTEMBP (dB) |  | ΔTEMBS (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n260, n261 |  | n260 |  | [0] |  | [1.6] |  |
| n260, n261 | n261 | [0] | [0] |
| n257, n258, n260, n261 | n257 | [2.1] | [1.9] |
| n257, n258, n260, n261 | n258 | [2.1] | [2.0] |
| n257, n258, n260, n261 | n260 | [1.5] | [1.8] |
| n257, n258, n260, n261 | n261 | [2.0] | [1.2] |
| NOTE 1: The requirements in this table are applicable to UEs which support only the indicated bands. NOTE 2: The ΔTEMBS is only applicable to UEs which do not support beam correspondence as defined in Section 6.2.5. |  |  |  |  |  |  |

<<<end of change>>>