3GPP TSG RAN WG4 Meeting #88bis	R4-1812423

Chungdu, China, 8 - 12 October 2018

Agenda item:	9.4.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-11: EN-DC_1-8_n79

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_1-8_n79 was approved in RAN#81[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n79 | CA_1-8 | n79 | no |

Table 2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n79A | DC_1A_n79A DC_8A_n79A | CA_1A-8A | n79A |

3.  Co-existence

When Uplink EN-DC configuration is DC_1A_n79A, (1) IMD3 of (2*B1 - n79) and IMD4 of (3*B1-n79) will fall into Rx band of Band 8. When Uplink EN-DC configuration is DC_8A_n79A, (2) IMD4 of (3*B8 – n79) will fall into Rx band of Band 1.

Thus, it is proposed to evaluate (1) and (2).

[Proposal-1] It is proposed to evaluate (1) IMD3 of (2*B1 -n79) to B8 Rx and (2) IMD4 of (3*B8 – n79) to B1 Rx.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked from TR37.863-01-01.

Table 3: ΔTIB of constituent 2 band combo.

| ΔTIB | B1+B8 | B1+n79 | B8+n79 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0.3 | 0 |  | 0.3 |
| B8 | 0.3 |  | 0 | 0.3 |
| n79 |  | 0 | 0 | 0 |

Table 4: ΔRIB of constituent 2 band combo.

| ΔRIB | B1+B8 | B1+n79 | B8+n79 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0 | 0 |  | 0 |
| B8 | 0 |  | 0 | 0 |
| n79 |  | 0 | 0 | 0 |

Then the proposal for this 2LTE-1NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n79 | 0 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n79 | 1 | 0 |
|  | 8 | 0 |
|  | n79 | 0 |

5.  REFSENS

As mentioned in section 3, REFSENS exceptions due to IMD3 and IMD4 are expected. TP includes skeleton so that the actual values are filled when evaluation results come up.

Table 5: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-8A_n79A | 1 |  |  |  |  | N/A | FDD | N/A |  |
|  | n79 |  |  |  |  | N/A | TDD | N/A |  |
|  | 8 |  |  |  |  | TBD | FDD | IMD3 |  |
| DC_1A-8A_n79A | 8 |  |  |  |  | N/A | FDD | N/A |  |
|  | n79 |  |  |  |  | N/A | TDD | N/A |  |
|  | 1 |  |  |  |  | TBD | FDD | IMD4 |  |

6.  Conclusion

This paper proposes study results of EN-DC 1-8_n79. As in proposal-1, REFSENS exception results for IMD3 and IMD4 are needed to complete the combo.

TP is shown in Annex.

7.  Reference

[1]      RP-181855      Revised WID on EN-DC for 3 bands DL with 2 bands UL (2 LTE bands + 1 NR band), Huawei, HiSilicon

Annex.   Proposed text for TR37.716-21-11(v0.1.0): LTE 2B -NR 1B DC

[Unchanged Portions Skipped]

5.1.x	DC_a-b_nc

### 5.1.x.1	Operating bands for DC_1-8_n79

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n79 | CA_1-8 | n79 | no |

### 5.1.x.2	Configurations for DC_1-8_n79

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n79A | DC_1A_n79A DC_8A_n79A | CA_1A-8A | n79A |

### 5.1.x.3	Co-existence studies

### When Uplink EN-DC configuration is DC_1A_n79A, IMD3 of (2*B1 - n79) and IMD4 of (3*B1 - n79) will fall into Rx band of Band 8. When Uplink EN-DC configuration is DC_8A_n79A, IMD4 of (3*B8 - n79) will fall into Rx band of Band 1.

### 5.1.x.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n79 | 0 |

Table 5.1.x.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n79 | 1 | 0 |
|  | 8 | 0 |
|  | n79 | 0 |

### 5.1.x.5	REFSENS requirements

As mentioned above, IMD3 of B1 and n79 to Band 8 Rx and IMD4 of B8 and n79 to Band 1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-8A_n79A | 1 |  |  |  |  | N/A | FDD | N/A |  |
|  | n79 |  |  |  |  | N/A | TDD | N/A |  |
|  | 8 |  |  |  |  | TBD | FDD | IMD3 |  |
| DC_1A-8A_n79A | 8 |  |  |  |  | N/A | FDD | N/A |  |
|  | n79 |  |  |  |  | N/A | TDD | N/A |  |
|  | 1 |  |  |  |  | TBD | FDD | IMD4 |  |

[Unchanged Portions Skipped]