3GPP TSG-RAN WG4 RAN4#88Bis	R4-1812440

Chengdu, China, October 8th – 12th, 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PHR Mapping Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current PHR mapping table is incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PHR mapping table is correction |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PHR reporting cannot be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.17.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of changes >>

### 10.1.17	Power Headroom

10.1.17.1 Power Headroom Report

#### 10.1.17.1.1	Power Headroom Report Mapping

The power headroom reporting range is from -32 ...+38 dB. Table 10.1.17.1-1 defines the report mapping.

Table 10.1.17.1-1: Power headroom report mapping

| ## Reported value | ## Measured quantity value (dB) |
| --- | --- |
| ## POWER_HEADROOM_0 | ## PH -32 |
| ## POWER_HEADROOM_1 | ## -32 PH -31 |
| ## POWER_HEADROOM_2 | ## -31 PH -30 |
| ## POWER_HEADROOM_3 | ## -30 PH -29 |
| ## POWER_HEADROOM_53 | ## 20  PH  21 |
| ## POWER_HEADROOM_54 | ## 21  PH  22 |
| ## POWER_HEADROOM_55 | ## 22  PH  24 |
| ## POWER_HEADROOM_56 | ## 24  PH  26 |
| ## POWER_HEADROOM_57 | ## 26  PH  28 |
| ## POWER_HEADROOM_58 | ## 28  PH  30 |
| ## POWER_HEADROOM_59 | ## 30  PH  32 |
| ## POWER_HEADROOM_60 | ## 32  PH  34 |
| ## POWER_HEADROOM_61 | ## 34  PH  36 |
| ## POWER_HEADROOM_62 | ## 36  PH  38 |
| ## POWER_HEADROOM_63 | ## PH ≥ 38 |

## << End of changes >>