3GPP TSG-RAN WG4 Meeting #88-Bis	R4-1812496

# Chengdu, China, 8th- 12th October, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for SSB-based intra-frequency measurements (Section 9.2.5 and Section 9.2.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  |  | Date: |  | 2018-28-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Correct the location of Kca value in the requirements of SSB-based intra-frequency measurements and specify its value under different scenarios. - There is no active BWP before SCell being activated, so there should be no requirements for deactivated SCells of intra-frequency measurement with measurement gaps - Align the requirement formulation with inter-frequency measurement requirements for better readability |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove Kca from the tables - Specify CSFintra is a carrier specific scaling factor and is determined according to section [9.1.5.1] for non-gap based measurement and according to section [9.1.5.2] for gap based measurement. - Delete the requirement tables for deactivated SCells of intra-frequency measurement with measurement gaps |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements on intra-frequency measurements requirements will be unclarified in TS 38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 9.2.5, 9.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change ------------------------------------------------------------

### 9.2.5	Intrafrequency measurements with no measurement gaps

#### 9.2.5.1	Intrafrequency cell identification

Editor’s Note :The requirements below have been derived without considering gap sharing when all SMTC occasion are fully overlapping with measurement gaps.

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index, or the UE has been indicated that the neighbour cell is synchronous with the serving cell. Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that  deriveSSB-IndexFromCellis always enabled for FR2, and hence the identification time is always Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-4 (deactivated Scell) or 9.2.5.1-5 (deactivated SCell)

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.5.1-3 or  9.2.5.1-6 (deactivated SCell)

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.5.2-1, table 9.2.5.2-2 table 9.2.5.2-3 (deactivated Scell) or 9.2.5.2-4(deactivated SCell)

CSFintra: it is a carrier specific scaling factor and is determined according to CSFoutside_gap_i in section [9.1.5.1] for measurement conducted outside measurement gaps or according to CSFwithin_gap_i in section [9.1.5.2] for measurement conducted within measurement gaps.

Mpss/sss_sync_w/o_gaps : For a UE supporting power class 1(fixed wireless access), Mpss/sss_sync=40. For a UE supporting power class 2(vehicle mounted), Mpss/sss_sync_w/o_gaps =[24].  For a UE supporting power class 3(handheld), Mpss/sss_sync_w/o_gaps =[24]. For a UE supporting power class 4, Mpss/sss_sync_w/o_gaps =TBD

Mmeas_period_w/o_gaps : For a UE supporting power class 1 (fixed wireless access), Mmeas_period_w/o_gaps =40. For a UE supporting power class 2 (vehicle mounted), Mmeas_period_w/o_gaps =[24]. For a UE supporting power class 3 (handheld), Mmeas_period_w/o_gaps =[24]. For a UE supporting power class 4, Mmeas_period_w/o_gaps =TBD.

When intrafrequency SMTC is fully non overlapping with measurement gaps, Kp=1

When intrafrequency SMTC is partially overlapping with measurent gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP

For FR2 when RLM-RS outside measurement gap is fully overlapping with intra-frequency SMTC, KRLM= 1.5, otherwise KRLM=1.

Editor’s note : It is FFS how requirements are defined for the case that SMTC are fully overlapping with measurement gap

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max[ 600ms, ceil( [5] x Kp) x SMTC period x CSFintra ]Note 1 |
| DRX cycle≤ 320ms | max[ 600ms, ceil(1.5x [5] x Kp) x max(SMTC period,DRX cycle) x CSFintra ] |
| DRX cycle>320ms | Ceil([5] x Kp) x DRX cycle x CSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max[ 600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x KRLM) x SMTC period x CSFintra ]Note 1 |
| DRX cycle≤ 320ms | max[ 600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x KRLM) x max(SMTC period,DRX cycle) x CSFintra ] |
| DRX cycle>320ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp x KRLM) x DRX cycle x CSFintra |
| … | … |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.1-3: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max[120ms, ceil( 3 x Kp ) x SMTC period x CSFintra]Note 1 |
| DRX cycle≤ 320ms | max[120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle) x CSFintra ] |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | [5] x measCycleSCell x CSFintra |
| DRX cycle≤ 320ms | [5] x max(measCycleSCell, 1.5xDRX cycle) x CSFintra |
| DRX cycle> 320ms | [5] x max(measCycleSCell, DRX cycle) x CSFintra |

Table 9.2.5.1-5: Time period for PSS/SSS detection, deactivated SCell (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Mpss/sss_sync_w/o_gaps x measCycleSCell x CSFintra |
| DRX cycle≤ 320ms | Mpss/sss_sync_w/o_gaps x max(measCycleSCell, 1.5xDRX cycle) x CSFintra |
| DRX cycle> 320ms | Mpss/sss_sync_w/o_gaps x max(measCycleSCell, DRX cycle) x CSFintra |

Table 9.2.5.1-6: Time period for time index detection, deactivated SCell (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | [3] x measCycleSCell x CSFintra |
| DRX cycle≤ 320ms | [3] x max(measCycleSCell, 1.5xDRX cycle) x CSFintra |
| DRX cycle> 320ms | [3] x max(measCycleSCell, DRX cycle) |

Table 9.2.5.1-7: Void

Table 9.2.5.1-8: Void

#### 9.2.5.2	Measurement period

Editor’s Note : The requirements below have been derived so far assuming no configured Scell or E-UTRA SCell. The requirements when one or more SCells or E-UTRA SCells are configured is for further study. The requirements below have been derived without considering gap sharing when all SMTC occasion are fully overlapping with measurement gaps.

The measurement period for intrafrequency measurements without gaps is as shown in table 9.2.5.2-1, 9.2.5.2-2, 9.2.5.2-3 (deactivated SCell) or 9.2.5.2-4(deactivated SCell).

Table 9.2.5.2-1: Measurement period for intrafrequency measurements without gaps(Frequency FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max[ 200ms, ceil( 5 x Kp) x SMTC period x CSFintra ]Note 1 |
| DRX cycle≤ 320ms | max[ 200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle) x CSFintra] |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.2-2: Measurement period for intrafrequency measurements without gaps(Frequency FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max[ 400ms, ceil(Mmeas_period_w/o_gaps x Kp x KRLM) x SMTC period x CSFintra]Note 1 |
| DRX cycle≤ 320ms | max[ 400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x KRLM) x max(SMTC period,DRX cycle) x CSFintra ] |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x KRLM ) x DRX cycle x CSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.2-3: Measurement period for intrafrequency measurements without gaps (deactivated SCell) (Frequency range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | [5] x measCycleSCell x CSFintra |
| DRX cycle≤ 320ms | [5] x max(measCycleSCell, 1.5xDRX cycle) x CSFintra |
| DRX cycle> 320ms | [5] x max(measCycleSCell, DRX cycle) x CSFintra |

Table 9.2.5.2-4: Measurement period for intrafrequency measurements without gaps (deactivated SCell) (Frequency range FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Mmeas_period with_gaps x measCycleSCell x CSFintra |
| DRX cycle≤ 320ms | Mmeas_period with_gaps x max(measCycleSCell, 1.5xDRX cycle) x CSFintra |
| DRX cycle> 320ms | Mmeas_period with_gaps x max(measCycleSCell, DRX cycle) x CSFintra |

#### 9.2.5.3	Scheduling availability of UE during intra-frequency measurements

UE are required to be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE. When the measurement signal has different subcarrier spacing than PDSCH/PDCCH or on frequency range FR2, there are restrictions on the scheduling availability as described in the following clauses.

##### 9.2.5.3.1	Scheduling availability of UE performing measurements with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to measurements performed with a same subcarrier spacing as PDSCH/PDCCH on FR1.

##### 9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to measurements. For UE which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols and 1 data symbol after each consecutive SSB symbols within SMTC window duration. If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on all symbols within SMTC window duration. If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When intra-band carrier aggregation is performed, the scheduling restrictions apply to all serving cells on the band. When inter-band carrier aggregation within FR1 is performed, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to measurements performed on FR1 serving cell frequency layer in different bands.

##### 9.2.5.3.3	Scheduling availability of UE performing measurements on FR2

The following scheduling restriction applies due to SS-RSRP or SS-SINR measurement on an FR2 intra-frequency cell

The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured, and on 1 symbol before each consecutive SSB symbols and 1 data symbol after each consecutive SSB symbols within SMTC window duration (The signaling deriveSSB_IndexFromCell is always enabled for FR2) . If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

The following scheduling restriction applies to SS-RSRQ measurement on an FR2 intra-frequency cell

-	UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB/RSSI symbols and 1 data symbol after each consecutive SSB/RSSI symbols within SMTC window duration (The signaling deriveSSB_IndexFromCellc is always enabled for FR2). If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When intra-band carrier aggregation is performed, the scheduling restrictions apply to all serving cells on the band. When inter-band carrier aggregation within FR2 is performed, the scheduling restrictions apply to all serving cells on the bands.

Editor’s Note: FFS scheduling restrictions for inter-band carrier aggregation will be defined depending on band combination in future.

##### 9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 serving cell(s) due to measurements performed on FR2 serving cell frequency layer.

There are no scheduling restrictions on FR2 serving cell(s) due to measurements performed on FR1 serving cell frequency layer.

### 9.2.6	Intrafrequency measurements with measurement gaps

#### 9.2.6.1	Intra gap sharing

[Editor’s note : This is being studied]

#### 9.2.6.2	Intrafrequency cell identification

Editor’s Note : The impact of gap sharing between intrafrequency and interfrequency measurements has not been include in the requirements below.

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index, or the UE has been indicated that the neighbour cell is synchronous with the serving cell.. Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that  deriveSSB-IndexFromCell is always enabled for FR2, and hence the identification time is always Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + T SSB_measurement_period_intra + TSSB_time_index_intra

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2.6.2-1, 9.2.6.2-2,  or 9.2.6.2-4 or 9.2.6.2-5 (deactivated SCell)

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.6.2-3or 9.2.6.2-6(deactivated SCell).

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.6.2-1, 9.2.6.2-2, 9.2.6.2-3 (deactivated Scell) or 9.2.6.2-4(deactivated SCell)

CSFintra: it is a carrier specific scaling factor and is determined according to CSFwithin_gap_i in section [9.1.5.2] for measurement conducted within measurement gaps.

Mpss/sss_sync_with_gaps : For a UE supporting power class 1(fixed wireless access), Mpss/sss_sync with_gaps=40. For a UE supporting power class 2(vehicle mounted), Mpss/sss_sync with_gaps =[24].  For a UE supporting power class 3(handheld), Mpss/sss_sync with_gaps =[24]. For a UE supporting power class 4, Mpss/sss_sync with_gaps =TBD

Mmeas_period_ with_gaps: For a UE supporting power class 1(fixed wireless access), Mmeas_period_ with_gaps =40. For a UE supporting power class 2(vehicle mounted), Mmeas_period_ with_gaps =[24]. For a UE supporting power class 3(handheld), Mmeas_period_ with_gaps =[24]. For a UE supporting power class 4, Mmeas_period with_gaps =TBD.

Table 9.2.6.2-1: Time period for PSS/SSS detection (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max[ 600ms, [5] x max(MGRP, SMTC period) x CSFintra] |
| DRX cycle≤ 320ms | max[ 600ms, ceil(1.5x [5]) x max(MGRP, SMTC period,DRX cycle) x CSFintra] |
| DRX cycle>320ms | [5] x max(MGRP, DRX cycle) x CSFintra |

Table 9.2.6.2-2: Time period for PSS/SSS detection (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max[ 600ms, Mpss/sss_sync_with_gaps x max(MGRP, SMTC period) x CSFintra ] |
| DRX cycle≤ 320ms | max[ 600ms, ceil(1.5x Mpss/sss_sync_with_gaps) x max(MGRP, SMTC period, DRX cycle) x CSFintra ] |
| DRX cycle>320ms | Mpss/sss_sync_with_gaps x max(MGRP, DRX cycle) x CSFintra |

Table 9.2.6.2-3: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max[ 120ms, 3 x max(MGRP, SMTC period) x CSFintra ] |
| DRX cycle≤ 320ms | max[ 120ms, ceil(1.5x 3) x max(MGRP, SMTC period,DRX cycle) x CSFintra] |
| DRX cycle>320ms | 3 x max(MGRP, DRX cycle) x CSFintra |

Table 9.2.6.2-7: Void

Table 9.2.6.2-8: Void

#### 9.2.6.3	Intrafrequency Measurement Period

Editor’s Note : The requirements below have been derived so far assuming no configured Scell or E-UTRA SCell. The requirements when one or more SCells or E-UTRA SCells are configured is for further study. . The impact of gap sharing between intrafrequency and interfrequency measurements has not been include in the requirements below.

The measurement period for FR1 intrafrequency measurements with gaps is as shown in table 9.2.6.3-1 or 9.2.6.3-3(deactivated Scell)

The measurement period for FR2 intrafrequency measurements with gaps is as shown in table 9.2.6.3-2 or 9.2.6.3-4(deactivated Scell)

Editor’s note: The values of X, Y and N in the following tables are to be updated.

Table 9.2.6.3-1: Measurement period for intrafrequency measurements with gaps(Frequency Range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max[ 200ms, 5 x max(MGRP, SMTC period) x CSFintra ] |
| DRX cycle≤ 320ms | max[ 200ms, ceil(1.5x 5) x max(MGRP, SMTC period,DRX cycle) x CSFintra] |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSFintra |

Table 9.2.6.3-2: Measurement period for intrafrequency measurements with gaps(Frequency Range FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max[ 400ms, Mmeas_period with_gaps x max(MGRP, SMTC period) x CSFintra ] |
| DRX cycle≤ 320ms | max[ 400ms, ceil(1.5 x Mmeas_period with_gaps) x max(MGRP, SMTC period, DRX cycle) x CSFintra] Note 1 |
| DRX cycle>320ms | Mmeas_period with_gaps x max(MGRP, DRX cycle) x CSFintra |

----------------------------------------------------------- End of Change ----------------------------------------------------------------