3GPP TSG-RAN WG4 Meeting #88-BIS                                           	           R4-1812511

# Chengdu, China, 8th – 12th Oct, 2018

Agenda item:            7.12.5.13

Source:	MediaTek Inc.

Title:	Simulation assumptions for SSB-based RLM

Document for:          Approval

- Simulation assumption

- The proposed simulation assumptions for PDCCH are listed in Table 1 and Table 2.

Table 1: Simulation assumptions for PDCCH performance, out-of-sync

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  | GHz | FR1, 4 GHz | FR1, 4 GHz | FR2, 30 GHz | FR2, 30 GHz |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Physical layer cell identity, |  |  | 0 | 0 | 0 | 0 |
| NR Channel Bandwidth (BWchannel) |  | MHz | 10 | 40 | 100 | 100 |
| Subcarrier Spacing for SSB |  | KHz | 15 | 30 | 120 | 240 |
| Subcarrier Spacing for PDCCH |  | KHz | 15 | 30 | 120 | 120 |
| Antenna Configuration |  |  | 2x2 Low, 2x4 Low | 2x2 Low, 2x4 Low | 2x2 Low (Note 1) | 2x2 Low (Note 1) |
| Out of sync transmission parameters (based on Table 8.1.2.1-1 in TS 38.133) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | DCI size without CRC |  | 39 bits | 41 bits | 40 bits | 40 bits |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Bandwidth of RMSI CORESET | PRB | 24 | 24 | 24 | 24 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | CORESET start PRB |  | 0 | 0 | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size, L |  | 6 | 6 | 6 | 6 |
|  | Mapping from REG to CCE |  | Interleaved | Interleaved | Interleaved | Interleaved |
|  | nshift |  | 0 | 0 | 0 | 0 |
|  | InterleaverSize. R |  | 2 | 2 | 2 | 2 |
| Propagation channel |  |  | TDL-C 300ns 100Hz | TDL-C 300ns 100Hz | TDL-A 30ns 75Hz | TDL-A 30ns 75Hz |
| TRS configuration |  |  | - slots pattern - Periodicity 20ms - Offset 10ms from SSB - OFDM symbols with TRS: #4 and #8 - Bandwidth: full BWP - 0 dB boosting |  |  |  |
| Outcome metric |  |  | SNR levels corresponding to 10% PDCCH BLER |  |  |  |
| Note 1: The antenna configuration of 2x2 in FR2 is just for simulation purpose on PDCCH performance, not for OTA test cases Note 2: UE receiver assumptions: Real channel estimation, noise estimation and synchronization Note 3: precoder model for 2TX: random for each REG bundle Note 4: percoder matrix is Type I single-Pannel Codebook for 1 layer defined in Table 5.2.2.2.1-1 in TS 38.214 |  |  |  |  |  |  |

Table 2: Simulation assumptions for PDCCH performance, in-sync

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  | GHz | FR1, 4 GHz | FR1, 4 GHz | FR2, 30 GHz | FR2, 30 GHz |
| Physical layer cell identity, |  |  | 0 | 0 | 0 | 0 |
| CP length |  |  | Normal | Normal | Normal | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | 10 | 40 | 100 | 100 |
| Subcarrier Spacing for SSB |  | KHz | 15 | 30 | 120 | 240 |
| Subcarrier Spacing for PDCCH |  | KHz | 15 | 30 | 120 | 120 |
| Antenna Configuration |  |  | 2x2 Low, 2x4 Low | 2x2 Low, 2x4 Low | 2x2 Low (Note 1) | 2x2 Low (Note 1) |
| In-sync transmission parameters (based on Table 8.1.2.1-2 in TS 38.133) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | DCI size without CRC |  | 39 bits | 41 bits | 40 bits | 40 bits |
|  | Aggregation level | CCE | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | Bandwidth of RMSI CORESET | PRB | 24 | 24 | 24 | 24 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | CORESET start PRB |  | 0 | 0 | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size, L |  | 6 | 6 | 6 | 6 |
|  | Mapping from REG to CCE |  | Interleaved | Interleaved | Interleaved | Interleaved |
|  | nshift |  | 0 | 0 | 0 | 0 |
|  | InterleaverSize. R |  | 2 | 2 | 2 | 2 |
| Propagation channel |  |  | TDL-C 300ns 100Hz | TDL-C 300ns 100Hz | TDL-A 30ns 75Hz | TDL-A 30ns 75Hz |
| Outcome metric |  |  | SNR levels corresponding to 2% PDCCH BLER |  |  |  |
| TRS configuration |  |  | - slots pattern - Periodicity 20ms - Offset 10ms from SSB - OFDM symbols with TRS: #4 and #8 - Bandwidth: full BWP - 0 dB boosting |  |  |  |
| Note 1: The antenna configuration of 2x2 in FR2 is just for simulation purpose on PDCCH performance, not for OTA test cases Note 2: UE receiver assumptions: Real channel estimation, noise estimation and synchronization Note 3: precoder model for 2TX: random for each REG bundle Note 4: percoder matrix is Type I single-Pannel Codebook for 1 layer defined in Table 5.2.2.2.1-1 in TS 38.214 |  |  |  |  |  |  |