3GPP TSG-RAN WG4 Meeting #88bis                	R4-1812694

Chengdu, China, 8 – 12 Oct 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NR SCell activation delay

Agenda item:	7.11.7.3

Document for:	Discussion

Introduction

The NR SCell activation delay has been discussed in the past several RAN4 meetings. In RAN4 #88 meeting, Requirement on NR SCell activation delay was agreed in [1]. However, it has still some open issues for the requirement. In this contribution, we will provide our view on this delay requirement in FR2.

Discussion

SCell activation delay

In the Gothenburg meeting, RAN4 has agreed the requirement on NR SCell activation delay as below:

Tactivation_time is the SCell activation delay. If the SCell is known and belongs to FR1, Tactivation_time is: - [3ms+ 1* TSMTC_SCell +2ms], if the SCell measurement cycle is equal to or smaller than [160ms]. - [3ms+TSMTC_MAX+TSMTC_SCell +2ms], if the SCell measurement cycle is larger than [160ms]. If the SCell is unknown and belongs to FR1, Tactivation_time is: - [3ms+ 2*TSMTC_MAX+2*TSMTC_SCell +2ms] provided the SCell can be successfully detected on the first attempt. If the SCell being activated belongs to FR2, and there is at least one active serving cell on that FR2 band, then Tactivation_time is: [3ms+ TBD], If the SCell being activated belongs to FR2, and there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1, then, Tactivation_time is [3ms+ 25*TSMTC_SCell +2ms].

The SCell activation time in FR2 is still open, Similar to the time in FR1, this needs to include time for MAC-CE message decode and application, AGC settling, search and timing refinement if needed.

3ms is the time needed by UE for MAC-CE message decode and application (SW program both baseband and RF), For UE, the procedure for MAC-CE message decode and SW program for baseband and RF should be no difference in FR1 and FR2.

In case there already exists at least one active cell on that FR2 band, UE can re-use the timing information from that cell. it should not need any more extra time for search or timing refinement.

SCell activation delay for intra-band cells in FR2 could be 3ms

In case there is no active cell on that FR2 band, since the difference between known and unknown SCell, we would split to two cases as in FR1: known SCell and unknown SCell.

In FR2, since Tx and Rx beamforming is used, we need to consider the impact by beamforming.

Known SCell is per definition already detected, measured and reported. In FR2, since Tx and Rx beamforming is used, we need to consider the impact by beamforming. Since it is known SCell and already detected, the Tx beam should not changed, Considering Rx beamforming, scaling factor N1 is added. As side conditions are usually good (-3dB), we think N1 SMTC periodicity should be enough for known SCell to perform final time and frequency fine tuning upon activation when SCell measurement cycle is equal to or smaller than 160ms, and N1 extra SMTC periodicity if SCell measurement cycle is larger than 160ms.

If it is unknown cell, cell detection is needed. In FR2, since Tx and Rx beamforming is used, we need to consider the impact by beamforming. Scaling factor N1 is added. 4*N1 SMTC periodicity is needed if the cell can be successfully detected on the first attempt.

N1 is the scaling factor due to Rx beam sweeping, we could discuss further.

SCell activation delay for known cell in FR2 and with no active cell in that FR2 band could be same as in FR1

- -	[3ms+N1*TSMTC_SCell +2ms], if the SCell measurement cycle is equal to or smaller than [160ms].

- -	[3ms+N1*(TSMTC_MAX+TSMTC_SCell)+2ms], if the SCell measurement cycle is larger than [160ms].

SCell activation delay for unknown cell in FR2 and with no active cell in that FR2 band could be [3ms + 2*N1*(TSMTC_MAX+TSMTC_SCell) + 2ms]

Proposals are included in our CR [3].

SCell known condition

In [2], RAN4 has agreed the requirement on NR SCell known condition as below:

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max([5] measCycleSCell, [5] DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in section 9.2 and 9.3. - the SSB measured during the period equal to max([5] measCycleSCell, [5] DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in section 9.2 and 9.3. Otherwise SCell in FR1 is unknown.

In the agreement, the SCell known condition in FR1 is ready, but in FR2, there is no definition for SCell known condition.

For FR2, there are some additional consideration as beamforming used in Tx and Rx. We could use the definition in FR1 as baseline and need to consider if there is necessary to introduce to identify beam issue when a SCell remains detectable in the cell known condition.

As UE Rx beam forming is related to UE implementation and not testable from radio side, UE Rx beam should not be part of the UE SCell activation delay requirements. While the SCell is deactivated the UE will continuously measure the deactivated SCell according to the requirements. Such measurements include measuring using UE Rx beam sweeping. I.e. the UE should be able to track the SCell and which Rx beam to use once the SCell is being activated. From requirements point of view, RAN4 can include a condition that for a known SCell it is assumed that the Tx beam that transmitted SSB when the UE reported the cell has not changed, and under such condition the cell is and remains detectable.

In current SCell known condition in FR1, it is already indicated that the SSB measured remains detectable. We could reuse the definition of SCell known condition in FR1 for FR2 directly.

NR SCell known condition in FR2 could be the same as FR1.

Proposals are included in our CR [3].

Conclusion

In this contribution we have discussed NR SCell activation delay requirement and SCell known condition. We have made the following proposal for FR2:

- SCell activation delay for intra-band cells in FR2 could be 3ms

- SCell activation delay for known cell in FR2 and with no active cell in that FR2 band could be same as in FR1

- -	[3ms+N1*TSMTC_SCell +2ms], if the SCell measurement cycle is equal to or smaller than [160ms].

- -	[3ms+N1*(TSMTC_MAX+TSMTC_SCell)+2ms], if the SCell measurement cycle is larger than [160ms].

- SCell activation delay for unknown cell in FR2 and with no active cell in that FR2 band could be [3ms + 2*N1*(TSMTC_MAX+TSMTC_SCell) + 2ms]

- NR SCell known condition in FR2 could be the same as FR1.

References

- R4-1811697, CR on Scell activation requirements in FR2, CATT

- 3GPP TS 38.133 v15.3.0

- R4-1812695, CR on NR Scell activation, Nokia, Nokia Shanghai Bell