3GPP TSG-RAN WG4 Meeting #88bis                                                     R4-1812697

# Chengdu, China, 8 – 12 Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5952 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PSCell addition delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NR PSCell addition delay in FR2 need to be updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | To provide the requirements for NR PSCell addition delay in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are not complete in NR PSCell addition delay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.31.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ...TS36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

### 7.31.2	NR PSCell Addition Delay Requirement

The requirements in this section shall apply for the UE which is configured with PCell, and may also be configured with one or more SCells.

Upon receiving NR PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards NR PSCell no later than in subframe n + Tconfig PSCell:

Where:

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2ms

TRRC_delay is the RRC procedure delay as specified in [2].

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20ms if NR PSCell is in FR1, Tprocessing = 40ms if NR PSCell is in FR2.

Tsearch is the time for AGC settling and PSS/SSS detection.

-	For NR PSCell in FR1: if the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown cell and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 3*SMTC periodicity ms;

-	For NR PSCell in FR2: if the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown cell and signal quality is sufficient for successful cell detection, then Tsearch = 24*SMTC periodicity ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1 SMTC periodicity ms.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the NR PSCell. TPSCell_ DU is up to x*10 +10 ms. x is defined in the table 6.3.3.2-2 of [42].

In FR1 and FR2, the NR PSCell is known if it has been meeting the following conditions:

During the last 5 seconds before the reception of the NR PSCell configuration command:

-	the UE has sent a valid measurement report for the NR PSCell being configured and

-	One of the SSBs measured from the NR PSCell being configured remains detectable according to the cell identification conditions specified in section 9.3 of TS 38.133 [50],

-	One of the SSBs measured from NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 9.3 of TS 38.133 [50].

otherwise it is unknown.

The PCell interruption specified in section 7.32 is allowed only during the RRC reconfiguration procedure [2].

Editor’s note: It is FFS whether SRS carrier based switching during NR PSCell addition procedure impacts NR PSCell addition delay requirement.

Editor’s note: It is FFS whether delay uncertainty due to PCell PRACH preamble transmission has an impact on Tconfig_PSCell.

<< End of changes>>