3GPP TSG-RAN WG4 Meeting 88bis	R4-1812819

Chengdu, China, 8 – 12 October 2018

Agenda Item:	6.7.7.1.1

Source:	Ericsson

Title:	Simulation results of PDSCH for sTTI UE demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN4#AH1807 agreed with the way forward on UE demodulation requirements for sTTI [1].

- Slot-based PDSCH demodulation requirements - In slot-based PDSCH test cases, for a TB first transmitted at slot n (=0,1), any retransmission can only happen on the slots satisfying following table - When the NAK is received at slot m, retransmission can happen no earlier than slot m+4 - Subslot-based PDSCH demodulation requirements - In subslot-based PDSCH test cases, for a TB first transmitted at subslot n (=1,2,3,4,5), any retransmission can only happen on the subslots satisfying following table - When the NAK is received at sub-slot m, retransmission can happen no earlier than sub-slot m+4 or m+6, depending on UE capability - Slot/Subslot-PDSCH demodulation requirements - No PDSCH allocation on subframe 0 and subframe 5 for all sPDSCH test cases - Target coding rate 0.45 for subslot-based CRS-based PDSCH demodulation - Consider 2 NZP CSI-RS ports for DMRS-based slot/subslot-PDSCH test cases - Companies are encouraged to provide the simulation results of sPDSCH based on the agreement - Simulation assumption: R4-1805493

This contribution provides the simulation results based on the draft CR [4].

# 2	Simulation results

RAN4#AH1807 agreed with the retransmission scheduling assumption to avoid the coding rate change between the initial transmission and retransmissions. Table 1 and Table 2 show the retransmission rules for slot-PDSCH and subslot-PDSCH, respectively. For DMRS-based subslot-PDSCH, as we also discussed in [3], if BS does not schedule PDSCH in subslot 1, BS does not schedule PDSCH in subslot 2, and if BS does not schedule PDSCH in subslot 3, BS should not schedule PDSCH in subslot 4.

Table 1	TBS scheduling for slot-PDSCH.

| Slot for the initial transmission | Slots used for the retransmission |
| --- | --- |
| Slot 0 | Slot 0 |
| Slot 1 | Slot 1 |

Table 2	TBS scheduling for subslot-PDSCH.

| Subslot for the initial transmission | Subslots used for the retransmission |
| --- | --- |
| Subslot 1 | Subslot 1, Subslot 5 |
| Subslot 2 | Subslot 2, Subslot 4 |
| Subslot 3 | Subslot 3 |
| Subslot 4 | Subslot 2, Subslot 4 |
| Subslot 5 | Subslot 1, Subslot 5 |

Figure 1 shows the simulation result of CRS-based (TM3) slot-PDSCH and subslot-PDSCH. This figure also shows the simulation result with TDD. For TDD, we assume eNB does not transmit PDSCH in special subframes.

Figure 2 shows the simulation result of DMRS-based (TM9) slot-PDSCH and subslot-PDSCH. This figure also shows slot-PDSCH with TDD. As same as TM3 scenario, we assume eNB does not transmit PDSCH in special subframes for TDD.

Figure 1	Simulation results of slot/subslot-PDSCH TM3 dual layer.

Figure 2	Simulation results of slot/subslot-DPSCH TM9 dual layer.

Table 3 summarizes the SNR achieving 70% of the maximum throughput.

Table 3	Simulation results of Slot-PDSCH and Subslot-PDSCH.

|  | Slot-PDSCH (FDD) | Subslot-PDSCH (FDD) | Slot-PDSCH (TDD) |
| --- | --- | --- | --- |
| CRS-based (TM3) | 11.1dB | 9.5dB | 11.3dB |
| DMRS-based (TM9) | 3.8dB | 3.8dB | 4.0dB |

# References

- R4-1809375, “Way forward on reference channels for sTTI demodulation and CSI requirements”, Qualcomm, Ericsson, Huawei.

- R4-1808812, “Update of simulation assumption for sTTI UE demodulation and CSI requirements”, Ericsson.

- R4-1808808, “Simulation results of PDSCH for sTTI UE demodulation requirements”, Ericsson.

- R4-1812820, “Introduction of Slot/Subslot-PDSCH demodulation requirements” ,Ericsson.