3GPP TSG-RAN WG4 Meeting 88bis	R4-1812824

Chengdu, China, 8 – 12 October 2018

Agenda Item:	6.7.7.2.2

Source:	Ericsson

Title:	CQI reporting tests for DMRS-based PDSCH

Document for:	Discussion

# 1	Introduction

RAN4#AH1807 agreed with the way forward on sTTI demodulation and CQI reporting test [1], and we provided the simulation results in RAN4#88 [2]. We resubmit the same contribution to discuss the sTTI CQI reporting test for DMRS-based PDSCH.

# 2	Discussion

## 2.2	CQI to MCS table

This section discusses the CQI to MCS table used for DMRS-based wideband CQI test based on the test parameters in Table 1. Table 2 also shows the SPDCCH parameters agreed in AH1807.

Table 1	Parameters for wideband CQI reporting test.

|  | Test 1 |
| --- | --- |
| Bandwidth | 10MHz |
| TTI length | Slot-based / Subslot-based |
| Number of PDCCH symbols | 2 |
| Cyclic prefix | Normal |
| Subframe configuration | Non-MBSFN |
| Propagation channel | EPA5 |
| Antenna configuration | High (2 x 2) |
| Transmission modes | TM9 |
| Beamforming model | TS 36.101 B.4.3 |
| CRS reference signals | Antenna port 0 |
| CSI reference signals | Antenna ports 15 and 16 |
| CSI-RS periodicity and subframe offset | 5ms / 1 |
| CSI-RS reference signal configuration | 0 |
| CodeBookSubsetRestrction bitmap | 000001 |
| Reporting interval | 5ms |
| CQI delay | n+4 slots for slot-based n+4 or n+6 subslots for subslot-based |
| Reporting mode | PUSCH 1-1 |
| PDSCH transmission bandwidth | 50 RB |
| Number of HARQ transmissions | 1 |
| Duplex mode | FDD / TDD |

Table 2	SPDCCH parameters for Wideband CQI test.

|  | SPDCCH parameters |
| --- | --- |
| Reference symbol | CRS |
| Transmission type | Localized |
| Number of OFDM symbols | 2 |
| Number of PRBs | 16 |
| Allocated PRBs | {0, 1, … 15} |
| Rate matching mode | Mode 1 |
| SPDCCH L1 Reuse Indication | Not configured |
| Total number of SCCEs | 8 (32 SREGs) |

Table 3 and Table 4 show our proposal on the CQI to MCS table used for the wideband CQI for slot-based transmission and subslot-based transmission, respectively. We calculate the available resource elements based on the assumption:

- SF#0 and SF#5 are DTX as same as the existing wideband CQI reporting test. In this case, we only need to consider the subframes without PSS/SSS/PBCH and SI transmission.

- Reduce the REs used for SPDCCH transmission (AL8 = 16 RBs with two OFDM symbols)

- CFI=2

- 1 CRS port

- 2 DMRS ports (CSI-RS configuration 0).

- 2 NZP-CSI-RS ports (SF#1 and #6)

Since we set 2 CSI-RS ports with CSI-RS configuration 0, CSI-RS is transmitted in slot 0 for the slot-based transmission and in subslot 2 for the subslot-based transmission.

Table 3	CQI-to-MCS table for DMRS-based Slot-PDSCH

|  |  |  | Non CSI-RS |  | 2 CRS-RS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Slot 0 | Slot 1 | Slot 0 | Slot 1 |
|  |  | PRB | 50 | 50 | 50 | 50 |
|  |  | Available REs | 2600 | 3348 | 2500 (CSI-RS) | 3348 |
| CQI index | Code rate | Modulation |  |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 0 | 0 | 0 |
| 3 | 0.19 | 2 | 1 | 2 | 1 | 2 |
| 4 | 0.3 | 2 | 3 | 4 | 3 | 4 |
| 5 | 0.44 | 2 | 5 | 7 | 5 | 7 |
| 6 | 0.59 | 2 | 7 | 9 | 7 | 9 |
| 7 | 0.37 | 4 | 10 | 12 | 10 | 12 |
| 8 | 0.48 | 4 | 12 | 14 | 12 | 14 |
| 9 | 0.6 | 4 | 14 | 16 | 13 | 16 |
| 10 | 0.46 | 6 | 17 | 19 | 17 | 19 |
| 11 | 0.55 | 6 | 18 | 21 | 18 | 21 |
| 12 | 0.65 | 6 | 20 | 23 | 20 | 23 |
| 13 | 0.75 | 6 | 22 | 26 | 22 | 26 |
| 14 | 0.85 | 6 | 24 | 27 | 23 | 27 |
| 15 | 0.93 | 6 | 25 | 28 | 24 | 28 |

Table 4	CQI-to-MCS table for DMRS-based Subslot-PDSCH

|  |  |  | Non CSI-RS |  |  |  |  | 2 CSI-RS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Subslot 1 | Subslot 2 | Subslot 3 | Subslot 4 | Subslot 5 | Subslot 1 | Subslot 2 | Subslot 3 | Subslot 4 | Subslot 5 |
|  |  | PRB | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  |  | Available REs | 1180 | 680 | 612 | 680 | 1212 | 1180 | 612 (CSI-RS) | 612 | 680 | 1212 |
| CQI index | Code rate | Modulation |  |  |  |  |  |  |  |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 3 | 0.19 | 2 | 2 | 2 | 0 | 2 | 3 | 2 | 0 | 0 | 2 | 3 |
| 4 | 0.3 | 2 | 5 | 5 | 2 | 5 | 5 | 5 | 2 | 2 | 5 | 5 |
| 5 | 0.44 | 2 | 7 | 7 | 3 | 7 | 7 | 7 | 3 | 3 | 7 | 7 |
| 6 | 0.59 | 2 | 9 | 9 | 5 | 9 | 9 | 9 | 5 | 5 | 9 | 9 |
| 7 | 0.37 | 4 | 12 | 12 | 10 | 12 | 12 | 12 | 10 | 10 | 12 | 12 |
| 8 | 0.48 | 4 | 14 | 14 | 10 | 14 | 15 | 14 | 10 | 10 | 14 | 15 |
| 9 | 0.6 | 4 | 16 | 16 | 11 | 16 | 16 | 16 | 11 | 11 | 16 | 16 |
| 10 | 0.46 | 6 | 19 | 19 | 17 | 19 | 20 | 19 | 17 | 17 | 19 | 20 |
| 11 | 0.55 | 6 | 22 | 22 | 17 | 22 | 22 | 22 | 17 | 17 | 22 | 22 |
| 12 | 0.65 | 6 | 24 | 24 | 17 | 24 | 25 | 24 | 17 | 17 | 24 | 25 |
| 13 | 0.75 | 6 | 27 | 27 | 18 | 27 | 27 | 27 | 18 | 18 | 27 | 27 |
| 14 | 0.85 | 6 | 28 | 28 | 19 | 28 | 28 | 28 | 19 | 19 | 28 | 28 |
| 15 | 0.93 | 6 | 28 | 28 | 20 | 28 | 28 | 28 | 20 | 20 | 28 | 28 |

# 3	Simulation results

The test metrics of the existing wideband CQI reporting requirements are specified as follows:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to δ

Table 5 and Table 6 show our simulation results of wideband CQI reporting with slot-PDSCH and subslot-PDSCH, respectively, with regard to the metrics above. We list the test points around the median CQI index 4 (1/3 QPSK), 8 (1/2 16QAM), and 13 (3/4 64QAM). Figure 1 and Figure 2 show the median CQI and throughput ratio values for reference.

We propose to set two sets of SNR test points {3, 4} and {12, 13} because the SNR estimation in the lower SNR (e.g. < 0dB) is not accurate. The reason we propose the test points corresponding to median CQI 12 is to consider higher throughput ratio values. For the test metrics, our simulation results show we can reuse the existing metrics, i.e., α=20%, γ=1.05, and δ=0.02.

Proposal 1: Set two sets of test points {3, 4} and {12, 13} for both slot-based and subslot-based wideband CQI reporting test.

Proposal 2: Reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

Table 5	Simulation results of wideband CQI with slot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| -4 | 4 | 38.9 | 0.15 | 0.97 |
| -3 | 4 | 43.1 | 0.12 | 1.16 |
| 3 | 8 | 40.6 | 0.09 | 1.40 |
| 4 | 8 | 40.9 | 0.09 | 1.31 |
| 12 | 12 | 36.2 | 0.09 | 1.40 |
| 13 | 13 | 36.3 | 0.08 | 1.56 |
| 14 | 13 | 32.8 | 0.07 | 1.39 |
| 15 | 13 | 37.0 | 0.07 | 1.28 |

Table 6	Simulation results of wideband CQI with subslot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| -4 | 4 | 36.4 | 0.34 | 1.25 |
| -3 | 4 | 36.8 | 0.31 | 1.30 |
| 4 | 8 | 37.7 | 0.18 | 1.27 |
| 5 | 8 | 37.2 | 0.17 | 1.24 |
| 12 | 12 | 37.7 | 0.16 | 1.31 |
| 13 | 12 | 36.2 | 0.16 | 1.22 |
| 14 | 12 | 38.7 | 0.17 | 1.15 |
| 15 | 13 | 32.8 | 0.17 | 1.23 |
| 16 | 13 | 34.2 | 0.19 | 1.13 |

Figure 1	Median CQI.

Figure 2	Throughput ratio between the follow CQI and Fixed median CQI.

# 4	Conclusion

Proposal 1: Set two sets of test points {3, 4} and {12, 13} for both slot-based and subslot-based wideband CQI reporting test.

Proposal 2: Reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

# 5	References

R4-1809375, “Way forward on reference channels for sTTI demodulation and CSI requirements”, Qualcomm, Ericsson, Huawei.

R4-1810151, “CQI reporting tests for DMRS-based PDSCH”, Ericsson.

# Appendix

Table 7	TBS table for slot/subslot-PDSCH of 50PRB.

|  |  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMCS | Qm | ITBS | TBS with 50PRB | TBSscaled | TBSslot | TBSscaled | TBSsubslot |
| 0 | 2 | 0 | 1384 | 692 | 696 | 230.7 | 224 |
| 1 | 2 | 1 | 1800 | 900 | 904 | 300 | 296 |
| 2 | 2 | 2 | 2216 | 1108 | 1096 | 369.3 | 376 |
| 3 | 2 | 3 | 2856 | 1428 | 1416 | 476 | 472 |
| 4 | 2 | 4 | 3624 | 1812 | 1800 | 604 | 600 |
| 5 | 2 | 5 | 4392 | 2196 | 2216 | 732 | 744 |
| 6 | 2 | 6 | 5160 | 2580 | 2600 | 860 | 872 |
| 7 | 2 | 7 | 6200 | 3100 | 3112 | 1033.3 | 1032 |
| 8 | 2 | 8 | 6968 | 3484 | 3496 | 1161.3 | 1160 |
| 9 | 2 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 10 | 4 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 11 | 4 | 10 | 8760 | 4380 | 4392 | 1460 | 1480 |
| 12 | 4 | 11 | 9912 | 4956 | 4968 | 1652 | 1672 |
| 13 | 4 | 12 | 11448 | 5724 | 5736 | 1908 | 1928 |
| 14 | 4 | 13 | 12960 | 6480 | 6456 | 2160 | 2152 |
| 15 | 4 | 14 | 14112 | 7056 | 6968 | 2352 | 2344 |
| 16 | 4 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 17 | 6 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 18 | 6 | 16 | 16416 | 8208 | 8248 | 2736 | 2728 |
| 19 | 6 | 17 | 18336 | 9168 | 9144 | 3056 | 3112 |
| 20 | 6 | 18 | 19848 | 9924 | 9912 | 3308 | 3368 |
| 21 | 6 | 19 | 21384 | 10692 | 10680 | 3564 | 3624 |
| 22 | 6 | 20 | 22920 | 11460 | 11448 | 3820 | 3880 |
| 23 | 6 | 21 | 25456 | 12728 | 12576 | 4242.7 | 4264 |
| 24 | 6 | 22 | 27376 | 13688 | 13536 | 4562.7 | 4584 |
| 25 | 6 | 23 | 28336 | 14168 | 14112 | 4722.7 | 4776 |
| 26 | 6 | 24 | 30576 | 15288 | 15264 | 5096 | 5160 |
| 27 | 6 | 25 | 31704 | 15852 | 15840 | 5284 | 5352 |
| 28 | 6 | 26 | 36696 | 18348 | 18336 | 6116 | 6200 |