3GPP TSG-RAN WG4 Meeting #88bis	R4-1812840

Chengdu, China, 8 – 12 October 2018

Agenda Item:	6.6.4.4

Source:	Ericsson

Title:	Simulation results of CQI reporting test for 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#88 agreed with the evaluation parameters for CQI reporting test with 1024QAM as follow [1].

- Definition test:

  - Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols): CRS based TM single codeword with TM1 under static channel in Annex B.1

    - Bandwidth:

      - FDD: 10MHz

      - TDD: 20MHz

RAN4#88 also reached the following agreements online:

Agreement:

- No PDSCH allocation on subframe 0 and subframe 5 for 1024QAM CQI reporting test.

- Consider CFI 3 and full RB allocation for 1024QAM CQI reporting test.

In this contribution we provide our simulation results for CQI reporting test.

# 2	Simulation results

Figure 1 shows the median of reported CQI with the CQI table supporting 1024QAM (See Table 3 in Appendix). If we set the test points corresponding to CQI indexes supporting 1024QAM (CQI indexes = 14 and 15), the test point should be set to 27 or lager. Considering the SNR test point we are discussing in the PDSCH demodulation requirements, we propose to set SNR to 28 and 29 dB. For reference, we show our simulation results if CQI definition test with CQI table supporting 1024QAM in Table 1.

Observation: Considering the CQI index supporting 1024QAM, the test point is 27dB or higher.

Figure 1	Median of reported CQI.

Table 1	Simulation results of CQI definition test with CQI table supporting 1024QAM (CFI=3).

| SNR [dB] | Median CQI | Percentage of reported CQI indexes which is median CQI, median CQI +/- 1 | BLER at median CQI | BLER at Median CQI -1 | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| 25 | 13 | 100 | 0.02 | 0 | 1 |
| 26 | 13 | 100 | 0.01 | 0 | 1 |
| 27 | 14 | 100 | 0.41 | 0 | 1 |
| 28 | 14 | 100 | 0.12 | 0 | 1 |
| 29 | 14 | 100 | 0.04 | 0 | 1 |
| 30 | 14 | 100 | 0.02 | 0 | 1 |
| 31 | 14 | 100 | 0.01 | 0 | 1 |
| 32 | 15 | 100 | 0.86 | 0.01 | N/A |

# 3	Conclusion

Observation: Considering the CQI index supporting 1024QAM, the test point is 27dB or higher.

# References

- R4-1811378, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon.

# Appendix

Table 2	CQI to MCS table.

|  |  | CFI | 3 |
| --- | --- | --- | --- |
|  |  | PRB | 50 |
|  |  | Available REs | 6300 |
| CQI index | Code rate | Modulation |  |
| 1 | 0.08 | 2 | 0 |
| 2 | 0.19 | 2 | 2 => 1 |
| 3 | 0.44 | 2 | 6 => 3 |
| 4 | 0.37 | 4 | 11 => 5 |
| 5 | 0.6 | 4 | 15 => 7 |
| 6 | 0.55 | 6 | 18 => 9 |
| 7 | 0.65 | 6 | 21 => 11 |
| 8 | 0.75 | 6 | 23 => 13 |
| 9 | 0.85 | 6 | 24 (Note 1) => 14 |
| 10 | 0.69 | 8 | 28 => 17 |
| 11 | 0.78 | 8 | 30 => 19 |
| 12 | 0.86 | 8 | 32 => 21 |
| 13 | 0.93 | 8 | 32 (Note 2) => 21 |
| 14 | 0.83 | 10 | 34 => 23 |
| 15 | 0.93 | 10 | 36 => 25 |

Note 1: If we choose TBS from MCS with 16QAM, MCS index is 14. If we choose TBS from any MCS table, MCS index is 15.

Note 2: TBS=33 is the closest to the target code rate but it exceeds 0.925.

Table 3	CQI table supporting 1024QAM.

Table 7.2.3-4: 4-bit CQI Table 4

| CQI index | modulation | code rate x 1024 x | Efficiency x |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |