53GPP TSG-RAN WG4 Meeting 88bis	R4-1812853

Chengdu, China, 8 – 12 October 2018

Agenda Item:	7.13.1.5

Source:	Ericsson

Title:	Simplified propagation channel models for FR1 UE demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN4#88 discussed the simplification of TDL channel models for NR UE demodulation and CSI reporting requirements for FR1 and agreed with the way forward [1].

Way forward on channel models simplification for FR1 - Simplification methods - Option 1: Choose strongest paths that contribute to [X%] of total power - Option 2: Choose N paths for TDL-A and TDL-C using the frequency correlation method - Use option 1 and option 2 to generate up to 12 taps after 5 ns quantization for following FR1 models - RMS delay spread TDL-A (30 ns) - RMS delay spread TDL-B (100 ns) - RMS delay spread TDL-C (300 ns) - Further study the frequency correlation characteristics of channel models for Option 1 and 2 - e.g. The most suitable model is selected based on the visual inspection of the quantized FCF. The lower the correlation the better, preferably below 0.6 in full frequency range up to 100 MHz - Make final model down-selection in RAN4 88bis

In this contribution, we compare the option 1 and option 2 with regard to the UE demodulation performance.

# 2	Simplified TDL propagation channel models

## 2.1	Simplification procedure

As RAN4#87 discussed, the resolution of delay path grids for TDL channel models in TR38.901 [2] is 1 pico-second (1ps). To avoid the complexity of the test setting, RAN4 discussed to assume the simplification of the channel model.

We should also point of option 1 and option 2 are in principle equivalent. Therefore we use option 2 to find the optimal N values. Our simplification procedure is given as follows:

Step 1: Choose N strongest paths NLOS paths from the original PDPs shows in TR38.901. Step 2: Apply the desired delay spread after removing the weak paths. Step 3: Round the path delay position close to the grid. Set the grid step size to 5 ns. Step 4: If two or more paths are rounded to the same grid τ, the path power at τ is set as a sum of all the path power values (in linear) rounded to τ. Step 5: Normalize the power so that the relative power of the strongest path becomes 0dB.

## 2.2	Simplified TDL-A (DS=30ns)

Table 1 shows the simplified delay profile of TDL-A DS=30ns (TDLA30). To keep the original delay profile, we chose 18 strong paths. After combining the overlapped paths, the simplified path profile has 12 paths, possible maximum number of paths according to TE vendors. Figure 1 compares the original path profile and simplified path profile with regard to frequency correlation, and it is observed no strong frequency correlation.

Table 1	Tap delay and relative power for the simplified TDL-A with Desired DS=30ns.

|  | Tap delay [ns] | Relative power [dB] |
| --- | --- | --- |
| 1 | 0 | -16.1 |
| 2 | 15 | 0 |
| 3 | 20 | -4 |
| 4 | 25 | -10.2 |
| 5 | 50 | -18.6 |
| 6 | 65 | -9.3 |
| 7 | 75 | -15.1 |
| 8 | 85 | -12.1 |
| 9 | 100 | -14 |
| 10 | 135 | -15.4 |
| 11 | 150 | -18.9 |
| 12 | 165 | -19.3 |
|  |  | DS=29.5ns |

Figure 1	Simplified TDL-A (Desired DS=30ns). PDP and frequency correlation characteristics.

## 2.3	Simplified TDL-B (DS=100ns)

Table 2 shows the simplified delay profile of TDL-B DS=100ns (TDLB100). As same as TDLA30, we select X strong paths so that the final path profile has up to 12 paths. According to our investigation, X=16. Figure 2 compares the original path profile and simplified path profile with regard to frequency correlation. We observed the frequency correlation exceeds 0.5 around 80MHz. However since the current FR1 simulation assumption sets 10MHz and 40MHz CBW, we don’t think it is an issue. However if RAN4 will introduce the test case CBW>80MHz for FR1, we may need to be careful to choose the channel model.

Table 2	Tap delay and relative power for the simplified TDL-B with Desired DS=100ns.

|  | Tap delay [ns] | Relative power [dB] |
| --- | --- | --- |
| 1 | 0 | -0.5 |
| 2 | 20 | -2.7 |
| 3 | 35 | -1.1 |
| 4 | 50 | -1.7 |
| 5 | 60 | 0 |
| 6 | 85 | -4.2 |
| 7 | 95 | -9.4 |
| 8 | 180 | -5.3 |
| 9 | 210 | -6.2 |
| 10 | 255 | -8 |
| 11 | 295 | -2.4 |
| 12 | 335 | -8.1 |
|  |  | DS=99.7ns |

Figure 2	Simplified TDL-B (Desired DS=100ns). PDP and frequency correlation characteristics.

## 2.3	Simplified TDL-C (DS=300ns)

Table 3 shows the simplified delay profile of TDL-C DS=300ns (TDLC300). We select X strong paths so that the final path profile has up to 12 paths. According to our investigation, X=13. Note if we select X=14 paths, the final path profile has more than 12 paths. Figure 3 compares the original path profile and simplified path profile with regard to frequency correlation. We observed the frequency correlation exceeds 0.5 from 0Hz to 30MHz. Original TDL-C with DS=300ns also shows similar frequency correlation.

Table 3	Tap delay and relative power for the simplified TDL-C with Desired DS=300ns.

|  | Tap delay [ns] | Relative power [dB] |
| --- | --- | --- |
| 1 | 0 | -4.4 |
| 2 | 150 | -1.2 |
| 3 | 155 | 0 |
| 4 | 165 | -5.2 |
| 5 | 450 | 0 |
| 6 | 455 | -2.2 |
| 7 | 465 | -2.3 |
| 8 | 560 | -7.1 |
| 9 | 870 | -5.1 |
| 10 | 925 | -6.8 |
| 11 | 1535 | -8.7 |
|  |  | DS=300.0ns |

Figure 3	Simplified TDL-C (Desired DS=300ns). PDP and frequency correlation characteristics.

# 3	Conclusion

We propose to set the following path profiles for UE demodulation requirements.

Tapped Delay Line A model with DS=30ns (TDLA30)

| Tap delay [ns] | Relative power [dB] |
| --- | --- |
| 0 | -16.1 |
| 15 | 0 |
| 20 | -4 |
| 25 | -10.2 |
| 50 | -18.6 |
| 65 | -9.3 |
| 75 | -15.1 |
| 85 | -12.1 |
| 100 | -14 |
| 135 | -15.4 |
| 150 | -18.9 |
| 165 | -19.3 |

Tapped Delay Line B model with DS=100ns (TDLB100)

| Tap delay [ns] | Relative power [dB] |
| --- | --- |
| 0 | -0.5 |
| 20 | -2.7 |
| 35 | -1.1 |
| 50 | -1.7 |
| 60 | 0 |
| 85 | -4.2 |
| 95 | -9.4 |
| 180 | -5.3 |
| 210 | -6.2 |
| 255 | -8 |
| 295 | -2.4 |
| 335 | -8.1 |

Tapped Delay Line C model with DS=300ns (TDLC300)

| Tap delay [ns] | Relative power [dB] |
| --- | --- |
| 0 | -4.4 |
| 150 | -1.2 |
| 155 | 0 |
| 165 | -5.2 |
| 450 | 0 |
| 455 | -2.2 |
| 465 | -2.3 |
| 560 | -7.1 |
| 870 | -5.1 |
| 925 | -6.8 |
| 1535 | -8.7 |

# 4	References

R4-1811725, “Way forward on channel model for NR performance requirements”, Huawei, Spirent.

3GPP TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.

# Appendix

Table 7.7.2-1. TDL-A

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0.0000 | -13.4 | Rayleigh |
| 2 | 0.3819 | 0 | Rayleigh |
| 3 | 0.4025 | -2.2 | Rayleigh |
| 4 | 0.5868 | -4 | Rayleigh |
| 5 | 0.4610 | -6 | Rayleigh |
| 6 | 0.5375 | -8.2 | Rayleigh |
| 7 | 0.6708 | -9.9 | Rayleigh |
| 8 | 0.5750 | -10.5 | Rayleigh |
| 9 | 0.7618 | -7.5 | Rayleigh |
| 10 | 1.5375 | -15.9 | Rayleigh |
| 11 | 1.8978 | -6.6 | Rayleigh |
| 12 | 2.2242 | -16.7 | Rayleigh |
| 13 | 2.1718 | -12.4 | Rayleigh |
| 14 | 2.4942 | -15.2 | Rayleigh |
| 15 | 2.5119 | -10.8 | Rayleigh |
| 16 | 3.0582 | -11.3 | Rayleigh |
| 17 | 4.0810 | -12.7 | Rayleigh |
| 18 | 4.4579 | -16.2 | Rayleigh |
| 19 | 4.5695 | -18.3 | Rayleigh |
| 20 | 4.7966 | -18.9 | Rayleigh |
| 21 | 5.0066 | -16.6 | Rayleigh |
| 22 | 5.3043 | -19.9 | Rayleigh |
| 23 | 9.6586 | -29.7 | Rayleigh |

Table 7.7.2-2. TDL-B

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0.0000 | 0 | Rayleigh |
| 2 | 0.1072 | -2.2 | Rayleigh |
| 3 | 0.2155 | -4 | Rayleigh |
| 4 | 0.2095 | -3.2 | Rayleigh |
| 5 | 0.2870 | -9.8 | Rayleigh |
| 6 | 0.2986 | -1.2 | Rayleigh |
| 7 | 0.3752 | -3.4 | Rayleigh |
| 8 | 0.5055 | -5.2 | Rayleigh |
| 9 | 0.3681 | -7.6 | Rayleigh |
| 10 | 0.3697 | -3 | Rayleigh |
| 11 | 0.5700 | -8.9 | Rayleigh |
| 12 | 0.5283 | -9 | Rayleigh |
| 13 | 1.1021 | -4.8 | Rayleigh |
| 14 | 1.2756 | -5.7 | Rayleigh |
| 15 | 1.5474 | -7.5 | Rayleigh |
| 16 | 1.7842 | -1.9 | Rayleigh |
| 17 | 2.0169 | -7.6 | Rayleigh |
| 18 | 2.8294 | -12.2 | Rayleigh |
| 19 | 3.0219 | -9.8 | Rayleigh |
| 20 | 3.6187 | -11.4 | Rayleigh |
| 21 | 4.1067 | -14.9 | Rayleigh |
| 22 | 4.2790 | -9.2 | Rayleigh |
| 23 | 4.7834 | -11.3 | Rayleigh |

Table 7.7.2-3. TDL-C

| Tap # | Normalized delays | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -4.4 | Rayleigh |
| 2 | 0.2099 | -1.2 | Rayleigh |
| 3 | 0.2219 | -3.5 | Rayleigh |
| 4 | 0.2329 | -5.2 | Rayleigh |
| 5 | 0.2176 | -2.5 | Rayleigh |
| 6 | 0.6366 | 0 | Rayleigh |
| 7 | 0.6448 | -2.2 | Rayleigh |
| 8 | 0.6560 | -3.9 | Rayleigh |
| 9 | 0.6584 | -7.4 | Rayleigh |
| 10 | 0.7935 | -7.1 | Rayleigh |
| 11 | 0.8213 | -10.7 | Rayleigh |
| 12 | 0.9336 | -11.1 | Rayleigh |
| 13 | 1.2285 | -5.1 | Rayleigh |
| 14 | 1.3083 | -6.8 | Rayleigh |
| 15 | 2.1704 | -8.7 | Rayleigh |
| 16 | 2.7105 | -13.2 | Rayleigh |
| 17 | 4.2589 | -13.9 | Rayleigh |
| 18 | 4.6003 | -13.9 | Rayleigh |
| 19 | 5.4902 | -15.8 | Rayleigh |
| 20 | 5.6077 | -17.1 | Rayleigh |
| 21 | 6.3065 | -16 | Rayleigh |
| 22 | 6.6374 | -15.7 | Rayleigh |
| 23 | 7.0427 | -21.6 | Rayleigh |
| 24 | 8.6523 | -22.8 | Rayleigh |