3GPP TSG-RAN WG4 #88bis	R4-1812983

# Chengdu, China, 8 - 12 October, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 2 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for scheduling availability in 38.133 (section 8.1.7.2, 8.1.7.3, 8.5.7.3, 9.2.5.3.2 and 9.2.5.3.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Scheduling restriction is defined in 38.133 to deal with the collision of RRM measurement/RLM-RS/BFD/L1-RSRP and PDSCH/PDCCH/PUCCH/PUSCH. However, TRS and CSI-RS for CQI also should be considered in scheduling restriction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TRS also should be considered in scheduling restriction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.7.2, 8.1.7.3, 8.5.7.3, 9.2.5.3.2, 9.2.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

#### 8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For UE which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to radio link monitoring based on SSB as RLM-RS.

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured for radio link monitoring.

When intra-band carrier aggregation is performed, the scheduling restrictions apply to all serving cells on the band due to radio link monitoring performed on FR1 serving PCell or PSCell in the same band. When inter-band carrier aggregation within FR1 is performed, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to radio link monitoring performed on FR1 serving PCell or PSCell in different bands.

#### 8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2

The following scheduling restriction applies due to radio link monitoring on an FR2 serving PCell and/or PSCell.

-	If UE is not provided high layer parameter RadioLinkMonitoringRS and UE is provided by higher layer parameter TCI-state for PDCCH SSB/CSI-RS that has QCL-Type D, or if the SSB/CSI-RS configured for RLM is QCL-Type D with DM-RS for PDCCH

- There are no scheduling restrictions due to radio link monitoring performed with a same subcarrier spacing as PDSCH/PDCCH.

- Otherwise

- The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on RLM-RS symbols to be measured for radio link monitoring, except for RMSI PDCCH/PDSCH and PDCCH/PDSCH which is not required to be received by RRC_CONNECTED mode UE.

When intra-band carrier aggregation is performed, for other serving cells on the band than FR2 serving PCell or PSCell in the same band, the following scheduling restriction applies due to radio link monitoring on an FR2 serving PCell and/or PSCell.

-	If the RLM-RS is type-D QCLed with active TCI state for PDCCH/PDSCH, and N=1 applies for the RLM-RS as specified in section 8.1.2.2 if the RLM-RS is SSB and in section 8.1.3.2 if the RLM-RS is CSI-RS

-	There are no scheduling restrictions due to radio link monitoring performed with a same subcarrier spacing as PDSCH/PDCCH.

-	When performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH, for UE which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring. For UE which do not support simultaneousRxDataSSB-DiffNumerology [14] the UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured for radio link monitoring.

-	Otherwise

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on RLM-RS symbols to be measured for radio link monitoring.

Editor’s Note: FFS scheduling restrictions for inter-band carrier aggregation will be defined depending on band combination in future.

---END OF CHANGE 1-

---START OF CHANGE 2---

#### 8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to beam failure detection based on SSB as BFD-RS. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to beam failure detection based on SSB configured as BFD-RS.

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured for beam failure detection.

The following scheduling restrictions apply due to beam failure detection based on CSI-RS as BFD-RS.

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on CSI-RS symbols to be measured for beam failure detection.

When intra-band carrier aggregation in FR1 is configured,  the scheduling restrictions apply to all SCells that are aggregated in the same band as the PCell or PSCell. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell or PSCell is configured.

#### 8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2

The following scheduling restriction applies due to beam failure detection on an FR2 PCell and/or PSCell.

-	If UE is not provided higher layer parameter failureDetectionResources and UE is provided by higher layer parameter TCI-state for PDCCH SSB/CSI-RS that has QCL-Type D, or if the SSB/CSI-RS for BFD is QCL-Type D with DM-RS for PDCCH

- There are no scheduling restrictions due to beam failure detection performed with same SCS as PDSCH/PDCCH.

- Otherwise

- The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on BFD-RS symbols to be measured for beam failure detection, except for RMSI PDCCH/PDSCH and PDCCH/PDSCH which is not required to be received by RRC_CONNECTED mode UE.

When intra-band carrier aggregation is configured, the following scheduling restrictions apply to all SCells configured in the same band as the PCell and/or PSCell on which beam failure is detected.

-	For the case where no RSs are provided for BFD, or where BFD-RS is explicitly configured and is QCLed with active TCI state for PDCCH/PDSCH

-	There are no scheduling restrictions due to beam failure detection performed with a same SCS as PDSCH/PDCCH.

-	When performing beam failure detection with a different SCS than PDSCH/PDCCH, for UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to beam failure detection. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured for beam failure detection.

-	For the case where BFD-RS is explicitly configured and is not QCLed with active TCI state for PDCCH/PDSCH

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on BFD-RS symbols to be measured for beam failure detection.

Editor’s Note: FFS scheduling restrictions for inter-band carrier aggregation will be defined depending on band combination in future.

---END OF CHANGE 2-

---START OF CHANGE 3---

##### 9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to measurements. For UE which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols and 1 data symbol after each consecutive SSB symbols within SMTC window duration. If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When intra-band carrier aggregation is performed, the scheduling restrictions apply to all serving cells on the band. When inter-band carrier aggregation within FR1 is performed, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to measurements performed on FR1 serving cell frequency layer in different bands.

##### 9.2.5.3.3	Scheduling availability of UE performing measurements on FR2

The following scheduling restriction applies due to SS-RSRP or SS-SINR measurement on an FR2 intra-frequency cell

The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 symbol before each consecutive SSB symbols and 1 data symbol after each consecutive SSB symbols within SMTC window duration (The signaling deriveSSB_IndexFromCell is always enabled for FR2) . If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

The following scheduling restriction applies to SS-RSRQ measurement on an FR2 intra-frequency cell

-	UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB/RSSI symbols and 1 data symbol after each consecutive SSB/RSSI symbols within SMTC window duration (The signaling deriveSSB_IndexFromCellc is always enabled for FR2). If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When intra-band carrier aggregation is performed, the scheduling restrictions apply to all serving cells on the band. When inter-band carrier aggregation within FR2 is performed, the scheduling restrictions apply to all serving cells on the bands.

Editor’s Note: FFS scheduling restrictions for inter-band carrier aggregation will be defined depending on band combination in future.

---END OF CHANGE 3-