3GPP TSG-RAN WG4 Meeting #88bis                                                     R4-1813211

# Chengdu, China, 8 – 12 Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRM TC 20B: EN-DC interruptions due to active BWP switching (DCI- and timer-based switch) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test cases are being specified for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add interruption at BWP switch test cases for the following cases: - DCI- and timer-based BWP switch - PSCell in FR1, SCell in FR1 - FDD BWP switch without SCS change (SCS 15 kHz) - TDD BWP switch without SCS change (SCS 30 kHz) - PSCell in FR2, SCell in FR2 - TDD BWP switch without SCS change (SCS 120 kHz) BWP is switched on PSCell in the test, and SCell is the victim cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for BWP switch interruptions will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.2.1, A.5.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ...TS38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### << Start of change 1 >>

### A.4.5.2.1.a		Interruptions at DCI- and timer-based active BWP switch

### A.4.5.2.1.a.1	Interruption at DCI- and timer-based active BWP switch with FDD PSCell and SCell, FR1, 15 kHz SCS

#### A.4.5.2.1.a.1.1		Test Purpose and Environment

The purpose of this test is to verify that when the active BWP is switched on an NR cell in EN-DC based on DCI command and due to expiration of bwp-InactivityTimer, possible interruption that the BWP switch causes to another NR cell happens within the active BWP switch delay defined in section 8.6.2 and does not exceed the defined interruption duration in section 8.2.1.2.7.

The test parameters are given in tables A.4.5.2.1.a.1 -1 and A.4.5.2.1.a.1 -2 below. There are three cells in the test: Cell1, Cell2 and Cell3. Cell1 is an FDD E-UTRA PCell, Cell2 is an FDD PSCell in FR1 and Cell3 is an SCG FDD SCell in FR1. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During the whole test the UE is connected to all Cell1, Cell2 and Cell3. The UE shall be continuously scheduled in non-DRX mode on Cell1 and Cell3 throughout the whole test. The UE shall be continuously scheduled on Cell2 in non-DRX mode during T1 and T2 but shall not be scheduled at all during time period T3.

At the beginning of the test the UE shall operate on the initial BWP (BWP0) on Cell2 (PSCell) and Cell3 (SCell). BWP0 shall also be seen as the default BWP. Cell2 shall also have a dedicated BWP configuration (BWP1) within its RRC configuration. At the beginning of T1 the UE shall be able to receive PDSCH on BWP0 on Cell2 and Cell3.

The point of time at which the UE receives DCI format 1_1 at slot n with the BWP indicator field triggering the change of active BWP on Cell2 defines the start of time period T2. The UE shall switch from BWP0 to BWP1 during time duration T2, and shall be able to receive PDSCH on BWP1 no later than at slot n+X.

The stop of scheduling on Cell2 defines the start of time period T3. Upon the expiry of bwp-InactivityTimer at slot m, the UE shall be able to switch its BWP (BWP1) back to the default BWP (BWP0) and be able to receive PDSCH on BWP0 no later than at slot m+Y.

The test equipment verifies that the potential interruption to Cell3 at DCI-based BWP switch during T2 and at timer-based BWP switch during T3 happens within the active BWP switch delay defined in section 8.6.2, and that the interruption does not exceed the duration defined in section 8.2.1.2.7.

Table A.4.5.2.1.a.1-1: General test parameters for interruption at active BWP switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA RF channel |
| NR RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active cells |  | Cell1, Cell2, Cell3 | PCell on E-UTRA RF channel number 1, PSCell on NR RF channel number 1 and SCell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| bwp-InactivityTimer | ms | 10 | The duration after which the UE falls back to the default BWP according to section 6.3.2 of TS 38.331 |
| Default BWP |  | BWP0 | Initial BWP is set as the first active BWP on Cell1 and Cell2. Configuration for BWP0 is defined in Table A.4.5.2.1.a.1-3. |
| Dedicated BWP |  | BWP1 | UE shall switch Cell2 BWP from BWP0 to BWP1 during the test. Configuration for BWP1 is defined in Table A.4.5.2.1.a.1-3. |
| T1 | s | 1 |  |
| T2 | s | 1 |  |
| T3 | s | 1 |  |

Table A.4.5.2.1.a.1-2: Cell specific test parameters for interruption at active BWP switch

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |  |  | Cell3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 |  | T3 | T1 | T2 | T3 |
| Cell type |  | E-UTRA PCell |  |  | NR PSCell |  |  |  |  | NR SCell |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | - |  |  |  |  | - |  |  |
| NR RF Channel Number |  | - |  |  | 1 |  |  |  |  | 2 |  |  |
| Duplex mode |  | FDD |  |  | FDD |  |  |  |  | FDD |  |  |
| Bandwidth part |  | - |  |  | BWP0 | BWP0 >> BWP1Note1 |  | BWP1 >> BWP0Note1 |  | BWP0 |  |  |
| PDSCH parameters |  | TBD |  |  | TBD |  |  |  |  | TBD |  |  |
| PDCCH parameters |  | TBD |  |  | TBD |  |  |  |  | TBD |  |  |
| OCNG pattern |  | TBD |  |  | TBD |  |  |  |  | TBD |  |  |
| SMTC configuration |  | - |  |  | SMTC.1 FR1 (As specified in section A.3.2.2.1.1) |  |  |  |  | SMTC.1 FR1 (As specified in section A.3.2.2.1.1) |  |  |
| NocNote 2 | dBm/15 kHz | [-104] |  |  | [-104] |  |  |  |  | [-104] |  |  |
| RSRP Note 3 | dBm/15 kHz | [-87] |  |  | [-87] |  |  |  |  | [-87] |  |  |
| Ês/Iot | dB | [17] |  |  | [17] |  |  |  |  | [17] |  |  |
| SCH_RP Note 3 | dBm/15 kHz | [-87] |  |  | [-87] |  |  |  |  | [-87] |  |  |
| Ês/Noc | dB | [17] |  |  | [17] |  |  |  |  | [17] |  |  |
| Propagation Condition |  | [AWGN] |  |  | [AWGN] |  |  |  |  | [AWGN] |  |  |
| Correlation Matrix and Antenna Configuration |  | [1x2] |  |  | [1x2] |  |  |  |  | [1x2] |  |  |
| Time offset to Cell2 | s | - |  |  | - |  |  |  |  | [33] |  |  |
| Note 1: During T2, BWP of Cell2 is switched from BWP0 to BWP1. During T3, BWP of Cell2 is switched from BWP1 to BWP0 due to timer expiration. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.2.1.a.1-3: BWP configurations

| Parameter | Unit | BWP0 | BWP1 |
| --- | --- | --- | --- |
| Subcarrier spacing | kHz | [15] | [15] |
| Bandwidth | PRB | [24] | [51] |
| Configured k0 value | slot | [TBD] | [TBD] |
| Configured k1 value | slot | [TBD] | [TBD] |

#### A.4.5.2.1.a.1.2		Test Requirements

For Cell2 (PSCell):

During T1, the UE shall be able to receive PDSCH on BWP0 of Cell2 (PSCell).

During T2, the UE shall be able to receive PDSCH on BWP1 of Cell2 on which BWP switch occurs no later than at slot n+X, where n is the slot where the UE receives the DCI triggering BWP switching on Cell2 and X is [TBD].

During T3, the UE shall be able to switch its BWP (BWP1) back to the default BWP (BWP0) and receive PDSCH on BWP1 of Cell2 no later than at slot m+Y, where m is the slot where bwp-InactivityTimer expires and Y is [TBD].

For Cell3 (SCell):

During T1, T2 and T3, the UE shall be able to receive PDSCH on BWP0 of Cell3 (SCell).

The interruption to Cell3 during T2 due to DCI-based BWP switch and during T3 due to expiration of bwp-InactivityTimer shall happen within the active BWP switch delay defined in section 8.6.2 and shall not exceed the interruption duration defined in section 8.2.1.2.7.

### A.4.5.2.1.a.2	Interruption at DCI- and timer-based active BWP switch with TDD PSCell and SCell, FR1, 30 kHz SCS

#### A.4.5.2.1.a.2.1		Test Purpose and Environment

The purpose of this test is to verify that when the active BWP is switched on an NR cell in EN-DC based on DCI command and due to expiration of bwp-InactivityTimer, possible interruption that the BWP switch causes to another NR cell happens within the active BWP switch delay defined in section 8.6.2 and does not exceed the defined interruption duration in section 8.2.1.2.7.

The test parameters are given in tables A.4.5.2.1.a.2 -1 and A.4.5.2.1.a.2 -2 below. There are three cells in the test: Cell1, Cell2 and Cell3. Cell1 is a TDD E-UTRA PCell, Cell2 is a TDD PSCell in FR1 and Cell3 is an SCG TDD SCell in FR1. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During the whole test the UE is connected to all Cell1, Cell2 and Cell3. The UE shall be continuously scheduled in non-DRX mode on Cell1 and Cell3 throughout the whole test. The UE shall be continuously scheduled on Cell2 in non-DRX mode during T1 and T2 but shall not be scheduled at all during time period T3.

At the beginning of the test the UE shall operate on the initial BWP (BWP0) on Cell2 (PSCell) and Cell3 (SCell). BWP0 shall also be seen as the default BWP. Cell2 shall also have a dedicated BWP configuration (BWP1) within its RRC configuration. At the beginning of T1 the UE shall be able to receive PDSCH on BWP0 on Cell2 and Cell3.

The point of time at which the UE receives DCI format 1_1 at slot n with the BWP indicator field triggering the change of active BWP on Cell2 defines the start of time period T2. The UE shall switch from BWP0 to BWP1 during time duration T2, and shall be able to receive PDSCH on BWP1 no later than at slot n+X.

The stop of scheduling on Cell2 defines the start of time period T3. Upon the expiry of bwp-InactivityTimer at slot m, the UE shall be able to switch its BWP (BWP1) back to the default BWP (BWP0) and be able to receive PDSCH on BWP0 no later than at slot m+Y.

The test equipment verifies that the potential interruption to Cell3 at DCI-based BWP switch during T2 and at timer-based BWP switch during T3 happens within the active BWP switch delay defined in section 8.6.2, and that the interruption does not exceed the duration defined in section 8.2.1.2.7.

Table A.4.5.2.1.a.2-1: General test parameters for interruption at active BWP switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA RF channel |
| NR RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active cells |  | Cell1, Cell2, Cell3 | PCell on E-UTRA RF channel number 1, PSCell on NR RF channel number 1 and SCell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| bwp-InactivityTimer | ms | 10 | The duration after which the UE falls back to the default BWP according to section 6.3.2 of TS 38.331 |
| Default BWP |  | BWP0 | Initial BWP is set as the first active BWP on Cell1 and Cell2. Parameters for BWP0 are defined in Table A.4.5.2.1.a.2-3. |
| Dedicated BWP |  | BWP1 | UE shall switch Cell2 BWP from BWP0 to BWP1 during the test. Parameters for BWP1 are defined in Table A.4.5.2.1.a.2-3. |
| T1 | s | 1 |  |
| T2 | s | 1 |  |
| T3 | s | 1 |  |

Table A.4.5.2.1.a.2-2: Cell specific test parameters for interruption at active BWP switch

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |  |  | Cell3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 |  | T3 | T1 | T2 | T3 |
| Cell type |  | E-UTRA PCell |  |  | NR PSCell |  |  |  |  | NR SCell |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | - |  |  |  |  | - |  |  |
| NR RF Channel Number |  | - |  |  | 1 |  |  |  |  | 2 |  |  |
| Duplex mode |  | TDD |  |  | TDD |  |  |  |  | TDD |  |  |
| Bandwidth part |  | - |  |  | BWP0 | BWP0 >> BWP1Note1 |  | BWP1 >> BWP0Note1 |  | BWP0 |  |  |
| Special subframe configurationNote2 |  | 6 |  |  | 6 |  |  |  |  | 6 |  |  |
| Uplink-downlink configuration Note2 |  | 1 |  |  | 1 |  |  |  |  | 1 |  |  |
| PDSCH parameters |  | TBD |  |  | TBD |  |  |  |  | TBD |  |  |
| PDCCH parameters |  | TBD |  |  | TBD |  |  |  |  | TBD |  |  |
| OCNG pattern |  | TBD |  |  | TBD |  |  |  |  | TBD |  |  |
| SMTC configuration |  | - |  |  | SMTC.1 FR1 (As specified in section A.3.2.2.1.1) |  |  |  |  | SMTC.1 FR1 (As specified in section A.3.2.2.1.1) |  |  |
| NocNote 2 | dBm/15 kHz | [-104] |  |  | [-104] |  |  |  |  | [-104] |  |  |
| RSRP Note 3 | dBm/15 kHz | [-87] |  |  | [-87] |  |  |  |  | [-87] |  |  |
| Ês/Iot | dB | [17] |  |  | [17] |  |  |  |  | [17] |  |  |
| SCH_RP Note 3 | dBm/15 kHz | [-87] |  |  | [-87] |  |  |  |  | [-87] |  |  |
| Ês/Noc | dB | [17] |  |  | [17] |  |  |  |  | [17] |  |  |
| Propagation Condition |  | [AWGN] |  |  | [AWGN] |  |  |  |  | [AWGN] |  |  |
| Correlation Matrix and Antenna Configuration |  | [1x2] |  |  | [1x2] |  |  |  |  | [1x2] |  |  |
| Time offset to Cell2 | s | - |  |  | - |  |  |  |  | [33] |  |  |
| Note 1: During T2, BWP of Cell2 is switched from BWP0 to BWP1. During T3, BWP of Cell2 is switched from BWP1 to BWP0 due to timer expiration. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Special subframe and uplink-downlink configurations are defined in Tables 4.2-1 and 4.2-2 in TS 36.211 [23]. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.5.2.1.a.2-3: BWP configurations

| Parameter | Unit | BWP0 | BWP1 |
| --- | --- | --- | --- |
| Subcarrier spacing | kHz | [30] | [30] |
| Bandwidth | PRB | [24] | [51] |
| Configured k0 value | slot | [TBD] | [TBD] |
| Configured k1 value | slot | [TBD] | [TBD] |

#### A.4.5.2.1.a.2.2		Test Requirements

For Cell2 (PSCell):

During T1, the UE shall be able to receive PDSCH on BWP0 of Cell2 (PSCell).

During T2, the UE shall be able to receive PDSCH on BWP1 of Cell2 on which BWP switch occurs no later than at slot n+X, where n is the slot where the UE receives the DCI triggering BWP switching on Cell2 and X is [TBD].

During T3, the UE shall be able to switch its BWP (BWP1) back to the default BWP (BWP0) and receive PDSCH on BWP1 of Cell2 no later than at slot m+Y, where m is the slot where bwp-InactivityTimer expires and Y is [TBD].

For Cell3 (SCell):

During T1, T2 and T3, the UE shall be able to receive PDSCH on BWP0 of Cell3 (SCell).

The interruption to Cell3 during T2 due to DCI-based BWP switch and during T3 due to expiration of bwp-InactivityTimer shall happen within the active BWP switch delay defined in section 8.6.2 and shall not exceed the interruption duration defined in section 8.2.1.2.7.

<<End of change 1>>

<<Start of change 2>>

### A.5.5.2.1.a		Interruptions at DCI- and timer-based active BWP switch

A.5.5.2.1.a.1	Interruption at DCI- and timer-based active BWP switch with TDD PSCell and SCell, FR2, 120 kHz SCS

#### A.5.5.2.1.a.1.1		Test Purpose and Environment

The purpose of this test is to verify that when the active BWP is switched on an NR cell in EN-DC based on DCI command and due to expiration of bwp-InactivityTimer, possible interruption that the BWP switch causes to another NR cell happens within the active BWP switch delay defined in section 8.6.2 and does not exceed the defined interruption duration in section 8.2.1.2.7. This test verifies the interruption requirements for the case when BWP switch involves change of bandwidth and center frequency of the BWP.

The test parameters are given in tables A.5.5.2.1.a.1 -1 and A.5.5.2.1.a.1-2 below. There are three cells in the test: Cell1, Cell2 and Cell3. Cell1 is a TDD E-UTRA PCell, Cell2 is a TDD PSCell in FR2 and Cell3 is an SCG TDD SCell in FR2. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During the whole test the UE is connected to all Cell1, Cell2 and Cell3. The UE shall be continuously scheduled in non-DRX mode on Cell1 and Cell3 throughout the whole test. The UE shall be continuously scheduled on Cell2 in non-DRX mode during T1 and T2 but shall not be scheduled at all during time period T3.

At the beginning of the test the UE shall operate on the initial BWP (BWP0) on Cell2 (PSCell) and Cell3 (SCell). BWP0 shall also be seen as the default BWP. Cell2 shall also have a dedicated BWP configuration (BWP1) within its RRC configuration. At the beginning of T1 the UE shall be able to receive PDSCH on BWP0 on Cell2 and Cell3.

The point of time at which the UE receives DCI format 1_1 at slot n with the BWP indicator field triggering the change of active BWP on Cell2 defines the start of time period T2. The UE shall switch from BWP0 to BWP1 during time duration T2, and shall be able to receive PDSCH on BWP1 no later than at slot n+X.

The stop of scheduling on Cell2 defines the start of time period T3. Upon the expiry of bwp-InactivityTimer at slot m, the UE shall be able to switch its BWP (BWP1) back to the default BWP (BWP0) and be able to receive PDSCH on BWP0 no later than at slot m+Y.

The test equipment verifies that the potential interruption to Cell3 at DCI-based BWP switch during T2 and at timer-based BWP switch during T3 happens within the active BWP switch delay defined in section 8.6.2, and that the interruption does not exceed the duration defined in section 8.2.1.2.7.

Table A.5.5.2.1.a.1-1: General test parameters for interruption at active BWP switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA RF channel |
| NR RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active cells |  | Cell1, Cell2, Cell3 | PCell on E-UTRA RF channel number 1, PSCell on NR RF channel number 1 and SCell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| bwp-InactivityTimer | ms | 10 | The duration after which the UE falls back to the default BWP according to section 6.3.2 of TS 38.331 |
| Default BWP |  | BWP0 | Initial BWP is set as the first active BWP on Cell1 and Cell2. Parameters for BWP0 are defined in Table A.4.5.2.1.a.1-3. |
| Dedicated BWP |  | BWP1 | UE shall switch Cell2 BWP from BWP0 to BWP1 during the test. Parameters for BWP1 are defined in Table A.4.5.2.1.a.1-3. |
| T1 | s | 1 |  |
| T2 | s | 1 |  |
| T3 | s | 1 |  |

Table A.5.5.2.1.a.1-2: Cell specific test parameters for interruption at active BWP switch

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |  |  | Cell3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 |  | T3 | T1 | T2 | T3 |
| Cell type |  | E-UTRA PCell |  |  | NR PSCell |  |  |  |  | NR SCell |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | - |  |  |  |  | - |  |  |
| NR RF Channel Number |  | - |  |  | 1 |  |  |  |  | 2 |  |  |
| Duplex mode |  | TDD |  |  | TDD |  |  |  |  | TDD |  |  |
| Bandwidth part |  | - |  |  | BWP0 | BWP0 >> BWP1Note1 |  | BWP1 >> BWP0Note1 |  | BWP0 |  |  |
| Special subframe configurationNote2 |  | 6 |  |  | 6 |  |  |  |  | 6 |  |  |
| Uplink-downlink configuration Note2 |  | 1 |  |  | 1 |  |  |  |  | 1 |  |  |
| PDSCH parameters |  | TBD |  |  | TBD |  |  |  |  | TBD |  |  |
| PDCCH parameters |  | TBD |  |  | TBD |  |  |  |  | TBD |  |  |
| OCNG pattern |  | TBD |  |  | TBD |  |  |  |  | TBD |  |  |
| SMTC configuration |  | - |  |  | SMTC.1 FR2 (As specified in section A.3.2.2.2.1) |  |  |  |  | SMTC.1 FR2 (As specified in section A.3.2.2.2.1) |  |  |
| NocNote 2 | dBm/15 kHz | [-104] |  |  | [-104] |  |  |  |  | [-104] |  |  |
| RSRP Note 3 | dBm/15 kHz | [-87] |  |  | [-87] |  |  |  |  | [-87] |  |  |
| Ês/Iot | dB | [17] |  |  | [17] |  |  |  |  | [17] |  |  |
| SCH_RP Note 3 | dBm/15 kHz | [-87] |  |  | [-87] |  |  |  |  | [-87] |  |  |
| Ês/Noc | dB | [17] |  |  | [17] |  |  |  |  | [17] |  |  |
| Propagation Condition |  | [AWGN] |  |  | [AWGN] |  |  |  |  | [AWGN] |  |  |
| Correlation Matrix and Antenna Configuration |  | [1x2] |  |  | [1x2] |  |  |  |  | [1x2] |  |  |
| Time offset to Cell2 | s | - |  |  | - |  |  |  |  | [33] |  |  |
| Note 1: During T2, BWP of Cell2 is switched from BWP0 to BWP1. During T3, BWP of Cell2 is switched from BWP1 to BWP0 due to timer expiration. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Special subframe and uplink-downlink configurations are defined in Tables 4.2-1 and 4.2-2 in TS 36.211 [23]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.5.5.2.1.a.1-3: BWP parameters

| Parameter | Unit | BWP0 | BWP1 |
| --- | --- | --- | --- |
| Subcarrier spacing | kHz | 120 | 120 |
| Bandwidth | PRB | [24] | [51] |
| Configured k0 value | slot | [TBD] | [TBD] |
| Configured k1 value | slot | [TBD] | [TBD] |

#### A.5.5.2.1.a.1.2		Test Requirements

For Cell2 (PSCell):

During T1, the UE shall be able to receive PDSCH on BWP0 of Cell2 (PSCell).

During T2, the UE shall be able to receive PDSCH on BWP1 of Cell2 on which BWP switch occurs no later than at slot n+X, where n is the slot where the UE receives the DCI triggering BWP switching on Cell2 and X is [TBD].

During T3, the UE shall be able to switch its BWP (BWP1) back to the default BWP (BWP0) and receive PDSCH on BWP1 of Cell2 no later than at slot m+Y, where m is the slot where bwp-InactivityTimer expires and Y is [TBD].

For Cell3 (SCell):

During T1, T2 and T3, the UE shall be able to receive PDSCH on BWP0 of Cell3 (SCell).

The interruption to Cell3 during T2 due to DCI-based BWP switch and during T3 due to expiration of bwp-InactivityTimer shall happen within the active BWP switch delay defined in section 8.6.2 and shall not exceed the interruption duration defined in section 8.2.1.2.7.

<< End of change 2 >>