3GPP TSG-RAN WG4 Meeting #88bis		R4-1813242
Chengdu, China, 8 – 12 October 2018

Agenda item:	7.13.2.5

Source:	Keysight Technologies

Title:	Channel model for BS demodulation

Document for:	Approval

# 1	Introduction

In RAN4#88 Way forward document [1] was agreed to “reuse the conclusions from UE demodulation discussions” for FR1 and FR2. Moreover, for FR2 other options are not precluded. For FR2: “TE vendors and interested companies are invited to check if the channel model used by UE demodulation performance is applicable for BS demodulation performance test”. The channel model choice for UE demodulation was TDL model for both FR1 and FR2 [8].

In this paper we discuss why non-spatial TDL models are not suitable for BS demodulation performance evaluation and propose to use geometric clustered delay line (CDL) models instead.

# 2	Why not TDL models?

-Jakes DPS is not realistic for FR2 and beamforming Tx & Rx as described in [2]

-BS antenna is highly directive and relies on polarizations of antenna elements, for this reason physical antenna modelling and geometric propagation modelling are recommended

-TDL models do not consider any antenna characteristics, neither polarization nor directivity

-CDL provides model is more realistic result than TDL (TR38.901)

-CDL is a forward compatible choice as it enables MU-MIMO modelling by introducing users with angularly rotated CDL models, while TDL model does not contain or support any directional information

-CDL is a forward compatible choice as it enables dynamic geometry modelling with a simple parameter interpolation scheme

Depending on the scope of Rel-15 BS performance requirements, some of the reasons above could not apply (as the functionality could be deferred to Rel-16). However, it is recommended to consider them at this stage in order to enable meaningful comparisons in the future.

# 3	Proposed CDL models

We propose to use CDL of TR38.901[3]. Detailed steps for using CDL model are described in [4]. Five models, CDL-A…E, their mapping to scenarios and the corresponding scaling parameters are specified in Table 1.

Table 1. Target spread (LS) parameters for CDL models from the base-line model of TR38.901 [calculated for 28GHz frequency, and applicable for FR2 frequency].

| Model | Scenario | [ns] | [deg] |  |  |  | Kf [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ASD | ASA | ZSD | ZSA |  |
| CDL-A | UMi O2I NLOS | 239.9 | 17.8 | 57.5 | 1.6 | 10.2 | - |
| CDL-B | UMi NLOS | 65.9 | 15.6 | 49.3 | 1.1 | 7.3 | - |
| CDL-C | Indoor NLOS | 26.2 | 41.7 | 50.4 | 12.0 | 14.7 | - |
| CDL-D | Indoor LOS | 19.7 | 33.3 (39.8) | 26.0 (31.9) | 1.4 | 11.5 | 7 |
| CDL-E | UMi LOS | 32.3 | 11.7 (13.7) | 28.0 (41.0) | 1.6 | 3.8 | 9 |

The target spread values of Table 1 are median values from Table 7.5-6 in TR38.901[3]. However, the ASD and ASA values marked in Table 1 in red font are modified target spreads and the actual median values are given in parenthesis. The modification is done, because the median values are not achievable with the Ricean K-factor values of 7 and 9 dB.

Table 2. Original (non-circular) angle spreads of CDL models.

| Model | [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-A | 73.7 | 85.3 | 28.6 | 21.1 |
| CDL-B | 41.6 | 59.3 | 6.0 | 10.4 |
| CDL-C | 39.1 | 71.1 | 4.1 | 10.4 |
| CDL-D | 19.0 | 21.1 | 3.0 | 1.9 |
| CDL-E | 13.2 | 37.6 | 1.5 | 2.5 |

Table 2 contains the angle spread values of the original CDL models of TR38.901[3] before any angular scaling. The values are calculated for the angles AOD, AOA, ZOD, and ZOA angles after removing the mean angle ϕn-μϕ,model following the definition of rms angular spread in TR25.996 [5], without finding the minimum over circular shifts. UE and gNB antenna parameters, like field pattern, orientation, and polarization, are taken from [6], which is based example arrays of TR38.901 and R4-1711826.

This model is using following parameters

- Base CDL model from CDL models defined in [3, Subclause 7.7.1] characterized by normalized delay, power, AOA, AOD, ZOA and ZOD for each cluster; delay spread, angular spread and mean angle for AOA, AOD, ZOA and ZOD.

- Emulated gNB related parameters: gNB antenna field pattern, gNB antenna orientation and polarization

- Emulated UE related parameters: UE antenna field pattern, UE antenna orientation and polarization, UE speed and direction

- Initial phases  for each ray m of each cluster n for NLOS paths and distance d3D (explained in [3, Figure 7.4.1-1] to model LOS path loss) for LOS path.

- Tx/Rx beam selection procedure between emulated gNB and emulated UE antenna

- Rician K factor for LOS CDL base models

- Carrier Frequency

- Fixed pairing between angles of arrival and angles of departure in Step 2 described in [3, Subclause 7.7.1].

# 4	Channel model simplification

The number of clusters, i.e. delay taps, of CDL models is reduced when beamforming antennas are embedded to both link ends and the dynamic range of 25dB is used to cut weak clusters. Corresponding parameter tables are shown with example antenna arrays in [7]. There the number of clusters within 25dB dynamic range is 8, 12, 6, 1, 3 in models CDL-A,B,C,D,E, respectively. Thus, the number of delay taps to simulate/emulate is reduced and the model is simplified substantially.

# 5	Conclusions

Two proposals are made:

Proposal 1: Choose CDL modelling approach for BS demodulation performance evaluations.

Proposal 2: Choose five CDL models (CDL-A…E) coupled to five scenarios (UMi O2I NLOS, UMi NLOS, Indoor NLOS, Indoor LOS, UMi LOS) with angular and delays scaling parameters from TR 39.901, as shown in Tables 1 and 2 above.

# 6	References

- R4-1811726 “Way forward on General part of NR BS demodulation performance”, 3GPP TSG-RAN WG4 Meeting #88, Gothenburg, Sweden, Aug 2018.

- R4-1809221, “On suitability of Jakes Doppler spectrum model in FR2 (further updates)”, 3GPP TSG-RAN WG4 Meeting #AH1807, Montreal, Canada, July 2018.

- 3GPP TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz” V14.0.0 (2017-03)

- R4-1809586, “TP to TR 38.810 on option 2 channel model generation methodology for FR2: parameters and procedure updates”, 3GPP TSG-RAN WG4 Meeting #AH1807, Montreal, Canada, July 2018.

- 3GPP TR 25.996, "3rd Generation Partnership Project; technical specification group radio access networks; Spatial channel model for MIMO simulations", V6.1.0.

- R4-1809236, “TP to TR 38.810 on option 2 channel model generation methodology for FR2: parameters and procedure updates”, 3GPP TSG-RAN WG4 Meeting #AH1807, Montreal, Canada, July 2018.

- R4-1809237, “Channel model simplification proposal for FR2”, 3GPP TSG-RAN WG4 Meeting #AH1807, Montreal, Canada, July 2018.

- R4-1811725, “Way forward on channel model for NR performance requirements” RAN4#88, Gothenburg, Sweden, Aug 2018