3GPP TSG-RAN WG4 Meeting #88bis 	R4-1813541

# Chengdu, Sichuan, China, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-2: adding TRP measurement procedures in Annex F. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-10-8 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TRP measurement procedures which agreed at the last RAN4 #88 meeting, were captured in TR 37.843 but not in TS 37.145-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TRP measurement procedures for 2 cut with pattern multiplcation, 2 or 3 cut with dense sampling, full sphere with sparse sampling, beam-based directions, peak method, equal sector with peak average and pre-scan are included in the annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not all TRP measurement procedures are included in the TS 37.145-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

Annex F (normative):  TRP measurement grids

# F.1		General

The annex describes various sampling grids for OTA TRP performance of AAS BS.

# F.2	Spherical equal angle grid

## F.2.1	General

TRPEstimate is defined as

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

where N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM.

## F.2.2	Rayleigh sampling criteria

If EUT is mounted along the yz plane as shown in Figure F2.2-1, the Rayleigh resolution angles (reference step) can be determined by

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis and  is wavelength for the measured frequency.

Figure F2.2-1. Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor )of the grid is defined as

Where  and  are the actual angular steps used in the measurement.

When three dimensions of EUT are used, the reference angular steps Δϕ and Δθ in are radians are defined as

Δϕref=min⁡(λDcyl,[π12])

Δθref=⁡(λD,[π12])

Dcyl and D are calculated as

Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in Figure F2.2-2. The radiation source can be EUT antenna array or the whole of EUT.

Figure F2.2-2: Dimensions of a radiation source: depth (d), width (w) and height (h)

# F.3	Spherical equal area grid

TRPEstimate is defined as

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

N is the total number of samples and specified in FFS. Each (θn,ϕn) is a sampling point.

# F.4 	Spherical Fibonacci grid

TRPEstimate is defined as

TRPEstimate=1Ii=1IEIRP(θi,ϕi)

I is the total number of samples. Each (θi,ϕi) is a sampling point.

# F.5 Orthogonal 2 cut grid

Orthogonal 2 cut grid comprises:

- a horizontal cut passing through the peak direction of the main beam and

- a vertical cut passing through the peak direction of the main beam.

The total EIRP for each cut is defined as

EIRPav,cut-n=1Pj=1PEIRP(j)

where P is the number of sampling points. The final contribution for all cuts is calculated as

TRPEstimate=1Nn=1NEIRPav,cut-n

where N is the number of cuts.

# F.6 Wave vector space grid

If EUT is mounted along the yz plane as shown in Figure F2.2-1, ,the Rayleigh resolutions (reference step) in wave vector space can be determined by

Δugrid=λDy

Δvgrid=λDzwhere Dy is the length of radiating  parts of EUT along  y-axis, Dz is the length of radiating parts of EUT along the z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be  represented as following:

u=sinθsinϕ, v=cos⁡(θ)

The total radiated power (TRP) in the wave vector space is determined by

TRP=ΔugridΔvgrid4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor)of the grid is defined as

Where  and  are the actual steps used in the wave vector space in the measurement.

# F.X1 Orthogonal 2 cut with pattern multiplication

# F.X2 Orthogonal 2 or 3 cut with dense sampling

# F.X3 Full sphere with sparse sampling

# F.X4 Beam-based directions

Beam-based direction can be used if directivity of the EUT antenna is known for the base station operating band. TRPEstimate is defined as

TRPEstimate=EIRPpeakDEUT, where EIRPpeak is the maximum EIRP in the beam peak direction within a particular beam direction pair and DEUT is directivity of the EUT antenna.

# F.X5 Peak method

# The peak method can be used when frequencies with unwanted peak emissions are identified during pre-scan. The method does not provide an estimate of TRP.

# For each peak emission frequency identified during pre-scan, measure peak EIRP or power density as follows:

# Move EUT and test antenna to the same position where the peak emission is recorded during the pre-scan.

# Move the EUT around the position and test antenna orientation to find the final peak EIRP or power density.

# The measured peak power density or EIRP shall be used to demonstrate conformance.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarised components.

# F.X6 Equal sector with peak average

# Equal sector with peak average can be performed on frequencies with unwanted peak emission, which are considered by the peak method for further measurements.

# The spherical angle ϕ is divided into K equal sectors. If the largest dimension of EUT is less than 60 cm, then each sector is a half quadrant of 45°.

For each peak emission frequency, measure peak EIRP of beams belonging to different sectors of the sphere as follows:

# Move EUT and test antenna to the same position where the emission peak is recorded during the pre-scan.

# Move EUT around the position and test antenna orientation to find the final peak EIRP.

# Repeat Steps 1 to 2 until all sectors are covered.

# Calculate TRPEstimate as

# TRPEstimate=1Kk=1KEIRPk, where EIRPk is the peak EIRP in the kth sector.

# NOTE:	Peak EIRP is the linear sum of two orthogonal polarised components.

# F.X7 Pre-scan

# Pre-scan is used to identify frequencies with unwanted emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP.

# The procedure for pre-scan is as follows:

# Scan the entire surface around EUT.

# Rotate test antenna to cover all possible polarisations of emissions to detect maximum emissions.

# Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur.

# Frequencies with unwanted emission power levels that are more than [20 dB] below a specified limit shall not require further measurements.

<End of modified section>