# 3GPP TSG- RAN WG4 Meeting #88bis 	R4-1813875

# Chengdu, China, 8-12 Aug 2018

Source: 	Huawei

Title: 	Ad-Hoc meeting minutes on NR BS conformance testing

Agenda Item:	7.9.2

Document for:	Approval

# Introduction

There was an AH meeting held Monday evening mainly focused on AI 7.9.2.

# Meeting minutes

## General

R4-1812582	Correction on NOTE for wanted signal mean power for NR BS RX requirements in FR1 and FR2 NTT DOCOMO, INC.

Discussion:

Huawei: some EIS may relate to declaration and thus it is a bit strange to refer to the core specification. Wording can be worked out.

Decision: Noted.

R4-1812583	TP to TS 38.141-1: Correction on NOTE for wanted signal mean power for NR BS RX requirements NTT DOCOMO, INC.

Discussion:

Decision: 	Approved

R4-1812584	TP to TS 38.141-2: Correction on NOTE for wanted signal mean power for NR BS RX requirements NTT DOCOMO, INC.

Discussion:

ZTE: MU for different FR should be considered.

Ericsson: even though EIS is declared, in the core requirement, it is clearly stated.

Decision: Return to

## Test configurations

R4-1812540	TP to TS 38.141-2_Applicability of test configurations for single-band RIB(4.7.3) ZTE Corporation

Discussion:

Huawei: for multi-band operation using single band RIB, if you do multi-band test, you can’t differentiate between the different RIBs.

Nokia: we share the same view as Huawei.

Decision: Noted

R4-1812681	On MB test for single band connector Huawei

Discussion:

ZTE: multi-band operation has several structures. The structure shown in the Figure is for LTE. We prefer to keep the table.

Ericsson: we still need to do some multi-band testing.

Huawei: we can check further.

Decision: Noted.

R4-1812682	TP to TS 38.141-1: On MB test for single band connector Huawei

Discussion:

Decision: return to

## Test cases

R4-1813564	RF channels for NR OTA tests Nokia, Nokia Shanghai Bell

Discussion:

Ericsson: we already approved the RF channels for many requirements. Would your proposal then override previous agreements?

Huawei: at last meeting, we made agreement on conducted requirement. For OTE requirements, there is no solid agreement yet, which can be optimized. We have some proposal too.

ZTE: for “OTA ACLR test with B and T channels”, do you test the adjacent channels on both sides. For receiver sensitivity, dynamic range and ICS, they don’t need to be tested for multi-carrier or multi-band cases.

Decision: noted.

R4-1812684	Discussion on RF channels for OTA testing Huawei, HiSilicon

Discussion:

Nokia: we can consider combining the proposal from Nokia.

Decision: Noted.

R4-1812604	TP for TS38.141-1: Adding a note for some specific requirements on RF channel ZTE Corporation

Discussion:

Ericsson: we have a similar proposal.

Nokia: do you plan to have something similar for OTA?

Decision: revised to

R4-1812758	pCR to TS 38.141-1 : Test Cases position update for IM and Blocking Ericsson

Discussion:

Decision: 	Noted

R4-1812605	TP for TS38.141-2: RF channel for BS OTA conformance test ZTE Corporation

Discussion:

NEC: this contribution proposed to change a wrong specification.

Huawei: the change on ON/OFF power is unclear. Also, what is the motivation for changes to “OTA transmitter spurious emissions”

Ericsson: we need to check with RF channels.

Decision: revised to

WF:

Nokia to lead WF on RF channels for OTA testing.

## Test model

R4-1812270, “Discussion on NR Test model”, CATT

R4-1812579, “Discussion on TDD configurations for NR test model”, ZTE

R4-1812581, “TDD configuration for NR BS test model for FR2”, NTT DoCoMo

R4-1812595, “Further discussion on the remaining issues for NR test model”, ZTE

R4-1812601, “TP to TS 38.141-1: FR1 test model(4.9.2)”, ZTE

R4-1812603, “TP to TS 38.141-2: FR2 test model(Section 4.9.3)”, ZTE

R4-1812743, “Power boosting/deboosting configuration for NR Test Models”, Ericsson

R4-1812744, “TP to TS 38.141-1: Section 4.9.2.3 Data content of PHY channels”, Ericsson

R4-1812745, “TP to TS 38.141-2: Section 4.9.2.3 Data content of PHY channels”, Ericsson

R4-1812746, “TP to TS 38.141-1: Section 4.9.3.2.2 NR test model 1”, Ericsson

R4-1812747, “TP to TS 38.141-2: Section 4.9.3.2.2 NR test model 2”, Ericsson

R4-1812748, “Synchronization considerations for Test Model Design”, Ericsson

R4-1813171, “TP to TS38.141-2: Test model (4.9) “, NEC

R4-1813277, “TP for FR1 base conformation test models”, Huawei, HiSilicon

R4-1813278, “TP for FR2 base conformation test models”, Huawei, HiSilicon

R4-1813279, “TDD configurations for FR2”, Huawei, HiSilicon

R4-1813280, “SS block in test models”, Huawei, HiSilicon

R4-1813281, “Boosting patterns in test models”, Huawei, HiSilicon

R4-1813282, “TDD configurations for test models used in MSR tests”,

R4-1813283, “Reference Symbol for Test Model”, Huawei, HiSilicon

R4-1813536, “TP to TS 38.141-1: Clause 4.9.3 Data content of Physical channels and Signals for NR FR1 Test models”, Nokia, Nokia Shanghai Bell

R4-1813537, “TP to TS 38.141-2: Clause 4.8.3 Data content of Physical channels and Signals for NR FR2 Test models”, Nokia, Nokia Shanghai Bell

R4-1813539, “TP to TS 38.141-1: Clause 4.9.2 Test models”, Nokia, Nokia Shanghai Bell

R4-1813540, “TP to TS 38.141-2: Clause 4.9.3 Test models”, Nokia, Nokia Shanghai Bell

## TDD pattern

### FR1

| Contribution | Source | Proposal / comments |
| --- | --- | --- |
| R4-1812579 | ZTE | States ambiguity in spec for type of signaling of TDD pattern |
| R4-1813539 | Nokia | Purpose to implement {DSUDD} Correction (shifting) of TDD pattern for test model to align in practice agreed NR TDD configuration with LTE configuration 2 and special sub-frame configuration 7, |
| R4-1810619 | Ericsson | - A combination of cell specific and UE specific signaling (TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated) can be used in order to consider radio frame misalignment to have MSR and NR coexist, if LTE config 2 with SS 7 is to be used. |

Spec Issues / comments:

- The general intent of the TDD configuration pattern to was to use SIB1 signaling (e.g., tdd-UL-DL-ConfigurationCommon) to convey the configuration. Are the TPs clear for that signaling

- Is periodicity of pattern needed

- Discussion:

- Nokia: shifting of DDDSU is not allowed in RAN1 specification. Therefore, we proposed to use two patterns.

- Docomo: not sure if shifting is needed. From actual operation’s pov, shifting is not needed.

- Huawei: DDDSU was used in the demod test. The 3ms periodicity was specified in 38.331 v15.3. Which version of 38.331 should be used in RAN4 testing consideration?

- Nokia: our understanding of using DDDSU is to align with LTE.

WF:

No need to use two patterns as proposed by Nokia

The format used in “Table 1. TDD configurations for FR1 NR test model” in R4-1812579 to capture the TDD pattern is agreed on.

It is ok to refer to the latest version of R15 38.331.

### FR2

| Contribution | Source | Proposal |
| --- | --- | --- |
| R4-1812579 | ZTE | Remove configuration for 15, 30, 240 kHz Similar comment for FR1 |
| R4-1812581 | NTT DoCoMo | TDD configuration for FR2 does not have to be aligned with for FR1, since there is a large frequency separation between FR1 and FR2 Option 2 (120 kHz) {DDDUS} S={D10,G2,U2} P=0.625 ms Option 3 (120 kHz) {DDUS} S={D12,U2} P=0. 5 ms Use option 2 for 60, 120, 240 kHz |
| R4-1813279 | Huawei | Propose {DDDSU} with S=4D, 6G, 4U 60 kHz (reference model A.3.3 in 38.101-2) Derive 120 kHz pattern from 60 kHz pattern |
| R4-1813540 | Nokia | Using DDDSU S={D10,G2,U2} for both 60 and 120 kHz |

Spec Issues / comments:

- (same as FR1) The general intent of the TDD configuration pattern to was to use SIB1 signaling (e.g., tdd-UL-DL-ConfigurationCommon) to convey the configuration. Are the TPs clear for that signaling

- (same as FR1) Is periodicity of pattern needed

- Remove configurations for 15, 30 kHz SCS (ZTE, NTT DoCoMo, NEC, Nokia)

- Remove configuration for 240 kHz SCS (ZTE, Nokia)

Technical Issue:

- If TDD pattern for FR2 is revised, how should the pattern be?

  - NTT DoCoMo: design based on {DDDUS} S={D10,G2,U2} P=0.625 ms (120 kHz)

  - Nokia: {DDDSU} S={D10,G2,U2} for both 60 and 120 kHz

  - Huawei: {DDDSU} with S=4D, 6G, 4U 60 kHz (reference model A.3.3 in 38.101-2) and derive rest 120 kHz from that

Discussion:

Docomo: our original proposal is the same as Nokia {DDDSU} S={D10,G2,U2} for both 60 and 120 kHz.

Ericsson: will there be any coexistence issue?

Huawei: we agree with Ericsson. We use the 60kHz pattern to derive the one for 120kHz.

Docomo: do we need to consider the mixed numerology case?

Huawei: we are talking about two carriers one using 60kHz and the other using 120kHz.

ZTE: we need to consider this for intra-band CA.

WF:

The format used in “Table 3. TDD configurations for FR2 NR test model” in R4-1812579 to capture the TDD pattern is agreed on.

It is agreed to remove configurations for 15, 30 kHz and 240kHz SCS

### MSR

| Contribution | Source | Proposal |
| --- | --- | --- |
| R4-1812579 | ZTE | Proposes TDD configuration values using combination of broadcast {DSUUD} and dedicated signaling |
| R4-1813282 | Huawei | Use a TDD configuration with two common configurations with a 4 ms period and 1 ms period when NR is used in MSR testing |

Spec Issues / comments:

- Should design of TDD configuration be considered for MSR now if 37.141 is not under discussion at this moment

- Technical questions

- Which version of 38.331 spec should be used to generate TDD configuration?

WF:

It is agreed to use combination of broadcast {DSUUD} and dedicated signalling as proposed by ZTE to align with MSR pattern.

## PDSCH Power boosting

| Contribution | Source | Proposal |
| --- | --- | --- |
| R4-1812270 | CATT | use the same number of boosted and de-boosted RBs for NR test model since the RE power dynamic range requirement are the same for LTE and NR. NR test model; [dB]; modulation scheme of boosted PDSCH; modulation scheme of de-boosted PDSCH. NR-TM1.2; 0.4; 3; QPSK; QPSK. NR-TM3.2; 0.6; -3; QPSK; 16QAM. NR-TM3.3; 0.5; -6; 16QAM; QPSK. |
| R4-1812595 | ZTE | use the same power boosting/de-boosting levels for FR1 NR test model as that for E-UTRA test model +3dB power boosting for UEM/ACLR measurement in NR-TM1.2; -3dB power de-boosting for 64QAM EVM measurement PRBs in NR-TM3.3; -6dB power de-boosting for QPSK EVM measurement PRBs in NR-TM3.3. every 2nd PRB are assumed to be power boosted for NR-TM1.2, NR-TM3.2 and NR-TM3.3 |
| R4-1812743 | Ericsson | Introduce power boosting of +3 dB for every 3rd PRB starting from the second PRB in an OFDM symbol and adopt the deboosting level according to Table 3. Typo in formula for deboosting |
| R4-1813281 | Huawei | TM2: P(s)=(s×⌈N_RB/(10×2^μ)⌉+f"mod" 2)"mod" N_RB,s=0,⋯,10×2^μ-1 TM1.2, P(k,s)=(s "mod " N+⌊k/M⌋N+k "mod " M)"mod" N_RB |
| R4-1813536 | Nokia | Provided formula for TM2 based on spacing (NRB/10*2µ) |

Boosting values

|  | TM1.2 | TM3.2 | TM3.3 |
| --- | --- | --- | --- |
| CATT | 0.4; 3dB | 0.6; -3 dB | 0.5, -6 dB |
| ZTE | 0.5; 3dB | 0.5; -3dB | 0.5; -6dB |
| Ericsson | 0.33, 3dB | None | None |
| Huawei | 0.4; 3dB | 0.6; -3 dB | 0.5, -6 dB |
| Nokia | ~50%, 3 dB | ~50%, -3 dB | ~50%, -6dB |

TM1.2

|  | Formula |
| --- | --- |
| Huawei | Every M RB each N RBs are boosted |
| Nokia | Every other |
| ZTE | Every other |
| Ericsson | Every 3rd |

TM2

|  | Formula |
| --- | --- |
| Huawei | Ps=s×NRB10×2μ+fmod2mod NRB, s=0,⋯,10×2μ-1 |
| Nokia | AllocSpacing = NRB10*2μ, RbNumber(i) = (AllocSpacing * i) mod (NRB-1); |

WF:

Huawei to lead a WF on power boosting.

## PDCCH

2 main topics

Number of symbols

Span of PDCCH

| Contribution | Source | Proposal |
| --- | --- | --- |
| R4-1812270 | CATT | Two symbol PDCCH 1 CCE |
| R4-1812595 | ZTE | Placeholder for simulations results for PDSCH based on PDCCH duration |
| R4-1813536, R4-1813540 | Nokia | 1 symbol CORESET FR1 - [1,2] - 1 CCE, 6 RBs FR2 - 2 symbols - Span entire NRB |
| R4-1813277, R4-1813278, | Huawei | Starting RB of PDCCH should be specified |
| R4-1810616 | Ericsson | # of symbols used for control channel =[1,2] # of CCEs allocated to PDCCH = 1 # of available REGs= 6 Aggregation level(s) =1 # of REGs not allocated by PDCCH in the first two symbols =2* NRB – 6 DM-RS configuration and density Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs PDSCH DM-RS configuration and density Type 1, Comb2 (every other subcarrier) in symbol 3 PT-RS pattern and density Distributed pattern, every symbol for every second PRB (time density 1 and frequency density ½) - # of QPSK PDSCH PRBs which are boosted or deboosted 0 |

Number of symbols

| Company | FR1 | FR2 |
| --- | --- | --- |
| CATT | 2, 6 RB |  |
| ZTE | 2? [1,2] 6 RB | [1] 6 RB |
| Nokia | 2 (entire BW) | 2 (entire BW) |
| Ericsson | [1,2] 6 RB | [1,2] 6 RB |
| Huawei | [1,2] 6 RB | [1,2] 6 RB |

WF:

It is agreed to use 1CCE, over 2 symbols for PDCCH. The unoccupied RBs will be used for PDSCH. CCE wills start from RB 0.

## SS Block

| Contribution | Source | Proposal |
| --- | --- | --- |
| R4-1812748 | Ericsson | 3 options for synchronization: using Synchronisation Signal (SS) and PBCH (SS Block); External Clock; PDSCH DMRS Some parameters for SS: SS burst length: [<=3 GHz, L is 4, 3 - 6 GHz, L is 8, 6 - 52.6 GHz L is 64] - SS burst set periodicity: [20 ms] - SS burst set mapping - # of SS burst sets - SS Beam sweeping aspects |
| R4-1813280 | Huawei | The first 2µ slots in the subframe should be reserved for SS blocks (i.e. measurements in a subframe begin on slot 2µ) for FR1. The maximum number of SS blocks for 1 ms should be used for FR1: - - for 15 kHz SCS, SS blocks beginning at symbols 2 and 8 (slot 0) - - for 30 kHz SCS and pattern B, SS blocks beginning at symbols 4 and 8 of slot 0 and 2 and 6 of slot 1; - for 30 kHz SCS and pattern C, SS blocks beginning at symbols 2 and 8 of slot 0 and 2 and 8 of slot 1. For bands 5 and 66, 15 kHz SCS should be used for the initial access SS block. For bands supporting two numerologies, the smaller numerology is used for the initial access SS block. for the slot(s) designated for the SS blocks, leave the RBs outside the SS block region unoccupied. Where to place the SS block is must be determined. Among some options: - - close to the center of the channel - - distribute across the channel - - top / middle / bottom For FR2, the initial access numerology is 120 kHz SCS. For FR2, the number of slots bearing initial access should be at least 2 (0.25 ms). |
| R4-1813536 R4-1813539 R4-1813540 | Nokia | Center – 10 RBs 2 positions in slot - FR1: - (10/20 ms, 1 burst - FR2 - 20 ms periodicity - 1 burst - Start symbol: 8 |

Technical issues:

The need and if the need is identified, then the location of SSB (Huawei near center with regards to GSCN, Nokia near center)

SCS: Huawei – band specific, use lower SCS when multiple SCS possible; Nokia (verify)

Number per measurement period

Discussion:

The group understands there will be some complexity associated with the use of SSB in the test.

Keysight commented that in the tests, measurement results would be the same with or without the transmission of SSB.

WF:

It is agreed to seek views from other TE vendors at this meeting. If their feedback is the same as Keysight’s, no SSB would be used in the test.

## FR1 TP other aspects

| Contribution | Source | Proposal |
| --- | --- | --- |
| R4-1812601 | ZTE |  |
| R4-1812746 | Ericsson |  |
| R4-1813277 | Huawei |  |
| R4-1813536 | Nokia |  |
| R4-1812744 | Ericsson |  |

## FR2 TP other aspects

| Contribution | Source | Proposal |
| --- | --- | --- |
| R4-1812603 | ZTE |  |
| R4-1813171 | NEC |  |
| R4-1813278 | Huawei |  |
| R4-1813540 | Nokia |  |
| R4-1813537 | Nokia |  |
| R4-1812745 | Ericsson |  |

WF:

Nokia to prepare TP for data content for FR1

Ericson to prepare TP for data content for FR2

Huawei to prepare TP for test model for FR1

ZTE to prepare TP for test model for FR2