3GPP TSG-RAN WG4 Meeting #88bis	R4-1813881

Chengdu, China, 8 – 12 October 2018

Source:	Ericsson

Title:	TP to TS 38.141-2: Section 4.9.2.3 Data content of PHY channels

Agenda item:	7.9.4.2.2

Document for:	Approval

# 1	Introduction

In the last RAN4 meeting, there was initial drafting of TS 38.141 regarding specifics for NR test models.  This contribution builds upon initial text for NR TM 1.  The data content within the defined test model also needs to be further discussed and defined to have complete specified TM for NR.

This contribution aims to have a starting point for the data content of the physical channels based upon previous meeting agreements on the PDCCH and PDSCH structure.

# 2	References

- R4-1811901, “TS 38.141-1 v0.4.0”, Huawei

[Start of Text Proposal]

#### 4.9.3.2 	NR test models

The set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	The test models are defined for a single antenna port, 1 layer, precoding is not used; unless specified otherwise

[Unchanged Sections]

### 4.9.3.3	Data content of Physical channels and Signals for NR-TM

Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS38.211, Clause 5.2 [xx] which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of ‘0’ bits shall be generated prior to the scrambling.

Initialization of the scrambler and RE-mappers as defined in TS38.211 [xx] use the following additional parameters:

-	[ = 1 for the lowest configured carrier,  = 2 for the 2nd lowest configured carrier,…,  = n for the nth configured carrier]

-  Antenna ports starting with 1000 for PDSCH

- 	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	Rank 1 (single layer)

-   = 0

#### 4.9.3.3.1	PDCCH

-	PDCCH modulation to be QPSK as described in TS38.211, clause 5.1.3

-	For each subframe the required amount of bits for all PDCCHs is as follows: (# of PDCCH)*(# of CCE per PDCCH)* 6 (REG per CCE)*12 (RE per REG)*2( bits per RE) with these parameters according to the NR -TM definitions in subclause 4.4.9.2.2

-	Generate this amount of bits according to ‘all 0’ data

- 	NsymbCORESET= 1

-	1 CCE shall be according to TS38.211, clause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping.  PDCCH should occupy first symbol for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	[Perform PDCCH multiplexing and scrambling according to TS38.211, clause 7.3.2.3]

-	Perform mapping to REs according to TS38.211, clause 7.3.2.5

- 	NID = 0 in DM-RS sequence generation

#### 4.9.3.3.2		PDSCH

-	For each slot generate the required amount of bits for all PRBs according to ‘all 0’ data

-	NR-TMs utilize 1 user PDSCH transmissions

-

The required amount of PDSCH ‘0’ bits within a slot and allocated PRBs shall be generated

-	[Perform user specific scrambling according to TS38.211, clause 7.3.1.1.]

-	[Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS38.211, clause 6.3.2]

-	Perform mapping of the complex-valued symbols to layer according to TS38.211, clause 7.3.1.3.   Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is equal to 1.

-	Perform PDSCH mapping type A according to TS38.211

-	DM-RS sequence generation according to TS38.211, clause 7.4.1.1.1 where l is the OFDM symbol number within the slot, using symbol 3 for every other subcarrier

-	DM-RS mapping according to TS38.211, clause 7.4.1.1.2 where type 1 configuration is considered

-	For NR-FR2-TM PT-RS sequence generation according to TS38.211, clause 7.4.1.2.1

-	For NR-FR2-TM PT-RS mapping according to TS38.211, clause 7.4.1.2.2, where l = 4, k = 2

[End of Text Proposal]