3GPP TSG-RAN WG4 Meeting #88-Bis                 	R4-1813983

Chengdu, China, Oct 8th – 12th 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on EN-DC NR UE Transmit Timing Accuracy Tests in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-09-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Adding test for NR UE Transmit timing for FR2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding tests for NR UE Transmit timing for FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 transmit timing tests will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section A.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR…CR... TS38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------START of CHANGES-------

A.5.4.1.1 NR UE Transmit Timing Test for FR2

A.5.4.1.1.1 Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected

gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment,

minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in

clause 7.1.2.

Supported test configurations are shown in Table 5.4.1.1.1-1

Table A.5.4.1.1.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 240 KHz, data SCS 120kHz, BW 100MHz |

For this test a single NR cell is used. Table A.5.4.1.1.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.5.4.1.1.1-3.

Table A.5.4.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  |  | Test2 |  | Band Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1 | Freq1 |  |  | Freq1 |  |  |
| Duplex Mode |  | 1 | TDD |  |  |  |  |  |
| TDD configuration |  | 1 |  | TDDConf.1.2 |  |  |  |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |  |  |  |  |
| BWP BW | MHz | 1 | 100: NRB,c = 66 |  |  |  |  |  |
| DRx Cycle | ms | 1 | N/A |  |  |  | 320Note5 |  |
| PDSCH Reference measurement channel |  | 1 | SR.3.1 TDD |  |  |  |  |  |
| CORESET Reference Channel |  | 1 | CR.3.1 TDD |  |  |  |  |  |
| OCNG Patterns |  | 1 | OCNG pattern 1 |  |  |  |  |  |
| SMTC configuration |  | 1 | FR2 pattern 2 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 | 120 |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | TBD |  |  | TBD |  |  |
| Note2 | dBm/SCS | 1 | TBD |  |  | TBD |  |  |
|  |  | 1 | 3 |  |  | 3 |  |  |
|  |  | 1 | 3 |  |  | 3 |  |  |
| SS-RSRPNote3 | dBm/SCS | 1 | TBD |  |  | TBD |  |  |
| IoNote3 | dBm/95MHz | 1 | TBD |  |  | TBD |  |  |
| Propagation condition |  | 1 | AWGN |  |  |  |  |  |
| SRS Config |  | 1 | Config1Note6 |  | Config2Note6 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table A.5.4.1.1.1-4 Note6: SRS configs are given in Table A.5.4.1.1.1-3 |  |  |  |  |  |  |  |  |

Table A.5.4.1.1.1-3 SRS Configuration for Timing Accuracy Test

|  | Field | Config1 | Config 2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
|  | SRS-ResourceSetId | 0 | 0 |  |
| SRS-Resource | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | sl1 | sl1 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1 | sl640 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

Table A.5.4.1.1.1-4: DRX-Configuration for UL Timing Tests.

| Field | Test 2 |
| --- | --- |
|  | Value |
| drx-onDurationTimer | TBD |
| drx-InactivityTimer | TBD |
| drx-RetransmissionTimerDL | TBD |
| drx-RetransmissionTimerUL | TBD |
| longDRX-CycleStartOffset | TBD |
| shortDRX | TBD |

A.5.4.1.1.2 Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

- Set up E-UTRA PCell according to parameters given in Table A.3.7.2.1-1 and setup NR PSCell according to parameters given in Table A.5.4.1.1.1-1.

- After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ± Te of the first detected path of DL SSB.

  - The NTA offset value (in Tc units) is 25600 for FR1 and 13792 for FR2

  - The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

- The test system shall adjust the timing of the DL path by values given in Table A.5.4.1.1.2-1

Table A.5.4.1.1.2-1 Adjustment Value for DL Timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |
| 120 | +16*64Tc | +8*64Tc |
| 240 | +8*64Tc | +4*64Tc |

- The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Section 7.1.2 Table 7.1.2-3. This will only be done for Test1.

- The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ± Te of the first detected path of DL SSB. For Test 2 and Test 4 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

--------------------END of CHANGES------------