3GPP TSG-RAN WG4 Meeting #89	R4-1815239

Spokane, WA, USA, 12 – 16 November 2018

Agenda Item:	6.6.4.1

Source:	Ericsson

Title:	Simulation results of PDSCH demodulation for 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#88bis agreed with the simulation assumption for PDSCH demodulation with 1024QAM as follow [1].

| No. | TM | PMI feedback (pre-coding) | MIMO layer | MCS | Antenna config. |
| --- | --- | --- | --- | --- | --- |
| 1 | TM4 | Wideband PMI Feedback (PUSCH 3-1, Reporting interval =1) | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 2 | TM4 | Wideband PMI Feedback (PUSCH 3-1, Reporting interval =1) | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |
| 3 | TM9 | Per-PRG Random Precoder | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 4 | TM9 | Wideband PMI Feedback | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |

In this contribution we provide the TDD simulation results in addition to our FDD results [2].

# 2	Simulation results

## 2.1	TM4

Table 1 summarizes the required SNR [dB] to achieve 70% of the maximum throughput for PDSCH TM4 (corresponding to tests 1 and 2).

Table 1	Required SNR [dB] to achieve 70% of maximum throughput for TM4.

|  | SNR of 70% of maximum throughput |  |
| --- | --- | --- |
|  | FDD | TDD |
| Test 1 | 22.4 dB | 23.7 dB |
| Test 2 | 25.2 dB | 26.3 dB |

## 2.2	TM9

Table 2 summarizes the required SNR [dB] to achieve 70% of the maximum throughput for PDSCH TM9 (corresponding to tests 3 and 4).

Table 2	Required SNR [dB] to achieve 70% of maximum throughput for TM9.

|  | SNR of 70% of maximum throughput |  |
| --- | --- | --- |
|  | FDD | TDD |
| 4x2 rank 1 follow PMI | 23.3 dB |  |
| 4x2 rank 1 random PMI (Test 3) | 27.4 dB | 26.0 dB |
| 4x4 rank 2 follow PMI (Test 4) | 26.6 dB | 27.0 dB |
| 4x4 rank 2 random PMI | 28.9 dB |  |

# References

- R4-1813705, “Simulation assumption for 1024QAM demodulation requirement”, Qualcomm.

- R4-1813686, “Simulation results of PDSCH demodulation for 1024QAM”, Ericsson.