3GPP TSG-WG RAN4 Meeting #90	R4-1900072

# Athens, Greece, 25st Feb – 1st Mar, 2019

Source: 	Nokia, Verizon

Title: 						TP to TR 36.716-04-01: DL_2A-13A-46X-66A and DL_2A-13A-46X-66A-66A_BCS0

Agenda Item: 			7.4.2

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA (4 bands DL with 1 band UL) for TR 36.716-04-01

- DL_2A-13A-46A-66A_BCS0

- DL_2A-13A-46C-66A_BCS0

- DL_2A-13A-46D-66A_BCS0

- DL_2A-13A-46A-66A-66A_BCS0

- DL_2A-13A-46C-66A-66A_BCS0

- DL_2A-13A-46D-66A-66A_BCS0

**************************** Start of changes ********************************************

## 5.X	CA_2-13-46-66

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-13A-46A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  | 13 |  |  | Yes | Yes |  |  |  |  |
|  | 46 |  |  |  |  |  | Yes |  |  |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-13A-46C-66A | 2 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  | 13 |  |  | Yes | Yes |  |  |  |  |
|  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-13A-46D-66A | 2 |  |  | Yes | Yes | Yes | Yes | 110 | 0 |
|  | 13 |  |  | Yes | Yes |  |  |  |  |
|  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-5A-46E-66A | 2 |  |  | Yes | Yes | Yes | Yes | 130 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-13A-46A-66A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  | 13 |  |  | Yes | Yes |  |  |  |  |
|  | 46 |  |  |  |  |  | Yes |  |  |
|  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-13A-46C-66A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 110 | 0 |
|  | 13 |  |  | Yes | Yes |  |  |  |  |
|  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |
|  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-13A-46D-66A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 130 | 0 |
|  | 13 |  |  | Yes | Yes |  |  |  |  |
|  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |
|  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

### 5.X.2	∆TIB and ∆RIB values

The ∆TIB and ∆RIB values of CA_2-13-66 are specified in TS36.101 and are applied to CA_2-13-46-66 and CA_2-13-46-66-66.

Table 5.1.2-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-13-46-66, CA_2-13-46-66-66 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |

Table 5.1.2-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-13-46-66, CA_2-13-46-66-66 | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |

### 5.X.3	REFSENS requirements

REFSENS requirements of CA_2A-46A-66A and CA_2A-13A-46A are defined in TS36.101 and can be applied for CA_2-5-46-66 and CA_2-5-46-66-66.

Table 5.X.3-1: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-13A-46A-66A CA_2A-13A-46C-66A CA_2A-13A-46D-66A CA_2A-13A-46A-66A-66A CA_2A-13A-46C-66A-66A CA_2A-13A-46D-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |

**************************** End of changes ********************************************