3GPP TSG-RAN WG4 Meeting #90	 	R4-1900117

Athens, GR, 25th Feb – 1st March, 2019

Agenda item:       6.14.1.3.1

Source:                Intel Corporation

Title:	                     NR PDCCH Simulation Results

Document for:     Discussion

# Introduction

In RAN4#89 PDCCH simulation results from companies were discussed to define PDCCH demodulation requirements. The simulation assumptions are captured in [1].

In this contribution we provide alignment and impairment results for PDCCH test cases in order to define requirements

# Simulation Results

## Alignment results with 2Rx

In this section we provide results for alignment as SNR at 1% Pm-dsg with 2Rx.

Table 1: PDCCH Alignment Results with 2Rx

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | CORESET-interleaver-size | Prop condition | Ant config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x2 Low | 6.04 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 2x2 Low | -0.72 |
| 3 | 15 | 1_1 | 24 | 52 | 2 | 2 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 1x2 Low | 5.92 |
| 4 | 15 | 1_1 | 48 | 52 | 2 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x2 Low | 3.66 |
| 5 | 15 | 1_1 | 48 | 52 | 2 | 8 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 2x2 Low | -3.99 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | 3 | TDLA30; 10Hz | 1x2 Low | 5.28 |
| 7 | 30 | 1_1 | 102 | 53 | 1 | 4 | Interleaved | 2 | 3 | TDLC300; 100Hz | 1x2 Low | 1.62 |
| 8 | 30 | 1_1 | 90 | 53 | 1 | 8 | Interleaved | 6 | 3 | TDLC300; 100Hz | 2x2 Low | -3.74 |
| 9 | 120 | 1_0 | 60 | 40 | 1 | 2 | Interleaved | 2 | 3 | TDL-A, 30ns, 75Hz | 1x2 Low | 4.26 |
| 10 | 120 | 1_1 | 60 | 56 | 1 | 4 | Interleaved | 6 | 2 | TDL-A, 30ns, 300Hz | 1x2 Low | 0.15 |
| 11 | 120 | 1_1 | 60 | 56 | 1 | 8 | Interleaved | 2 | 3 | TDL-A, 30ns, 75Hz | 2x2 Low | -1.90 |
| 12 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x2 Low | 2.39 |
| 13 | 15 | 1_1 | 48 | 52 | 1 | 8 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 2x2 Low | -2.32 |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x2 Low | -3.91 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 1x2 Low | -6.63 |
| 16 | 120 | 1_0 | 60 | 40 | 2 | 16 | Interleaved | 2 | 3 | TDL-A, 30ns, 75Hz | 2x2 Low | -5.01 |

## Alignment results with 4Rx

In this section we provide results for alignment as SNR at 1% Pm-dsg with 4Rx.

Table 2: PDCCH Alignment Results with 4Rx

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | CORESET-interleaver-size | Prop condition | Ant config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Low | 1.02 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 2x4 Low | -4.02 |
| 3 | 15 | 1_1 | 24 | 51 | 2 | 2 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 1x4 Low | 1.25 |
| 4 | 15 | 1_1 | 48 | 51 | 2 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Low | -1.01 |
| 5 | 15 | 1_1 | 48 | 51 | 2 | 8 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 2x4 Low | -6.50 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | 3 | TDLA30; 10Hz | 1x4 Low | 0.46 |
| 6a | 30 | 1_0 | 90 | 41 | 1 | 2 | Interleaved | 6 | 3 | TDLA30; 10Hz | 1x4 Low | 0.35 |
| 7 | 30 | 1_1 | 102 | 53 | 1 | 4 | Interleaved | 2 | 3 | TDLC300; 100Hz | 1x4 Low | -2.39 |
| 7a | 30 | 1_1 | 90 | 53 | 1 | 4 | Interleaved | 6 | 3 | TDLC300; 100Hz | 1x4 Low | -2.79 |
| 8 | 30 | 1_1 | 90 | 53 | 1 | 8 | Interleaved | 6 | 3 | TDLC300; 100Hz | 2x4 Low | -5.72 |
| 12 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Low | -1.54 |
| 13 | 15 | 1_1 | 48 | 51 | 1 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 2x4 Low | -2.22 |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Low | -7.26 |
| 14a | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Med-A | -4.50 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 1x4 Low | -8.85 |
| 15a | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 1x4 Med-A | -7.10 |
| 15b | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Med-A | -4.77 |

## Impairment Results

For the agreed PDCCH test cases we provide the results with impairment margin added in the table below.

Table 3: Impairment results for PDCCH test cases

| Test No. | Ant config | SNR @ 1% Pm-dsg | Ant config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- |
| 1 | 1x2 Low | 7.54 | 1x4 Low | 2.52 |
| 2 | 2x2 Low | 0.78 | 2x4 Low | -2.52 |
| 3 | 1x2 Low | 7.42 | 1x4 Low | 2.75 |
| 4 | 1x2 Low | 5.16 | 1x4 Low | 0.49 |
| 5 | 2x2 Low | -2.49 | 2x4 Low | -5.00 |
| 6 | 1x2 Low | 6.78 | 1x4 Low | 1.96 |
| 6a | - | - | 1x4 Low | 1.85 |
| 7 | 1x2 Low | 3.12 | 1x4 Low | -0.89 |
| 7a | - | - | 1x4 Low | -1.29 |
| 8 | 2x2 Low | -2.24 | 2x4 Low | -4.22 |
| 9 | 1x2 Low | 5.76 | - | - |
| 10 | 1x2 Low | 1.65 | - | - |
| 11 | 2x2 Low | -0.40 | - | - |
| 12 | 1x2 Low | 3.89 | 1x4 Low | -0.04 |
| 13 | 2x2 Low | -0.82 | 2x4 Low | -0.72 |
| 14 | 1x2 Low | -2.41 | 1x4 Low | -5.76 |
| 14a | - | - | 1x4 Med-A | -3.00 |
| 15 | 1x2 Low | -5.13 | 1x4 Low | -7.35 |
| 15a | - | - | 1x4 Med-A | -5.60 |
| 15b | - | - | 1x4 Med-A | -3.27 |
| 16 | 2x2 Low | -3.51 | - | - |

# Conclusion

In this paper we present alignment and impairment results for PDCCH to define NR PDCCH demodulation requirements for Rel-15.

# Reference

- R4-1816390, “Way Forward on NR PDCCH demodulation requirements”, Ericsson