3GPP TSG-RAN WG4 Meeting #90 	R4-1900144

Athens, Greece, Feb 25 – Mar 1, 2019

Agenda item:	6.12.1

Source:	Intel Corporation

Title:	Consideration on UE behaviour on reception of channels or RS in the same OFDM symbol

Document for:	Discussion

# Introduction

In RAN1 #94 an LS [1] was approved to RAN4 to ask for clarification on UE behavior on reception of channels or RS in the same OFDM symbol. Since RAN4 was focusing on completion of remaining issues in release 15, the LS was never treated in 2018. However, we believe some discussion on this topic is necessary, not only to help RAN1 on their specification, but also to complete TS38.133. We found that UE behavior is not clear in some scenario according to current TS38.133. The consequence is that without further clarification, UE may not be able to meet some of the RRM requirements due to downlink reception colliding.

In this contribution, we summarize the status of current TS38.133 regarding UE behavior on colliding downlink reception according to the question in the LS. After summary some discussion is provided on the open items.

# Discussion

There are two questions in RAN1 LS:

Question #1: RAN1 requests RAN4 to clarify UE behaviour, if necessary, when a channel or RS in group 1 overlaps with a channel or RS in group 2 on the same OFDM symbol in the same serving cell or in different serving cells in the case of CA in FR2. Case; Channels/RS in group 1; Channels/RS in group 2. 1; SSB for L3 measurements, SSB for RLM, SSB for BFD, SSB for L1-RSRP measurements; CSI-RS for RLM, CSI-RS for L3 measurements, CSIRS for BFD, CSI-RS for L1-RSRP. 2; CSIRS for L3 measurements, CSIRS with repetition=OFF for L1-RSRP measurements; PDSCH. 3; CSIRS for L3 measurements, CSIRS with repetition=OFF for L1-RSRP measurements; PDCCH. 4; CSI-RS for RLM, CSI-RS for L3 measurements, CSIRS for BFD, CSI-RS for L1-RSRP measurements; CSI-RS for RLM, CSI-RS for L3 measurements, CSIRS for BFD, CSI-RS for L1-RSRP measurements.

Case by case, below is the summary of the status of current TS38.133 according to our understanding.

- Case 1

Table 1 – case 1

| Group 1 | Group 2 | UE behaviour according to current spec |  |  | comment |
| --- | --- | --- | --- | --- | --- |
| SSB for L3 | CSI-RS for RLM | FR1 | same SCS | scheduling availability in section 9.2.5.3.1 | both can be done |
|  |  |  | diff SCS | scheduling availability in section 9.2.5.3.2 | UE supporting diff SCS can do both. UE not supporting doesn't diff SCS will do SSB and drop CSI-RS |
|  |  | FR2 | same/diff SCS | scheduling availability in section 9.2.5.3.3 | UE will do SSB and drop CSI-RS |
|  | CSI-RS for L3 | no req for CSI-RS L3 in R15 |  |  | FFS in future release |
|  | CSI-RS for BFD | same as CSI-RS RLM |  |  | same as CSI-RS RLM |
|  | CSI-RS for L1 | same as CSI-RS RLM |  |  | same as CSI-RS RLM |
| SSB for RLM | CSI-RS for RLM | FR1 | same SCS | scheduling availability in section 8.1.7.1 | both can be done |
|  |  |  | diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  |  | FR2 | same/diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  | CSI-RS for L3 | no req for CSI-RS L3 in R15 |  |  | FFS in future release |
|  | CSI-RS for BFD | FR1 | same SCS | scheduling availability in section 8.1.7.1 and 8.5.7.1 | both can be done |
|  |  |  | diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  |  | FR2 | same/diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  | CSI-RS for L1 | FR1 | same SCS | scheduling availability in section 8.1.7.1 and 9.5.6.1 | both can be done |
|  |  |  | diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  |  | FR2 | same/diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
| SSB for BFD | CSI-RS for RLM | FR1 | same SCS | scheduling availability in section 8.1.7.1 and 8.5.7.1 | both can be done |
|  |  |  | diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  |  | FR2 | same/diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  | CSI-RS for L3 | no req for CSI-RS L3 in R15 |  |  | FFS in future release |
|  | CSI-RS for BFD | FR1 | same SCS | scheduling availability in section 8.5.7.1 | both can be done |
|  |  |  | diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  |  | FR2 | same/diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  | CSI-RS for L1 | FR1 | same SCS | scheduling availability in section 8.5.7.1 and 9.5.6.1 | both can be done |
|  |  |  | diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  |  | FR2 | same/diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
| SSB for L1 | CSI-RS for RLM | FR1 | same SCS | scheduling availability in section 9.5.6.1 and 8.1.7.1 | both can be done |
|  |  |  | diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  |  | FR2 | same/diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  | CSI-RS for L3 | no req for CSI-RS L3 in R15 |  |  | FFS in future release |
|  | CSI-RS for BFD | FR1 | same SCS | scheduling availability in section 9.5.6.1 and 8.5.7.1 | both can be done |
|  |  |  | diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  |  | FR2 | same/diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
|  | CSI-RS for L1 | FR1 | same SCS | scheduling availability in section 9.5.5.1 | both can be done |
|  |  |  | diff SCS | scheduling availability in section 9.5.5.2 | UE is not expected to perform simultaneous SSB and CSI-RS measurements. |
|  |  | FR2 | same/diff SCS | no specific clarification | need clarification in 38.133 Note 1 |
| Note 1: UE needs to meet RRM requirements for both procedure. Note 2: pending on RAN1 decision if SSB based BFD will be removed or not |  |  |  |  |  |

- Case 2&3

Table 2 – case 2&3

| Group 1 | Group 2 | UE behaviour according to current spec |
| --- | --- | --- |
| CSI-RS for L3 | PDSCH/PDCCH | no req for CSI-RS L3 in R15 |
| CSI-RS (repetition=OFF for L1) | PDSCH/PDCCH | Scheduling restriction in section 9.5.6.3 for FR2. No restriction in FR1. |

- Case 4

Table 3 – case 4

| Group 1 | Group 2 | UE behaviour according to current spec | comment |
| --- | --- | --- | --- |
| CSI-RS for RLM | CSI-RS for L3 | no req for CSI-RS L3 in R15 | FFS in future release |
|  | CSI-RS for BFD | no specific clarification Note 1 | need clarification in 38.133 |
|  | CSI-RS for L1 | no specific clarification Note 1 | need clarification in 38.133 |
| CSI-RS for L3 | CSI-RS for BFD | no req for CSI-RS L3 in R15 | FFS in future release |
|  | CSI-RS for L1 | no req for CSI-RS L3 in R15 | FFS in future release |
| CSI-RS for BFD | CSI-RS for L1 | no specific clarification Note 1 | need clarification in 38.133 |
| Note 1: UE needs to meet RRM requirements for both procedure, which may be unrealistic unless they are QCLed |  |  |  |

In tables above, the context highlighted in green means the UE behavior is specified in TS38.133. But for the content highlighted in yellow, so far there is no specific clarification in the specification. The consequence is that UE needs to meet RRM requirements for both procedure by default, which may be unrealistic for some scenario. Take colliding SSB based RLM and CSI-RS based RLM for example, considering different SCS between SSB and CSI-RS, UE not supporting simultaneousRxDataSSB-DiffNumerology cannot perform SSB RLM and CSI-RS RLM simultaneously, such that the UE cannot meet current RLM requirement.

Observation 1: without further clarification, UE needs to meet RRM requirements for both procedure in case of colliding downlink reception of group 1 and group 2, which is not realistic in some scenario.

Proposal 1: some clarification is needed in Release 15 to address the colliding downlink reception issue.

One thing needs to be highlighted here is that actually not only the scenarios listed in the RAN1 LS need to be addressed. There are also some other scenarios that need further clarification. For instance, the colliding SSB based L3 and SSB based RLM in FR2 has been addressed in TS38.133. However, there is no consideration on the collision of SSB based L3 and SSB based BFD. It is quite straightforward that the SSB resources shall be shared between L3 and BFD, similar with the case of colliding L3 and RLM (there would be no problem if RAN1 decides to remove SSB based BFD). Another example is colliding of CSI-RS based RLM and CSI-RS BFD in FR2, if they are not QCLed then UE would not be able to receive them simultaneously.

Observation 2: besides the colliding scenarios of group 1 and 2 mentioned in RAN1 LS, there are also other colliding scenarios need further clarification.

Proposal 2: RAN4 shall extensively study all the colliding scenarios and capture corresponding UE behaviours in TS38.133 if necessary.

To address the open items in the tables above, RAN4 shall discuss how to handle the collision. Since the tables are too detailed and there are quite a lot of scenarios, people are encouraged to try to reach some high level principle first. Here we tentatively list some high level principle when discussing UE behaviour on colliding downlink reception.

- RAN4 shall discuss the priority for different functionality

In current TS38.133 there is an example to treat the colliding RS for different functionality with priority, i.e. collision of SSB for RLM and SSB for L3 in FR2. People believe the L3 mobility shall have higher priority than RLM. Thus eventually it was agreed that UE will use 2/3 SMTC to do L3 measurement and 1/3 SMTC to do RLM.

Similarly, we can first discuss priority for other functionalities. For instance, in case of collision of SSB based RLM and CSI-RS based BFD with different SCS, which one shall be prioritized?

- RAN4 shall discuss how to treat the RS/channels with different priority

There could be several options, e.g. follow the example in current specification. Like SSB based LS and SSB based RLM, the colliding RS could be treated with different opportunity (or with different ratio).

Alternatively, UE can simply drop one of them, similar with the scenario of SSB L3 colliding with CSI-RS RLM with different SCS, where UE will only perform L3 measurement while drop CSI-RS RLM according to current specification. This option is not preferred since this may limit the network freedom when configuring RRM procedure and it can also be avoided by network configuration.

Another way is to give the control to network, e.g. network can decide the priority by taking into account various factors in real deployment by changing the ratio of resources for each functionality. The benefit is the network can have more freedom when configuring RRM procedure.

- RAN4 shall discuss if there is a need to discuss the priority for different RS

A simple example is the collision of SSB for RLM and CSI-RS for RLM with different SCS. It is not so obvious which one is more important.

Proposal 3: to address the colliding downlink reception, RAN4 shall discuss:

- the priority of different functionality

- how to treat the RS/channels with different priority

- if there is a need to discuss the priority for different RS

For question 2:

Question #2: If scheduling restriction due to Rx beamforming is applied on certain symbols, is the NW allowed to schedule PDSCH that spans a time duration containing these symbols but with configured rate matching resources used for protecting the symbols such that the UE does not receive PDSCH on symbols with scheduling restriction applied?

In our understanding the answer could be yes since the scheduling restriction defined in RAN4 is symbol based.

# Conclusion

In this contribution, we discuss the UE behavior on colliding downlink reception based on RAN1 LS. After discussion the following observations and proposals are made:

Observation 1: without further clarification, UE needs to meet RRM requirements for both procedure in case of colliding downlink reception of group 1 and group 2, which is not realistic in some scenario.

Observation 2: besides the colliding scenarios of group 1 and 2 mentioned in RAN1 LS, there are also other colliding scenarios need further clarification.

Proposal 1: some clarification is needed in Release 15 to address the colliding downlink reception issue.

Proposal 2: RAN4 shall extensively study all the colliding scenarios and capture corresponding UE behaviours in TS38.133 if necessary.

Proposal 3: to address the colliding downlink reception, RAN4 shall discuss:

- the priority of different functionality

- how to treat the RS/channels with different priority

- if there is a need to discuss the priority for different RS

# References

- R1-1809890, “LS on UE behaviour on reception of channels or RS in the same OFDM symbol”， Intel