3GPP TSG-RAN4 Meeting #90	R4-1900147

# Athens, Greece, Feb 25 – Mar 1, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test case for RRC Release with redirection to NR (section A.7.3.2.3.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2019-01-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no Test case for RRC Release with redirection to NR in FR2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce Test case for RRC Release with redirection to NR in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case for RRC Release with redirection to NR in FR2 is still missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.3.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### A.7.3.2.3	 SA: RRC Connection Release with Redirection

##### A.7.3.2.3.1 Redirection from NR in FR2 to NR in FR2

###### A.7.3.2.3.1.1 Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR to NR requirements specified in clause 6.2.3.2.1.

###### A.7.3.2.3.1.2 Test Parameters

Supported test configurations are shown in table A.7.3.2.3.1.2-1. The time delay is tested by using the parameters in table A.7.3.2.3.1.2-2, and A.7.3.2.3.1.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The “RRCConnectionRelease” message containing the relevant system information of Cell 2 shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.7.3.2.3.1.2-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.3.1.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 3 |  |

Table A.7.3.2.3.1.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| TCI configuration |  |  |  | TBD |  |  |  |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | TBD |  |  | TBD |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | TBD |  |  | TBD |  |  |  |
|  | Config 3 |  |  | TBD |  |  | TBD |  |  |  |
|  |  |  | dB | TBD | TBD |  | TBD |  | TBD |  |
|  |  |  | dB | TBD | TBD |  | TBD |  | TBD |  |
| IoNote3 | Config 1,2 |  | dBm/ BW | TBD | TBD |  | TBD |  | TBD |  |
|  | Config 3 |  | dBm/ BW | TBD | TBD |  | TBD |  | TBD |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone |  |  |  |  |  |  |  |  |  |  |

###### A.7.3.2.3.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [2040] ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

where:

TRRC_procedure_delay = [110] ms and is specified in clause 12 in TS 38.331 [2].

Tidentify-NR = [1760] ms in the test.

TSI-NR = 0 ms is assumed, since the UE is provided with the SI (including MIB and all relevant SIBs) of the target NR cell before the RRC connection is released by the old NR cell.

TRACH = [170] ms in the test.

This gives a total of [2040] ms.

End of Change 1