3GPP TSG-RAN WG4 Meeting #89                                         R4-1900293

Athens, GR, 25 Feb - 1 Mar, 2019

Agenda Item:   6.1

Source:        Samsung

Title:           Discussion on UTRA protection to NR operation

Document for:  Approval

# Introduction

It was CR to include UTRA UE spurious requirement to protect NR UE proposed in RAN4#89 as [1]. However, there is concern on the necessity to update NR band in UTRA spec. And the CR is noted. This contribution provides further considerations on this issue.

# Discussion

It’s understood that the concern to introduce UTRA protection to NR operation in UE spec is due to the query on possibility of UTRA&NR co-existence in the same geography area. Even though in NR UE spec of TS38.101-1 there is no explicit UMTS&NR co-existence spurious requirement defined, which can be verified by co-existence spurious requirement with corresponding LTE band sharing the same frequency range, there is still UTRA ACLR requirement specified in NR FR1 UE spec. Furthermore, the UTRA protection to NR operation has already been included in TS25.101 since Sep last year. And UTRA FDD BS spec is continually updated according to NR band definition since then on. They are all the evidences shows that the possibility UTRA and NR operation in same area is not precluded. In addition, update on UTRA FDD protection to LTE band newly introduced continues as well. The

Proposal 1, NR protection should be addressed in TS25.101.

In TS25.101, the spurious emission for co-existence is defined with specific protected frequency range rather than operating band#. And it facilities the NR protection impact on UTRA FDD UE spec since only new spectrum for NR operation needs to be added for co-existence.

So far, the purely new NR bands defined in Rel-15 are NR band n77, n78 and n79, which are not refarming band or subset of refarming band in FR1. These are only three bands should be involved in UTRA FDD protection for NR operation. And the protection for remaining spectrum has already reflected in legacy spec.

Observation 1, only requirements of UTRA FDD UE protection to NR new band n77, n78 and n79 are missing for UTRA FDD co-existence with NR. The impact on UTRA FDD is marginal.

It should be clarified that, the UTRA FDD protection to NR operation shall restrict within FR1 licnesed bands only .It is not envisioned that such protection will be extended in UTRA spec without any restriction. And the extension for UTRA FDD protection for any new NR band in newly extended or defined frequency range shall be discussed case by case.

Proposal 2, the necessity of UTRA FDD protection to new FR1 band shall be discussed case by case.

# Conclusion

It is proposed to incorporate the NR protection in UTRA FDD spec for Rel-15 R1 NR new bands, i.e., NR band n77, n78 and n79 as resubmission CR in [2]. For new NR band(s) arise in the future, it could be discussed case by case the necessity of UTRA protection to new FR1 NR band.

# Reference

[1] R4-1814495, Introduction of NR protection in TS25.101

[2] R4-1900298, Introduction of NR protection in TS25.101 (R15)