3GPP TSG-RAN WG4 Meeting RAN4#90                                              R4-1900305

Athens ,Greece, 25 Feb -1 March, 2019

Agenda Item:   6.14.1.4.2

Source:        Samsung

Title:          Simulation results for CQI test cases

Document for:  Discussion

# Introduction

In this contribution, simulation results for fading CQI and frequency selective CQI test cases were provided.

# Discussion

## Simulation results for wideband fading CQI test cases

In below tables, simulation results were provided for fading CQI test cases.

Table 1: Wideband fading CQI test case for FR1 FDD

| SNR [dB] | FR1 FDD 2*2 |  |  | FR1 FDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI | BLER with following CQI | TP ratio | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| 0 | 20.00 | 0.08 | 1.46 | 1.65 | 0.14 | 1.24 |
| 2 | 22.20 | 0.04 | 1.41 | 3.35 | 0.19 | 1.37 |
| 4 | 30.45 | 0.04 | 1.37 | 8.30 | 0.11 | 1.31 |
| 6 | 34.75 | 0.06 | 1.38 | 11.10 | 0.08 | 1.35 |
| 8 | 38.35 | 0.07 | 1.60 | 10.30 | 0.07 | 1.21 |
| 10 | 36.50 | 0.09 | 1.68 | 11.10 | 0.07 | 1.30 |
| 12 | 30.85 | 0.09 | 1.27 | 18.40 | 0.07 | 1.18 |
| 14 | 39.40 | 0.11 | 1.31 | 19.40 | 0.09 | 1.42 |
| 16 | 52.75 | 0.11 | 1.65 | 22.20 | 0.11 | 1.22 |
| 18 | 38.15 | 0.14 | 1.16 | 15.85 | 0.13 | 1.34 |
| 20 | 31.55 | 0.16 | 1.09 | 5.80 | 0.18 | 1.18 |

Table 2: Wideband fading CQI test case for FR1 TDD

| SNR [dB] | FR1 TDD 2*2 |  |  | FR1 TDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI | BLER with following CQI | TP ratio | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| 0 | 16.49 | 0.13 | 1.41 | 2.55 | 0.18 | 1.29 |
| 2 | 18.95 | 0.10 | 1.38 | 5.76 | 0.22 | 1.21 |
| 4 | 35.64 | 0.13 | 1.41 | 10.12 | 0.15 | 1.24 |
| 6 | 36.28 | 0.16 | 1.54 | 14.45 | 0.19 | 1.28 |
| 8 | 42.49 | 0.20 | 1.47 | 11.93 | 0.16 | 1.23 |
| 10 | 36.65 | 0.17 | 1.86 | 12.36 | 0.16 | 1.12 |
| 12 | 36.87 | 0.18 | 1.38 | 18.26 | 0.13 | 1.15 |
| 14 | 36.55 | 0.28 | 1.07 | 22.55 | 0.17 | 1.65 |
| 16 | 46.47 | 0.25 | 1.27 | 27.87 | 0.22 | 1.12 |
| 18 | 45.37 | 0.27 | 1.59 | 19.92 | 0.20 | 1.11 |
| 20 | 51.49 | 0.26 | 1.61 | 7.32 | 0.19 | 1.27 |

Table 3: Wideband fading CQI test case for FR2 TDD

| SNR [dB] | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| --- | --- | --- | --- |
| 0 | 3.38 | 0.18 | 1.12 |
| 2 | 4.82 | 0.15 | 1.34 |
| 4 | 5.86 | 0.13 | 1.25 |
| 6 | 18.39 | 0.11 | 1.09 |
| 8 | 14.12 | 0.18 | 1.20 |
| 10 | 11.90 | 0.17 | 1.61 |
| 12 | 11.31 | 0.12 | 1.15 |
| 14 | 4.81 | 0.09 | 1.37 |
| 16 | 3.65 | 0.08 | 1.83 |
| 18 | 2.44 | 0.06 | 1.20 |
| 20 | 5.15 | 0.05 | 1.15 |

Based on above simulation results, we have below proposals:

Proposal1: For FR1 2Rx wideband fading CQI test cases (FDD and TDD):

- Test point: 6/7dB  +12/13 dB

- Test requirements:

  - Aifa>20%

  - BLER>2%

  - Gamma>1.05

Proposal2: For FR1 4Rx wideband fading CQI test cases (FDD and TDD):

- Test point: 3/4dB  +9/10 dB

- Test requirements:

  - Aifa>5%

  - BLER>2%

  - Gamma>1.05

Proposal3: For FR2 2Rx wideband fading CQI test cases:

- Test point: 6/7dB  +12/13 dB

- Test requirements:

  - Aifa>5%

  - BLER>2%

  - Gamma>1.05

## Simulation results for frequency-selective CQI test cases

In below tables, simulation results were provided for sub-band CQI test cases.

Table 6: Sub-band CQI test case for FR1 FDD

| SNR [dB] | FR1 FDD 2*2 |  |  | FR1 FDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI | BLER with following CQI | TP ratio | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| 0 | 82.40 | 0.32 | 1.08 | 77.60 | 0.43 | 1.38 |
| 2 | 81.90 | 0.28 | 1.35 | 77.70 | 0.34 | 1.21 |
| 4 | 82.30 | 0.24 | 1.16 | 77.90 | 0.32 | 1.14 |
| 6 | 76.50 | 0.24 | 1.09 | 76.60 | 0.30 | 1.09 |
| 8 | 83.70 | 0.19 | 1.19 | 83.69 | 0.27 | 1.17 |
| 10 | 84.30 | 0.19 | 1.13 | 86.70 | 0.24 | 1.13 |
| 12 | 86.00 | 0.19 | 1.07 | 89.80 | 0.23 | 1.12 |
| 14 | 88.20 | 0.19 | 1.06 | 84.20 | 0.22 | 1.10 |
| 16 | 91.20 | 0.18 | 1.06 | 84.70 | 0.20 | 1.11 |
| 18 | 84.20 | 0.18 | 1.04 | 92.80 | 0.20 | 1.15 |
| 20 | 89.30 | 0.16 | 1.10 | 89.50 | 0.21 | 1.13 |
| 0 | 82.40 | 0.32 | 1.08 | 77.60 | 0.43 | 1.38 |
|  | FR1 FDD 1*2 |  |  | FR1 FDD 1*4 |  |  |
| 2 | 65.40 | 0.35 | 2.12 | 32.80 | 0.62 | 0.96 |
| 4 | 51.70 | 0.29 | 1.33 | 26.70 | 0.56 | 1.17 |
| 6 | 37.30 | 0.24 | 1.59 | 36.00 | 0.45 | 1.10 |
| 8 | 27.80 | 0.21 | 1.82 | 21.60 | 0.40 | 1.34 |
| 10 | 21.30 | 0.17 | 2.00 | 24.80 | 0.34 | 1.00 |
| 12 | 24.40 | 0.16 | 1.63 | 20.40 | 0.31 | 1.47 |
| 14 | 18.50 | 0.15 | 1.85 | 16.50 | 0.31 | 1.24 |
| 16 | 16.00 | 0.13 | 1.67 | 15.50 | 0.29 | 1.35 |
| 18 | 12.50 | 0.13 | 1.89 | 11.80 | 0.27 | 1.52 |
| 20 | 12.30 | 0.12 | 1.57 | 11.60 | 0.29 | 1.34 |

Table 2: Sub-band CQI test case for FR1 TDD

| SNR [dB] | FR1 TDD 2*2 |  |  | FR1 TDD 2*4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of Reported CQI | BLER with following CQI | TP ratio | Percentage of Reported CQI | BLER with following CQI | TP ratio |
| 0 | 84.11 | 0.30 | 1.12 | 73.41 | 0.40 | 1.45 |
| 2 | 83.28 | 0.24 | 1.40 | 76.00 | 0.33 | 1.33 |
| 4 | 84.60 | 0.20 | 1.24 | 79.99 | 0.28 | 1.25 |
| 6 | 87.68 | 0.17 | 1.19 | 83.00 | 0.24 | 1.22 |
| 8 | 89.08 | 0.15 | 1.15 | 83.20 | 0.20 | 1.29 |
| 10 | 92.09 | 0.14 | 1.20 | 86.77 | 0.18 | 1.24 |
| 12 | 89.57 | 0.13 | 1.17 | 84.74 | 0.17 | 1.23 |
| 14 | 93.14 | 0.12 | 1.17 | 83.62 | 0.15 | 1.22 |
| 16 | 91.11 | 0.11 | 1.17 | 89.78 | 0.14 | 1.21 |
| 18 | 89.84 | 0.11 | 1.16 | 92.02 | 0.14 | 1.25 |
| 20 | 92.30 | 0.10 | 1.19 | 91.81 | 0.14 | 1.24 |
|  | FR1 TDD 1*2 |  |  | FR1 TDD 1*4 |  |  |
| 0 | 72.64 | 0.31 | 2.23 | 33.73 | 0.48 | 1.04 |
| 2 | 53.25 | 0.24 | 1.38 | 26.52 | 0.36 | 0.93 |
| 4 | 38.14 | 0.19 | 1.63 | 21.55 | 0.36 | 1.07 |
| 6 | 30.86 | 0.15 | 1.96 | 28.34 | 0.28 | 1.25 |
| 8 | 23.51 | 0.13 | 2.00 | 25.40 | 0.22 | 1.00 |
| 10 | 18.47 | 0.12 | 1.70 | 19.87 | 0.26 | 1.29 |
| 12 | 20.43 | 0.11 | 1.94 | 17.07 | 0.21 | 1.15 |
| 14 | 11.55 | 0.10 | 1.73 | 16.31 | 0.20 | 1.28 |
| 16 | 8.68 | 0.09 | 1.95 | 11.48 | 0.16 | 1.43 |
| 18 | 7.56 | 0.09 | 1.63 | 8.61 | 0.13 | 1.33 |
| 20 | 12.46 | 0.08 | 1.79 | 6.72 | 0.10 | 1.46 |

Based on simulation results, we can find performance under 1Tx ports better than 2Tx ports.

Proposal4: Using 1Tx port to introduce frequency-selective CQI test case

Proposal5: For FR2 2Rx sub-band CQI test cases (FDD and TDD):

- Test point: 9/10dB  +14/15 dB

- Test requirements:

  - Aifa>2%

  - Beta>55%

  - Gamma>1.05

  - BLER>0.05

Proposal6: For FR2 4Rx sub-band CQI test cases (FDD and TDD):

- Test point: 6/7dB  +11/12dB

- Test requirements:

  - Aifa>2%

  - Beta>55%

  - Gamma>1.05

  - BLER>0.05

# Conclusion

In this contribution, we provided simulation results for CQI test with below observations and proposals.

Wideband Fading CQI test cases

Proposal1: For FR1 2Rx wideband fading CQI test cases (FDD and TDD):

- Test point: 6/7dB  +12/13 dB

- Test requirements:

  - Aifa>20%

  - BLER>2%

  - Gamma>1.05

Proposal2: For FR1 4Rx wideband fading CQI test cases (FDD and TDD):

- Test point: 3/4dB  +9/10 dB

- Test requirements:

  - Aifa>5%

  - BLER>2%

  - Gamma>1.05

Proposal3: For FR2 2Rx wideband fading CQI test cases:

- Test point: 6/7dB  +12/13 dB

- Test requirements:

  - Aifa>5%

  - BLER>2%

  - Gamma>1.05

Sub-band CQI test cases

Proposal4: Using 1Tx port to introduce frequency-selective CQI test case

Proposal5: For FR2 2Rx sub-band CQI test cases (FDD and TDD):

- Test point: 9/10dB  +14/15 dB

- Test requirements:

  - Aifa>2%

  - Beta>55%

  - Gamma>1.05

  - BLER>0.05

Proposal6: For FR2 4Rx sub-band CQI test cases (FDD and TDD):

- Test point: 6/7dB  +11/12dB

- Test requirements:

  - Aifa>2%

  - Beta>55%

  - Gamma>1.05

  - BLER>0.05

# Reference

[1] R4-1816491, “Way forward on NR UE CSI requirements”, Samsung

[2] RAN4 email reflector, “Simulation assumption of CSI test cases”, Samsung