3GPP TSG-RAN WG4 Meeting RAN4#90                                              R4-1900306

Athens ,Greece, 25 Feb -1 March, 2019

Agenda Item:   6.14.1.4.3

Source:        Samsung

Title:          Simulation results for PMI test cases

Document for:  Discussion

# Introduction

In this contribution, simulation results for PMI test cases were provided.

# Discussion

Simulation results for FR1 FDD

Figure 1 MCS 13 Rank1 with 4Tx2Rx WB PMI

Figure 2 MCS 13 Rank1 with 4Tx4Rx WB PMI

Figure 3 MCS 13 Rank2 with 8Tx2Rx WB PMI

Figure 4 MCS 13 Rank2 with 8Tx4Rx WB PMI

Simulation results for FR1 TDD

Figure 5 MCS 13 Rank1 with 4Tx2Rx WB PMI

Figure 6 MCS 13 Rank1 with 4Tx4Rx WB PMI

Figure 7 MCS 13 Rank2 with 8Tx2Rx WB PMI

Figure 8 MCS 13 Rank2 with 8Tx4Rx WB PMI

Reference SNR points with following PMI at  90% TP and relative throughput ratio as summarized in table below for FR1.

| Test cases | FDD | TDD |
| --- | --- | --- |
|  | 90% TP | 90% TP |
| 4Tx2Rx WB PMI with MCS13 rank1 | 2.0dB /2.2 | 2.4 dB/2.2 |
| 4Tx4Rx WB PMI with MCS13 rank1 | 0.7dB/2.2 | 1.1 dB/2.2 |
| 8Tx2Rx WB PMI with MCS13 rank2 | 7.4 dB/2.9 | 7.4 dB/2.9 |
| 8Tx4Rx WB PMI with MCS13 rank2 | 4.0 dB/3.3 | 3.9 dB/3.5 |

Simulation results for FR2 TDD

Figure 9 MCS 13 Rank1 with 2Tx2Rx WB PMI (ULA Low)-DDSU(FR2.120-1)

Figure 10 MCS 13 Rank1 with 2Tx2Rx WB PMI (ULA Medium)-DDSU(FR2.120-2)

Figure 11 MCS 13 Rank1 with 2Tx2Rx WB PMI (ULA Low) –DDDSU (FR2.120-1)

Figure 12 MCS 13 Rank1 with 2Tx2Rx WB PMI (ULA Medium) –DDDSU (FR2.120-1)

Reference SNR points with following PMI at 90% TP and relative throughput ratio as summarized in table below for FR2.

| Test cases | TDD |  |
| --- | --- | --- |
|  | DDSU (FR2.120-2) | DDDSU (FR2.120-1) |
| 2Tx2Rx WB PMI with MCS13 rank1 (ULA Low) | 5.2 dB/1.22 | 6.1 dB/1.08 |
| 2Tx2Rx WB PMI with MCS13 rank1 (ULA Medium) | 5.8 dB/1.30 | 6.9 dB/1.20 |

## Analysis

Based on simulation results above, we have below proposals and observations.

FR1 PMI test cases

Proposal 1: For FR1 8Tx PMI test cases, test requirement as 1.5 for FDD, TDD and 2Rx/4Rx.

FR2 PMI test cases

Observation 1: For FR2, with 2Tx ULA Medium correlation, throughput gain is larger than ULA Low MIMO correlation cases.

Proposal 2: For FR2, introducing 2Tx PMI test case under ULA Medium correlation.

Proposal 3: For FR2 PMI test case with TDD DL-UL pattern FR2.120-2, test requirement can 1.1

Proposal 4: For FR2 PMI test case with TDD DL-UL pattern FR2.120-1, test requirement can 1.1

# Conclusion

In this contribution, simulation results for agreed PMI test cases were provided.

FR1 PMI test cases

Proposal 1: For FR1 8Tx PMI test cases, test requirement as 1.5 for FDD, TDD and 2Rx/4Rx.

FR2 PMI test cases

Observation 1: For FR2, with 2Tx ULA Medium correlation, throughput gain is larger than ULA Low MIMO correlation cases.

Proposal 2: For FR2, introducing 2Tx PMI test case under ULA Medium correlation.

Proposal 3: For FR2 PMI test case with TDD DL-UL pattern FR2.120-2, test requirement can 1.1

Proposal 4: For FR2 PMI test case with TDD DL-UL pattern FR2.120-1, test requirement can 1.1

# Reference

[1] R4-1816491, “Way forward on NR UE CSI requirements”, Samsung

[2] RAN4 email reflector, “Simulation assumption of CSI test cases”, Samsung