# 3GPP TSG-RAN WG4 Meeting #90                                                                  R4-1900321

# Athens, Greece, 25th Feb - 1st March, 2019

Agenda Item:	6.14.2.3.1

Source:	ZTE Corporation

Title:	Simulation results for NR PUCCH FR2

Document for:	Approval

# Introduction

In RAN4#89, a WF on PUCCH demodulation performance was approved [1]. In this contribution, we provide the simulation results on PUCCH FR2 with both ideal and realistic assumptions. The missing ideal results for Format 0 will be provided during the meeting.

# Numerical results, FR2

| PUCCH FR2 1T2R, TDLA30-300 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | Ideal | Realistic |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 60kHz, 100MHz | 1 | 1 | 1 |  |  | 3.9 |
|  |  | 1 | 2 | 1 |  |  | -0.5 |
|  | 120kHz, 100MHz | 1 | 1 | 1 |  |  | 4.6 |
|  |  | 1 | 2 | 1 |  |  | -0.5 |
| Format 1 | 60kHz, 100MHz | 2 | 14 | 1 | ACK miss | -8.3 | -4.1 |
|  |  |  |  |  | NACK to ACK | -2.6 | 0.4 |
|  | 120kHz, 100MHz |  |  |  | ACK miss | -8.6 | -5.0 |
|  |  |  |  |  | NACK to ACK | -2.8 | -0.2 |
| Format 2 | 60kHz, 100MHz | 4 | 1 | 4 | ACK miss | 2.6 | 3.0 |
|  |  |  |  |  | NACK to ACK | -1.5 | 4.4 |
|  |  | 22 | 2 | 9 | BLER@1% | -2.2 | 0.0 |
|  | 120kHz, 100MHz | 4 | 1 | 4 | ACK miss | 2.3 | 2.7 |
|  |  |  |  |  | NACK to ACK | -2.4 | 3.0 |
|  |  | 22 | 2 | 9 | BLER@1% | -3.1 | -0.7 |
| Format 3 | 60kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -2.1 | 0.3 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1.2 | 0.9 |
|  | 120kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -2.1 | 0.7 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -3.1 | 0.1 |
| Format 4 | 60kHz, 100MHz | 22 | 14 | 1 | Without additional DMRS | 0.8 | 2.6 |
|  | 120kHz, 100MHz |  |  |  | Without additional DMRS | 0.8 | 2.8 |
| Format 0 | 60kHz, 50MHz | 1 | 1 | 1 |  |  | 4.4 |
|  |  | 1 | 2 | 1 |  |  | -1.1 |
|  | 120kHz, 50MHz | 1 | 1 | 1 |  |  | 4.5 |
|  |  | 1 | 2 | 1 |  |  | -0.5 |
|  | 120kHz, 200MHz | 1 | 1 | 1 |  |  | 5.2 |
|  |  | 1 | 2 | 1 |  |  | 0.4 |
| Format 1 | 60kHz, 50MHz | 2 | 14 | 1 | ACK miss | -8.7 | -4.3 |
|  |  |  |  |  | NACK to ACK | -2.6 | 0.5 |
|  | 120kHz, 50MHz |  |  |  | ACK miss | -9.1 | -5.1 |
|  |  |  |  |  | NACK to ACK | -3.2 | 0.3 |
|  | 120kHz, 200MHz |  |  |  | ACK miss | -8.8 | -5.2 |
|  |  |  |  |  | NACK to ACK | -3.3 | -0.1 |
| Format 2 | 60kHz, 50MHz | 4 | 1 | 4 | ACK miss | 2.5 | 2.9 |
|  |  |  |  |  | NACK to ACK | -1.2 | 4.6 |
|  |  | 22 | 2 | 9 | BLER@1% | -2.1 | 1.8 |
|  | 120kHz, 50MHz | 4 | 1 | 4 | ACK miss | 2.5 | 2.8 |
|  |  |  |  |  | NACK to ACK | -2.2 | 3.7 |
|  |  | 22 | 2 | 9 | BLER@1% | -3.1 | 0.8 |
|  | 120kHz, 200MHz | 4 | 1 | 4 | ACK miss | 2.4 | 2.7 |
|  |  |  |  |  | NACK to ACK | -2.3 | 3.2 |
|  |  | 22 | 2 | 9 | BLER@1% | -2.3 | 0.8 |
| Format 3 | 60kHz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -2.2 | 1.9 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1.1 | 2.8 |
|  | 120kHz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -2.1 | 1.9 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -2.2 | 0.9 |
|  | 120kHz, 200MHz | 16 | 14 | 1 | Without additional DMRS | -2.1 | 2.8 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -2.2 | 0.9 |
| Format 4 | 60kHz, 50MHz | 22 | 14 | 1 | Without additional DMRS | 0.8 | 3.9 |
|  | 120kHz, 50MHz |  |  |  | Without additional DMRS | 0.8 | 3.9 |
|  | 120kHz, 50MHz |  |  |  | Without additional DMRS | 0.8 | 4.8 |

# Reference

[1] R4-1816348	Way Forward for NR BS PUCCH demodulation performance, Huawei