# 3GPP TSG-RAN WG4 Meeting #90                                                                  R4-1900323

# Athens, Greece, 25th Feb - 1st March, 2019

Agenda Item:	6.14.2.2.1

Source:	ZTE Corporation

Title:	Simulation results for NR PRACH FR2

Document for:	Approval

# Introduction

In RAN4 #89, a WF was approved capturing further agreements on PRACH demodulation [1] together with another WF on how to derive the demodulation performance requirements according to the input simulation results from different companies [2]. And in our companion paper [3] we share an Excel script for implementing this derivation rule.

In this contribution, we provide the simulation results on PRACH FR2 with both ideal and realistic assumptions.

# Numerical results, FR2

| PRACH FR2 1T2R |  | Propagation condition | Frequency offset (Hz) | Ideal | Realistic |
| --- | --- | --- | --- | --- | --- |
| Format A1 | 60kHz | AWGN | 0 | -12.3 | -9.3 |
|  |  | TDLA30-300 | 4000 | -5.07 | -2.07 |
|  | 120kHz | AWGN | 0 | -11.2 | -8.2 |
|  |  | TDLA30-300 | 4000 | -4.72 | -1.72 |
| Format A2 | 60kHz | AWGN | 0 | -15.3 | -12.3 |
|  |  | TDLA30-300 | 4000 | -7.22 | -4.22 |
|  | 120kHz | AWGN | 0 | -14.2 | -11.2 |
|  |  | TDLA30-300 | 4000 | -7.67 | -4.67 |
| Format A3 | 60kHz | AWGN | 0 | -17.1 | -14.1 |
|  |  | TDLA30-300 | 4000 | -7.64 | -4.64 |
|  | 120kHz | AWGN | 0 | -16.1 | -13.1 |
|  |  | TDLA30-300 | 4000 | -9.06 | -6.06 |
| Format B4 | 60kHz | AWGN | 0 | -20.1 | -17.1 |
|  |  | TDLA30-300 | 4000 | -0.74 | 2.26 |
|  | 120kHz | AWGN | 0 | -19.1 | -16.1 |
|  |  | TDLA30-300 | 4000 | -10.11 | -7.11 |
| Format C0 | 60kHz | AWGN | 0 | -9.2 | -6.2 |
|  |  | TDLA30-300 | 4000 | -2.25 | 0.75 |
|  | 120kHz | AWGN | 0 | -8.2 | -5.2 |
|  |  | TDLA30-300 | 4000 | -1.71 | 1.29 |
| Format C2 | 60kHz | AWGN | 0 | -15.3 | -12.3 |
|  |  | TDLA30-300 | 4000 | -7.38 | -4.38 |
|  | 120kHz | AWGN | 0 | -14.2 | -11.2 |
|  |  | TDLA30-300 | 4000 | -7.86 | -4.86 |

# Reference

[1] R4-1816349 WF for NR PRACH demodulation performance, Ericsson

[2] R4-1816594 WF for test case SNR derivation procedure, Nokia

[3] R4-1900317 Excel script for deriving NR performance requirements, ZTE