3GPP TSG-RAN WG4 Meeting #90	R4-1900336

Athens, Greece, 25th Feb – 1st March, 2019

Agenda item:	5.3.4

Source: 	Qualcomm Incorporated

Title: 	Simulation results for R15 NB-IoT WUS

Document for:	Discussion

# Introduction

In RAN4#89 meeting, simulation assumptions for R15 NB-IoT WUS were agreed [1] which are reproduced here for reference. In this paper, we present simulation results for minimum number of warm-up subframes to achieve 1% false alarm and 99% detection probability.

| Parameter | Value |
| --- | --- |
| BS TX antenna configuration | 1 Tx, 2 Tx with transmit diversity, Note: Tx diversity scheme will depend on RAN1 agreements |
| UE Rx antenna configuration | 1 Rx |
| WUS BW | 180 kHz |
| Channel model | EPA 1Hz, ETU 1 Hz |
| SNR | -6 dB, -12 dB |
| False alarm rate | 1% |
| Detection probability | 99% |
| Combining | Option 1: No coherent combining over multiple subframes; Option 2: Coherent combining of 2 consecutive WUS subframes |
| Precoders | Orthogonal precoders alternate every 2 WUS subframes for 2 Tx based WUS transmissions |

| DRX cycle (s) | Frequency drift [Hz] | Timing drift [s] |
| --- | --- | --- |
| 1.28 | 170.8 | 4.0960e-08 |
| 2.56 | 311.6 | 1.6348e-07 |
| 5.12 | 593.2 | 6.5536e-07 |
| 10.24 | 1156.4 | 2.6212e-06 |

# Simulation results

Table 1 summarizes the minimum WUS to achieve 99% detection probability with 1% false alarm probability for 1Tx antenna. No significant performance improvement is seen from 2-sf coherent combining except in ETU1 channel under normal coverage. The performance is mainly dominated by 1% tail of fading channel gain.

Observation 1. No significant performance improvement is seen from 2-sf coherent combining. Performance is mainly dominated by 1% tail of fading channel gain.

Table 1 WUS for 1Tx

| SNR (dB) | DRX Cycle (s) | EPA1 (no coherent comb) | ETU1 (no coherent comb) | EPA1 (2sf coherent comb) | ETU1 (2sf coherent comb) |
| --- | --- | --- | --- | --- | --- |
| -6 | 1.28 | 64 | 64 | 64 | 32 |
|  | 2.56 | 64 | 64 | 64 | 32 |
|  | 5.12 | 64 | 64 | 64 | 32 |
|  | 10.24 | 64 | 64 | 64 | 32 |
| -12 | 1.28 | 128 | 128 | 128 | 128 |
|  | 2.56 | 128 | 128 | 128 | 128 |
|  | 5.12 | 128 | 128 | 128 | 128 |
|  | 10.24 | 128 | 128 | 128 | 128 |

Table 2 summarizes the minimum WUS to achieve 99% detection probability with 1% false alarm probability for 2Tx antenna. Significant performance improvement is seen compared to 1Tx scenario. Performance improvement from 2-sf coherent combining is now more evident particularly in lower DRX cycles. In higher DRX cycles, larger frequency error and time drift dictates the performance.

Observation 2. Performance improvement is seen in 2Tx compared to 1Tx. Performance improvement from 2-sf coherent combining is more evident in lower DRX cycles. In higher DRX cycle, larger frequency error and time drift dominates performance.

Table 2 WUS for 2Tx

| SNR (dB) | DRX Cycle (s) | EPA1 (no coherent comb) | ETU1 (no coherent comb) | EPA1 (2sf coherent comb) | ETU1 (2sf coherent comb) |
| --- | --- | --- | --- | --- | --- |
| -6 | 1.28 | 32 | 32 | 16 | 16 |
|  | 2.56 | 32 | 32 | 32 | 16 |
|  | 5.12 | 32 | 32 | 32 | 32 |
|  | 10.24 | 32 | 32 | 32 | 32 |
| -12 | 1.28 | 64 | 128 | 64 | 64 |
|  | 2.56 | 64 | 128 | 64 | 64 |
|  | 5.12 | 64 | 128 | 64 | 64 |
|  | 10.24 | 128 | 128 | 128 | 128 |

# Conclusions

Observation 1. No significant performance improvement is seen from 2-sf coherent combining. Performance is mainly dominated by 1% tail of fading channel gain.

Observation 2. Performance improvement is seen in 2Tx compared to 1Tx. Performance improvement from 2-sf coherent combining is more evident in lower DRX cycles. In higher DRX cycle, larger frequency error and time drift dominates performance.

Based on results in Tables 1 and 2, we propose the following for Tables 4.6.9.2-1 and 4.6.9.2-2 in [2]:

Table 4.6.9.2-1 Conditions for WUS reception for UE normal coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1Tx | Required number of repetition of WUS signal with 2Tx |
| --- | --- | --- |
| 1.28 | 64 | 16 |
| 2.56 | 64 | 32 |
| 5.12 | 64 | 32 |
| 10.24 | 64 | 32 |

Table 4.6.9.2-2 Conditions for WUS reception for UE enhanced coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1Tx | Required number of repetition of WUS signal with 2Tx |
| --- | --- | --- |
| 1.28 | 128 | 64 |
| 2.56 | 128 | 64 |
| 5.12 | 128 | 64 |
| 10.24 | 128 | 128 |

# References

[1] R4-1816332

[2] TS 36.133