3GPP TSG-RAN WG4 Meeting #90	R4-1900367

# Athens, Greece, 25 February 2019 - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | xxxx | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Normal FR1 PDSCH requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-15-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of Rel-15 NR FR1 Normal PDSCH requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add PDCCH parameters - Add paramaters for CSI-RS for beam management - Add “HARQ ACK/NACK bundling” configuration - Add TCI configuration - Remove square brackets in tables with tests purpose - Add K1 configuration for FDD tests - Move Enhanced requirements in separate table with requirements - Add new test for LTE-NR coexistence scenario - Update “LTE carrier Center” parameter for LTE-NR coexistence scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 NR FR1 Normal PDSCH requirements are not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 5.2	PDSCH demodulation requirements

The parameters specified in Table 5.2-1 are valid for all PDSCH tests unless otherwise stated.

Table 5.2-1: Common test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  | Symbols | 0, 1 |
|  | Number of PDCCH candidates and aggregation levels |  |  | AL16 |
|  | DCI format |  |  | 1_1 |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | ‘FD-CDM2’ |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | ‘FD-CDM2’ |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
| CSI-RS for beam management | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |
|  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2 30 kHz SCS: 40 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | Repetition |  |  | ON |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| PTRS configuration |  |  |  | PTRS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| Precoding configuration |  |  |  | SP Type I, Random per slot with PRB bundling granularity |
| Symbols for all unused Res |  |  |  | OCNG Annex A.5 |

### 5.2.1	1RX requirements

(Void)

### 5.2.2	2RX requirements

#### 5.2.2.1	FDD

##### 5.2.2.1.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.2.1.1-3 and Table 5.2.2.1.1-4, with the addition of test parameters in table 5.2.2.1.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.1-1.

Table 5.2.2.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 2-1, 2-2 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 2 receive antenna conditions and with 2 MIMO layers. | 3-1 |

Table 5.2.2.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 20 for Test 2-3 10 for other tests |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 51 for Test 2-3 52 for other tests |
|  | Subcarrier spacing | kHz | 30 for Test 2-3 15 for other tests |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 48 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Test 1-1 1 for other tests |
|  | Length |  | Single symbol |
| Number of HARQ Processes |  |  | 8 for Tests 1-4, [2-1] 4 for other tests |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | 2 |

Table 5.2.2.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | 70 | [-0.9] |
| 1-2 | R.PDSCH.1-1.2 FDD | QPSK, 0.30 | TDLC300-100 | 2x2, ULA Low | 70 | [0.5] |
| 1-3 | R.PDSCH.1-4.1 FDD | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | [24.5] |
| 1-4 | R.PDSCH.1-2.1 FDD | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 30 | [1.3] |

Table 5.2.2.1.1-4: Minimum performance for Rank 2

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD | 64QAM, 0.51 | TDLA30-10 | 2x2, ULA Low | 70 | TBD |
| 2-2 | R.PDSCH.2-1.1 FDD | 64QAM, 0.51 | TDLA30-10 | 2x2, ULA Low | 70 | TBD |

Table 5.2.2.1.1-5: Minimum performance for Rank 2 and Enhanced Type X Receiver

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Medium | 70 | [17.5] |

##### 5.2.2.1.2	Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH

The performance requirements are specified in Table 5.2.2.1.2-3, with the addition of test parameters in table 5.2.2.1.2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.2-1.

Table 5.2.2.1.2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and CSI-RS overlapped with PDSCH | 1-1 |

Table 5.2.2.1.2-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 10 |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 52 |
|  | Subcarrier spacing | kHz | 15 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 48 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| NZP CSI-RS for CSI acquisition | OFDM symbols in the PRB used for CSI-RS |  | l0 = 13 |
|  | CSI-RS periodicity |  | 5 |
| ZP CSI-RS for CSI acquisition | Subcarrier index in the PRB used for CSI-RS |  | (k0, k1, k2, k3)=(2, 4, 6, 8) |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CSI-RS periodicity |  | 5 |
| Number of HARQ Processes |  |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | 2 |

Table 5.2.2.1.2-3: Minimum performance for Rank 2

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-5.1 FDD | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | [14.7] |

##### 5.2.2.1.3	Minimum requirements for PDSCH Mapping Type B

The performance requirements are specified in Table 5.2.2.1.3-3, with the addition of test parameters in Table 5.2.2.1.3-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.3-1.

Table 5.2.2.1.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH mapping Type B performance under 2 receive antenna conditions | 1-1 |

Table 5.2.2.1.3-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 10 |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 52 |
|  | Subcarrier spacing | kHz | 15 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 48 |
| PDSCH configuration | Mapping type |  | Type B |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 5 |
|  | Length (L) |  | 7 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | 2 |

Table 5.2.2.1.3-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.4 FDD | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 70 | [-1.0] |

##### 5.2.2.1.4	Minimum requirements for PDSCH Mapping Type A and LTE-NR coexistence

The performance requirements are specified in Table 5.2.2.1.4-3, with the addition of test parameters in Table 5.2.2.1.4-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.4-1.

Table 5.2.2.1.4-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions with CRS rate matching configured | 1-1, 1-2 |

Table 5.2.2.1.4-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 10 |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 52 |
|  | Subcarrier spacing | kHz | 15 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 48 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 for Test 1-1 11 for Test 1-2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| CRS for rate matching | LTE carrier Center |  | Same as NR carrier Center |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | 2 |

Table 5.2.2.1.4-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.4 FDD | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | [-0.8] |
| 1-2 | R.PDSCH.1-1.5 FDD | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |

#### 5.2.2.2	TDD

##### 5.2.2.2.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.2.2.1-3 and Table 5.2.2.2.1-4, with the addition of test parameters in Table 5.2.2.2.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.1-1.

Table 5.2.2.2.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 1-6, 2-1, 2-2 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 2 receive antenna conditions and with 2 MIMO layers. | 3-1 |

Table 5.2.2.2.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 20 for Test 2-3 40 for other tests |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 51 for Test 2-3 106 for other tests |
|  | Subcarrier spacing | kHz | 30 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 48 for Test 2-3 102 for other tests |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Tests 1-1 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Test 1-1 1 for other tests |
|  | Length |  | 1 |
| Number of HARQ Processes |  |  | 16 for Test 1-4, [2-1] 8 for other tests |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | Specific to each UL-DL pattern |

Table 5.2.2.2.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-1.1 TDD | QPSK, 0.30 | FR1.30-1 | TDLB100-400 | 2x2, ULA Low | 70 | [-0.9] |
| 1-2 | R.PDSCH.2-1.2 TDD | QPSK, 0.30 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low | 70 | [0.3] |
| 1-3 | R.PDSCH.2-4.1 TDD | 256QAM, 0.82 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 70 | TBD |
| 1-4 | R.PDSCH.2-2.1 TDD | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low | 30 | [1.5] |
| 1-5 | R.PDSCH.2-5.1 TDD | QPSK, 0.30 | FR1.30-2 | TDLA30-10 | 2x2, ULA Low | 70 | [-0.9] |
| 1-6 | R.PDSCH.2-6.1 TDD | QPSK, 0.30 | FR1.30-3 | TDLA30-10 | 2x2, ULA Low | 70 | [-0.9] |

Table 5.2.2.2.1-4: Minimum performance for Rank 2

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.2-3.1 TDD | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 70 | TBD |
| 2-2 | R.PDSCH.2-3.2 TDD | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 70 | [19.2] |

Table 5.2.2.2.1-5: Minimum performance for Rank 2 and Enhanced Type X Receiver

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.2-2.2 TDD | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 2x2, ULA Medium | 70 | [18.0] |

##### 5.2.2.2.2	Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH

The performance requirements are specified in Table 5.2.2.2.2-3, with the addition of test parameters in Table 5.2.2.2.2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.2-1.

Table 5.2.2.2.2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and CSI-RS overlapped with PDSCH | 1-1 |

Table 5.2.2.2.2-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 40 |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
|  | Subcarrier spacing | kHz | 30 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 102 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| NZP CSI-RS for CSI acquisition | OFDM symbols in the PRB used for CSI-RS |  | l0 = 13 |
|  | CSI-RS periodicity |  | 5 |
| ZP CSI-RS for CSI acquisition | Subcarrier index in the PRB used for CSI-RS |  | (k0, k1, k2, k3)=(2, 4, 6, 8) |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CSI-RS periodicity |  | 5 |
| Number of HARQ Processes |  |  | 8 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | Specific to each UL-DL pattern |

Table 5.2.2.2.2-3: Minimum performance for Rank 2

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-7.1 TDD | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low | 70 | TBD |

##### 5.2.2.2.3	Minimum requirements for PDSCH Mapping Type B

The performance requirements are specified in Table 5.2.2.2.3-3, with the addition of test parameters in Table 5.2.2.2.3-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.3-1.

Table 5.2.2.2.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH mapping Type B performance under 2 receive antenna conditions | 1-1 |

Table 5.2.2.2.3-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 40 |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
|  | Subcarrier spacing | kHz | 30 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 102 |
| PDSCH configuration | Mapping type |  | Type B |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 5 |
|  | Length (L) |  | 7 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | Specific to each UL-DL pattern |

Table 5.2.2.2.3-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH,2-1.3 TDD | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 70 | [-1.0] |

### 5.2.3	4RX requirements

#### 5.2.3.1	FDD

##### 5.2.3.1.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.3.1.1-3, Table 5.2.3.1.1-4, Table 5.2.3.1.1-5 and Table 5.2.3.1.1-6, with the addition of test parameters in Table 5.2.3.1.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.1-1.

Table 5.2.3.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 2-1, 2-2, 3-1, 4-1 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 4 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 4 receive antenna conditions and with 3 MIMO layers. | 5-1 |

Table 5.2.3.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 20 for Test 2-2 10 for other tests |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 51 for Test 2-2 52 for other tests |
|  | Subcarrier spacing | kHz | 30 for Test 2-2 15 for other tests |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 51 for Test 2-2 52 for other tests |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 WB for Test 3-1 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Test 1-1 1 for other tests |
|  | Length |  | 1 |
| Number of HARQ Processes |  |  | 8 for Test 1-4, [2-1] 4 for other tests |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | 2 |

Table 5.2.3.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | QPSK, 0.30 | TDLB100-400 | 2x4, ULA Low | 70 | [-3.7] |
| 1-2 | R.PDSCH.1-1.2 FDD | QPSK, 0.30 | TDLC300-100 | 2x4, ULA Low | 70 | [-2.7] |
| 1-3 | R.PDSCH.1-4.1 FDD | 256QAM, 0.82 | TDLA30-10 | 2x4, ULA Low | 70 | [21.0] |
| 1-4 | R.PDSCH.1-2.1 FDD | 16QAM, 0.48 | TDLC300-100 | 2x4, ULA Low | 30 | [-1.5] |

Table 5.2.3.1.1-4: Minimum performance for Rank 2

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD | 64QAM, 0.51 | TDLA30-10 | 2x4, ULA Low | 70 | [TBD] |
| 2-2 | R.PDSCH.2-1.1 FDD | 64QAM, 0.51 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |

Table 5.2.3.1.1-5: Minimum performance for Rank 3

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.1-2.3 FDD | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | [10.9] |

Table 5.2.3.1.1-6: Minimum performance for Rank 4

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.1-2.4 FDD | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | [15.5] |

Table 5.2.3.1.1-7: Minimum performance for Rank 3 and Enhanced Type X Receiver

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5-1 | R.PDSCH.1-2.3 FDD | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Medium A | 70 | [22.2] |

##### 5.2.3.1.2	Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH

The performance requirements are specified in Table 5.2.3.1.2-3, with the addition of test parameters in table 5.2.3.1.2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.2-1.

Table 5.2.3.1.2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions and CSI-RS overlapped with PDSCH | 1-1 |

Table 5.2.3.1.2-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 10 |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 52 |
|  | Subcarrier spacing | kHz | 15 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 48 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| NZP CSI-RS for CSI acquisition | OFDM symbols in the PRB used for CSI-RS |  | l0 = 13 |
|  | CSI-RS periodicity |  | 5 |
| ZP CSI-RS for CSI acquisition | Subcarrier index in the PRB used for CSI-RS |  | (k0, k1, k2, k3)=(2, 4, 6, 8) |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CSI-RS periodicity |  | 5 |
| Number of HARQ Processes |  |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | 2 |

Table 5.2.3.1.2-3: Minimum performance for Rank 2

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-5.1 FDD | 16QAM, 0.48 | TDLC300-100 | 4x4, ULA Low | 70 | [9.0] |

##### 5.2.3.1.3	Minimum requirements for PDSCH Mapping Type B

The performance requirements are specified in Table 5.2.3.1.3-3, with the addition of test parameters in Table 5.2.3.1.3-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.3-1.

Table 5.2.3.1.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| PDSCH mapping Type B performance under 4 receive antenna conditions | 1-1 |

Table 5.2.3.1.3-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 10 |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 52 |
|  | Subcarrier spacing | kHz | 15 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 48 |
| PDSCH configuration | Mapping type |  | Type B |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 5 |
|  | Length (L) |  | 7 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | 2 |

Table 5.2.3.1.3-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.4 FDD | QPSK, 0.30 | TDLA30-10 | 4x4, ULA Low | 70 | [-4.0] |

##### 5.2.3.1.4	Minimum requirements for PDSCH Mapping Type A and LTE-NR coexistence

The performance requirements are specified in Table 5.2.3.1.4-3, with the addition of test parameters in Table 5.2.3.1.4-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.4-1.

Table 5.2.3.1.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions with CRS rate matching configured | 1-1, 1-2 |

Table 5.2.3.1.3-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 10 |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 52 |
|  | Subcarrier spacing | kHz | 15 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 48 |
| PDSCH configuration | Mapping type |  | 0 |
|  | k0 |  | 52 |
|  | Starting symbol (S) |  | 15 |
|  | Length (L) |  | 48 |
|  | PDSCH aggregation factor |  | Type A |
|  | PRB bundling type |  | 0 |
|  | PRB bundling size |  | 3 |
|  | Resource allocation type |  | 9 for Test 1-1 11 for Test 1-2 |
|  | VRB-to-PRB mapping type |  | 1 |
|  | VRB-to-PRB mapping interleaver bundle size |  | Static |
| PDSCH DMRS configuration | DMRS Type |  | 2 |
|  | Number of additional DMRS |  | Type 0 |
|  | Length |  | Non-interleaved |
| CRS for rate matching | LTE carrier Center |  | Same as NR carrier Center |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | 2 |

Table 5.2.3.1.3-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.4 FDD | QPSK, 0.30 | TDLA30-10 | 4x4, ULA Low | 70 | [-3.9] |
| 1-2 | R.PDSCH.1-1.5 FDD | QPSK, 0.30 | TDLA30-10 | 4x4, ULA Low | 70 | TBD |

#### 5.2.3.2	TDD

##### 5.2.3.2.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.3.2.1-3, Table 5.2.3.2.1-4, Table 5.2.3.2.1-5 and Table 5.2.3.2.1-6, with the addition of test parameters in Table 5.2.3.2.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.1-1.

Table 5.2.3.2.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under4 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 1-6, 2-1, 2-2, 3-1, 4-1 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 4 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 4 receive antenna conditions and with 3 MIMO layers. | 5-1 |

Table 5.2.3.2.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 20 for Test 2-2 40 for other tests |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 51 for Test 2-2 106 for other tests |
|  | Subcarrier spacing | kHz | 30 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 48 for Test 2-2 102 for other tests |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1 1 for other tests |
|  | Length |  | 1 |
| Number of HARQ Processes |  |  | 16 for Test 1-4, [2-1] 8 for other tests |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | Specific to each UL-DL pattern |

Table 5.2.3.2.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-1.1 TDD | QPSK, 0.30 | FR1.30-1 | TDLB100-400 | 2x4, ULA Low | 70 | [-3.9] |
| 1-2 | R.PDSCH.2-1.2 TDD | QPSK, 0.30 | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 70 | [-2.7] |
| 1-3 | R.PDSCH.2-4.1 TDD | 256QAM, 0.82 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | [TBD] |
| 1-4 | R.PDSCH.2-2.1 TDD | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 30 | [-1.1] |
| 1-5 | [R.PDSCH.2-5.1 TDD] | QPSK, 0.3 | FR1.30-2 | TDLA30-10 | 2x4, ULA Low | 70 | [-3.9] |
| 1-6 | [R.PDSCH.2-6.1 TDD] | QPSK, 0.30 | FR1.30-3 | TDLA30-10 | 2x4, ULA Low | 70 | [-3.9] |

Table 5.2.3.2.1-4: Minimum performance for Rank 2

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.2-3.1 TDD | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 2-2 | R.PDSCH.2-3.2 TDD | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | [13.8] |

Table 5.2.3.2.1-5: Minimum performance for Rank 3

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.2-2.3 TDD | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | [11.4] |

Table 5.2.3.2.1-6: Minimum performance for Rank 4

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.2-2.4 TDD | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | [16.1] |

Table 5.2.3.2.1-7: Minimum performance for Rank 3 and Enhanced Type X Receiver

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5-1 | R.PDSCH.2-2.3 TDD | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Medium A | 70 | [22.9] |

##### 5.2.3.2.2	Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH

The performance requirements are specified in Table 5.2.3.2.2-3, with the addition of test parameters in table 5.2.3.2.2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.2-1.

Table 5.2.3.2.2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions and CSI-RS overlapped with PDSCH | 1-1 |

Table 5.2.3.2.2-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 40 |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
|  | Subcarrier spacing | kHz | 30 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 102 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| NZP CSI-RS for CSI acquisition | OFDM symbols in the PRB used for CSI-RS |  | l0 = 13 |
|  | CSI-RS periodicity |  | 5 |
| ZP CSI-RS for CSI acquisition | Subcarrier index in the PRB used for CSI-RS |  | (k0, k1, k2, k3)=(2, 4, 6, 8) |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CSI-RS periodicity |  | 5 |
| Number of HARQ Processes |  |  | 8 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | Specific to each UL-DL pattern |

Table 5.2.3.2.2-3: Minimum performance for Rank 2

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-7.1 TDD | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 4x4, ULA Low | 70 | TBD |

##### 5.2.3.2.3	Minimum requirements for PDSCH Mapping Type B

The performance requirements are specified in Table 5.2.3.2.3-3, with the addition of test parameters in Table 5.2.3.2.3-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.3-1.

Table 5.2.3.2.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| PDSCH mapping Type B performance under 4 receive antenna conditions | 1-1 |

Table 5.2.3.2.3-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 40 |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
|  | Subcarrier spacing | kHz | 30 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 102 |
| PDSCH configuration | Mapping type |  | Type B |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 5 |
|  | Length (L) |  | 7 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | Specific to each UL-DL pattern |

Table 5.2.3.2.3-3: Minimum performance for Rank 1

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH,2-1.3 TDD | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | [-4.0] |

END OF CHANGE