3GPP TSG-RAN WG4 Meeting #90 	R4-1900371

Athens, Greece, Feburary 25 – March 1, 2018

Agenda item:	6.4.3

Source:	Intel Corporation

Title:	On SFTD accuracy requirement for NR-NR DC

Document for:	Discussion

# Introduction

In this contribution, we propose the SFTD measurement accuracy requirement for NR-NR DC.

# Discussion

Since SFTD is an extension of E-UTRA SSTD measurement reporting to allow reporting of time difference between E-UTRA and NR cells, it is reasonable to follow the same mechanism as SSTD accuracy requirement to design the accuracy requirement for SFTD measurement.

The SSTD accuracy requirement specified in TS36.133 section 9.1.20 is derived from the requirements on UE accuracy for uplink transmit timing are captured in TS 36.133 section 7.1[1]. The argument is that since the transmit timing is depending on the first detected path of the corresponding radio frame in the downlink, UE derives the transmit timing based on the DL reception timing. Hence the accuracy of the transmit timing is directly dependent on the accuracy of the downlink timing as determined by the UE.

Following the same design principle as EN-DC in LTE, the SFTD accuracy for NR-NR DC can be expressed as:

SFTD accuracy = TX timing error in NR PCell + TX timing error in NR PSCell + margin

For NR cell, the TX timing error is captured in TS38.133 section 7.1.2, which is shown in below.

It is shown that the maximum timing error is 12TS for FR1 and 3.5 TS for FR2. In EN-DC, the implementation margin for SFTD accuracy requirement is 4Ts [2].  Then the same value of 4Ts is applied for FR1 NR-NR DC. Since 4Ts margin is 1/3 of timing error in FR1, it is obtained that 3.5Ts/3 =1.2Ts, and we round it to 2TS for FR2, Similarly, we use 3TS for mixed FR1 and FR2. Thus, we propose that

Proposal 1: The SFTD accuracy requirement for NR-NR DC is

- 28Ts if both two NR cells are operated in FR1;

- 18.5Ts if one NR cell is operated in FR1 and the other NR cell is operated in FR2;

- 9Ts if both two NR cells are operated in FR2.

# Summary and Conclusions

This contribution discusses the remaining issues on SFTD measurement accuracy for NR-NR DC. The conclusions are drawn as follows:

Proposal 1: The SFTD accuracy requirement for NR-NR DC is

- 28Ts if both two NR cells are operated in FR1;

- 18.5Ts if one NR cell is operated in FR1 and the other NR cell is operated in FR2;

- 9Ts if both two NR cells are operated in FR2.

# References

- R4-156288, “Measurement accuracy of subframe offset reporting for Dual Connectivity”, Ericsson, RAN4 meeting #76bits

- R4-1809879, “On SFTD Accuracy Requirement”, Intel, RAN4 meeting #88