3GPP TSG RAN WG4 #90	R4-1900438

# Athens, Greece, 25 Feb 2019 - 1 Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | CRNum | rev | 0 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on FR2 PDSCH Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are a lot of TBD parameters in the requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some of the TBD parameters were defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for some scenarios will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-4 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of change>>

## 7.2	PDSCH demodulation requirements

The parameters specified in Table 7.2-1 are valid for all PDSCH demodulation tests unless otherwise stated.

Table 7.2-1: Common Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH transmission scheme |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  | dB | 0 |
| DL BWP configuration #1 | Cyclic prefix |  | Normal |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | 1 |
|  | SSB periodicity | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  | 2 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH |  | 0 |
|  | Number of PDCCH candidates and aggregation levels |  | 1/8 |
|  | DCI format |  | 1-0 |
| Cross carrier scheduling |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | CSI-RS resource 1: 6 CSI-RS resource 2: 10 CSI-RS resource 3: 6 CSI-RS resource 4: 10 |
|  | Number of CSI-RS ports (X) |  | 1 |
|  | CDM Type |  | No CDM |
|  | Density (ρ) |  | 3 |
|  | CSI-RS periodicity | Slots | 160 |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 12 |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | CSI-RS periodicity | Slots | 160 |
|  | CSI-RS offset |  | 0 |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 12 |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | CSI-RS periodicity | Slots | 160 |
|  | CSI-RS offset |  | 0 |
| PDSCH DMRS configuration | Antenna ports indexes |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 |
| PTRS configuration | Frequency density (KPT-RS) |  | 2 |
|  | Time density (LPT-RS) |  | 1 |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| Precoding configuration |  |  | SP Type I, Random per slot with PRB bundling granularity |
| Symbols for all unused Res |  |  | OCNG in Annex A.5 |

### 7.2.1	1RX requirements

(Void)

### 7.2.2	2RX requirements

#### 7.2.2.1	FDD

(Void)

#### 7.2.2.2	TDD

##### 7.2.2.2.1	Minimum requirements for PDSCH Mapping Type-A

For PDSCH Type-A scheduling, the requirements are specified in Table 7.2.2.2.1-3, 7.2.2.2.1-4 and 7.2.2.2.1-5, with the addition of the parameters in Table 7.2.2.2.1-2 and the downlink physical channel setup according to Annex C.5.1. The purpose is to verify the performance of PDSCH Type-A scheduling.

The test purposes are specified in Table 7.2.2.1.1-1.

Table 7.2.2.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and Rank1 | [1-1, 1-3] |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and Rank2 | [2-1, 2-2, 2-3, 2-4, 2-5, 2-6] |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | [1-2] |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 2 receive antenna conditions and with 2 MIMO layers. | [3-1] |

Table 7.2.2.2.1-2: Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 32 for 2-3, 132 for 2-4, 66 for other tests |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 32 for 2-3, 132 for 2-4, 66 for other tests |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | As defined in Annex A.1.3 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | WB for 1-1, 2 for other tests |
|  | Resource allocation type |  | Type 1 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | Single-symbol DM-RS |
|  | Antenna ports indexes |  | {1000} for Rank1 {1000,1001} for Rank2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 |
| Number of HARQ Processes |  |  | 8 for Test 1-1, 1-3, 2-4 10 for Test 2-1, 2-3, 2-5, 2-6, 3-1 16 for Test 1-2 TBD for Test 2-2 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | As defined in Annex A.1.3 |

Table 7.2.2.2.1-3: Minimum performance for Rank 1 (FRC)

| Test num. |  |  | Reference channel |  |  | Bandwidth/Subcarrier spacing | Modulation and code rate |  |  | TDD UL-DL pattern |  |  | Propagation condition |  |  | Correlation matrix and antenna configuration |  |  | Reference value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) |  | SNRBB (dB) |  |
| 1-1 | R.PDSCH. 5-1.1_TDD | 100MHz/120kHz | QPSK, 0.30 | FR2.120-1 | TDLC60-300 | 2x2 ULA Low | 70 | [-0.5] |
| 1-2 | R.PDSCH. 5-2.1_TDD | 100MHz/120kHz | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 30 | [1.6] |
| 1-3 | R.PDSCH. 5-3.1_TDD | 100MHz/120kHz | 64QAM, 0.45 | FR2.120-1 | TDLA30-300 | 2x2 XPL Med-A | 70 | [TBD] |

Table 7.2.2.2.1-4: Minimum performance for Rank 2 (FRC)

| Test num. |  | Reference channel |  | Bandwidth/Subcarrier spacing |  | Modulation and code rate |  | TDD UL-DL pattern |  | Propagation condition |  | Correlation matrix and antenna configuration |  | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) |  | SNRBB (dB) |  |
| 2-1 | R.PDSCH. 5-4.1_TDD |  | 100MHz/120kHz |  | QPSK, 0.30 |  | FR2.120-2 |  | TDLA30-75 |  | 2x2 ULA Low |  | 70 |  | [TBD] |  |
| 2-2 | R.PDSCH. 5-2.2_TDD |  | 100MHz/120kHz |  | 16QAM, 0.48 |  | FR2.120-1 |  | TDLA30-300 |  | 2x2 ULA Low |  | 70 |  | [TBD] |  |
| 2-3 | R.PDSCH. 5-5.2_TDD |  | 50MHz/120kHz |  | 16QAM,0.48 |  | FR2.120-2 |  | TDLA30-75 |  | 2x2 ULA Low |  | 70 |  | [13.9] |  |
| 2-4 | R.PDSCH. 5-2.3_TDD |  | 200MHz/120kHz |  | 16QAM, 0.48 |  | FR2.120-1 |  | TDLA30-300 |  | 2x2 ULA Low |  | 70 |  | [TBD] |  |
| 2-5 | R.PDSCH. 4-1.1_TDD |  | 50MHz/60kHz |  | 16QAM, 0.48 |  | FR2.60-1 |  | TDLA30-75 |  | 2x2 ULA Low |  | 70 |  | [14.3] |  |
| 2-6 | R.PDSCH. 5-6.1_TDD |  | 100MHz/120kHz |  | 64QAM, 0.43 |  | FR2.120-2 |  | TDLA30-75 |  | 2x2 ULA Low |  | 70 |  | [TBD] |  |

Table 7.2.2.2.1-5: Minimum performance for Rank 2 (FRC) for Enhanced Type X Receiver

| Test num. |  | Reference channel |  |  | Bandwidth/Subcarrier spacing |  | Modulation and code rate |  |  | TDD UL-DL pattern |  |  | Propagation condition |  |  | Correlation matrix and antenna configuration |  |  | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) |  | SNRBB (dB) |
| 3-1 |  |  | R.PDSCH. 5-5.1_TDD |  |  | 100MHz/120kHz |  |  | 16QAM, 0.48 |  |  | FR2.120-2 |  |  | TDLA30-75 |  |  | 2x2 ULA Med |  |  | 70 |  |  | [19.5] |

<< End of change>>