3GPP TSG-RAN WG4 Meeting #90	R4-1900603

Athens, GR, 25th Feb – 1th Mar, 2019

Agenda item:	6.14.1.4.3

Source: 		NTT DOCOMO, INC.

Title: 	Simulation results for PMI test in FR2

Document for:	Discussion

1. Introduction

This contribution presents our simulation results for following PMI test in FR2 (DDDSU) that was agreed in the last meeting [1].

2. Simulation results

We follow simulation assumption in Table. A1, which is identical to the simulation assumption of Test 4.6 discussed in the email reflector before RAN4 #90. Figure 1 and 2 show simulation results for follow PMI tests for 2Tx2Rx ULA Low and 2Tx2Rx ULA Medium, respectively.

Figure 1: Test 4.6 (ULA Low)                                                Figure 2: Test 4.6 (ULA Med.)

PMI requirements are specified in terms of the ratio as follows.

In the definition of γ, 70 and 90 % of the maximum throughput obtained at SNRUE using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRUE with random precoding.

Table I: Summary of link performance for Test 4.6

| SCS | CBW | Antenna | Test point | Reference SNR point | Test requirement | Difference between each test requirement |
| --- | --- | --- | --- | --- | --- | --- |
| 120 kHz | 100 MHz | 2Tx2Rx (ULA Low) | 90% | 5.5 dB | 1.12 | - |
|  |  |  | 70% | 3.1 dB | 1.20 | +0.08 |
|  |  | 2Tx2Rx (ULA Med.) | 90% | 5.6 dB | 1.22 | - |
|  |  |  | 70% | 3 dB | 1.33 | +0.11 |

From the link simulation results, we have following observation and proposals.

Observation: Both test points, i.e., 70 and 90%, are testable.

Proposal 1:  For FR2 test 4.6 (ULA Low), test requirement shall be 1.2.

Proposal 2:  For FR2 test 4.6 (ULA Med.), test requirement shall be 1.3.

4. Conclusion

This contribution presents our simulation results for following PMI test in FR2 (DDDSU) that was agreed in the last meeting [1]. Observation and proposals are presented as follows.

Observation: Both test points, i.e., 70 and 90%, are testable.

Proposal 1:  For FR2 test 4.6 (ULA Low), test requirement shall be 1.2.

Proposal 2:  For FR2 test 4.6 (ULA Med.), test requirement shall be 1.3.

References

[1] 3GPP, Samsung, R4-1816491 Way forward on NR UE CSI requirements, Nov. 2018.

Appendix

Table A1: FR2 2Tx PMI test cases (test 4.6)

| Parameter | Unit | Test 4.6 FR2 TDD with TDD DL-UL pattern DDDSU |
| --- | --- | --- |
| System parameter |  |  |
| Bandwidth/SCS | MHz/kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 |
| Duplex Mode |  | TDD |
| DLUL configuration |  | DDDSU (FR2.120-1) |
| Propagation channel |  | TDL_A 30ns, 35Hz |
| Correlation and antenna configuration |  | 2*2 Option 1: ULA Low Option 2: ULA Medium |
| CSI-RS resource setting (aperiodic) |  |  |
| resourceConfigType |  | aperiodic |
| NZP CSI-RS ports |  | 2 port {3000,3001} |
| CSI report setting (aperiodic) |  |  |
| reportConfigType |  | aperiodic |
| Reporting interval and slot offset |  | 5/1 |
| (N1,N2) |  | N.A |
| (O1,O2) |  | N.A |
| CodebookMode |  | N.A |
| Codebook Restriction |  | 001111 |
| Reporting granularity |  | Wide-band |
| MCS Table |  | 64QAM |
| Rank |  | 1 |
| FRC |  | MCS13 |
| Physical channel for reporting |  | PUSCH |
| CQI table |  | Table 1 without 256QAM |
| Minimum CSI delay |  | 1.75 ms |