3GPP TSG-RAN4 Meeting #90	R4-1900685

# Athens, Greece, 25th Feb – 1st Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on SRS Carrier Switching Interruption requirements (section 8.2.1.2.8, section 8.2.2.2.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No SRS carrier switching interruption in R4 spec.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new interruption section for SRS carrier switching interruption. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will be no requirements for SRS carrier switching interruption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.8, 8.2.2.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

8.2.1.2.8	Interruptions at SRS carrier based switching

A UE can be configured with SRS resource(s) on a carrier without PUCCH/PUSCH transmission configured. When a UE needs to transmit periodic, semi-persistent or aperiodic SRS on a carrier of a serving cell without PUCCH/PUSCH transmission configured, the UE can perform carrier based switching to one or more carriers without PUCCH/PUSCH transmission configured from a carrier with PUSCH or from another carrier without PUCCH/PUSCH transmission configured prior to transmitting SRS, provided that:

-	switching is from a configured carrier to an active BWP of another activated carrier;

-	the carriers of a serving cell without PUCCH/PUSCH transmission configured to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or indicated by MAC-CE for semi-persistent SRS transmission or configured via RRC [2] for periodic SRS transmission;

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex and srs-SwitchFromCarrier[2];

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS38.214.

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 38.331 [2], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101 [5], the SRS transmission are not simultaneously scheduled with DL SSB transmission on other carriers.

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

The interruption on PCC, PSCC and each of the activated SCCs during the switching to the carrier of a serving cell without PUCCH/PUSCH transmission configured shall not exceed X5 slots include the first slot where SRS transmission is configured on that carrier.

The interruption on PCC, PSCC and each of the activated SCCs during the switching from the carrier of a serving cell without PUCCH/PUSCH transmission configured shall not exceed X5 slots include the last slot where SRS transmission is configured on that carrier.

Table 8.2.1.2.8-1: Interruption length X5 at SRS carrier based switching

|  | Victim cell SCS(KHz) | Aggressor Cell SCS (KHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15 | 30 | 60 | 120 |
| 0 | 15 | 3 | 3 | 3 | 3 |
| 1 | 30 | 4 | 4 | 4 | 4 |
| 2 | 60 | 6 | 6 | 6 | 6 |
| 3 | 120 | 12 | 11 | 10 | 10 |

<< End of Change #1>>

<< Start of Change #2>>

8.2.2.2.6	Interruptions at SRS carrier based switching

A UE can be configured with SRS resource(s) on a carrier without PUCCH/PUSCH transmission configured. When a UE needs to transmit periodic, semi-persistent or aperiodic SRS on a carrier of a serving cell without PUCCH/PUSCH transmission configured, the UE can perform carrier based switching to one or more carriers without PUCCH/PUSCH transmission configured from a carrier with PUSCH or from another carrier without PUCCH/PUSCH transmission configured prior to transmitting SRS, provided that:

-	switching is from a configured carrier to an active BWP of another activated carrier;

-	the carriers of a serving cell without PUCCH/PUSCH transmission configured to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or indicated by MAC-CE for semi-persistent SRS transmission or configured via RRC [2] for periodic SRS transmission;

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex and srs-SwitchFromCarrier[2];

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS38.214.

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 38.331 [2], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101 [5], the SRS transmission are not simultaneously scheduled with DL SSB transmission on other carriers.

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

The interruption of up to the duration on PCC, PSCC and each of the activated SCCs during the switching to the carrier of a serving cell without PUCCH/PUSCH transmission configured is shown in table 8.2.2.2.6-1 including the first slot where SRS transmission is configured on that carrier.

The interruption of up to the duration on PCC, PSCC and each of the activated SCCs during the switching from the carrier of a serving cell without PUCCH/PUSCH transmission configured is shown in table 8.2.2.2.6-1 including the last slot where SRS transmission is configured on that carrier.

Table 8.2.2.2.6-1: Interruption duration at SRS carrier based switching

|  | Victim cell SCS(KHz) | Aggressor Cell SCS (KHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15 | 30 | 60 | 120 |
| 0 | 15 | 3 | 3 | 3 | 3 |
| 1 | 30 | 4 | 4 | 4 | 4 |
| 2 | 60 | 6 | 6 | 6 | 6 |
| 3 | 120 | 12 | 11 | 10 | 10 |

<< End of Change #2>>