3GPP TSG-RAN WG4 Meeting #90	R4-1900699

Athens, Greece, 25 Feb – 1 Mar, 2019

Agenda item:	9.2.2

Source:	Dish Network

Title:	CA_n70A-n71A requirements

Document for:	Approval

# 1	Introduction

It was discussed in RA€4#88bis (October 2018) whether the assumptions and hence resulting requirements used in LTE are still valid for NR [1]. Based on comments from several RFIC and RF Front-end vendors, they are still valid. Thus we are re-using those assumptions to derive requirements for CA_n70A-n71A.

# 2	Discussion

Requirements for LTE CA_70A-71A are shown below for convenience. In LTE, modified (relaxed) REFSENS is used as a requirement for combinations with harmonic relation. In NR, MSD is used instead of modified REFSENS.

Table 6.2.5-2: ΔTIB,c (two bands)

| Inter-band CA Combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| B70+B71 | 70 | 0.3 |
|  | 71 | 0.6 |

Table 7.3.1-1A: ΔRIB,c (two bands)

| Inter-band CA Combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| B70+B71 | 70 | 0 |
|  | 71 | 0 |

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA  (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92 | -87.5 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE X: These exceptions for the intra-band class C should be verified for the 20MHz carrier, according to note 6 frequency arrangements. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0b: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: 3 refers to the UL resource blocks shall be located between 2373-2400MHz. NOTE 4: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a high band. |  |  |  |  |  |  |  |  |

The proposed NR CA requirements are as follows. ΔTIB,c and ΔRIB,c are directly copied from LTE.

Table 1: ΔTIB,c

| Inter-band CA Combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| n70+n71 | 70 | 0.3 |
|  | 71 | 0.6 |

Table 2: ΔRIB,c

| Inter-band CA Combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| n70+n71 | 70 | 0 |
|  | 71 | 0 |

MSD is defined in REFSENS exceptions. Re-using assumptions from LTE the following MSD values are derived. Due to increased spectrum utilization, the NR REFSENS is slightly different than the respective LTE REFSENS. In addition, some rounding was used in LTE REFSENS, which is not used here. Because of slight differences in REFSENS between NR and LTE due to spectrum utilization, the NR MSD numbers also slightly differ from the REFSENS degaration in LTE.

Table 7.3A.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n71 | n701,2 | 9.9 | 7.1 | 6.7 | 4.9 | 4.1 |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

UL configurations are directly copied from LTE for 5MHz, 10MHz, and 15MHz n70 DL BW. For 20MHz and 25MHz, which were not applicable single CC bandwidths for Band 70 in LTE, the maximum applicable n71 UL allocation (20RB) is used.

Table 7.3A.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n71 | n70 | 8 | 16 | 20 | 20 | 20 |  |  |  |  |  |  |  |
| NOTE: 15kHz SCS is assumed for UL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 3	Conclusions

Requirements for CA_n70A-n71A are discussed and the following proposals are made.

Table 1: ΔTIB,c

| Inter-band CA Combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| n70+n71 | 70 | 0.3 |
|  | 71 | 0.6 |

Table 2: ΔRIB,c

| Inter-band CA Combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| n70+n71 | 70 | 0 |
|  | 71 | 0 |

Table 3: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n71 | n701,2 | 9.9 | 7.1 | 6.7 | 4.9 | 4.1 |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n71 | n70 | 8 | 16 | 20 | 20 | 20 |  |  |  |  |  |  |  |
| NOTE: 15kHz SCS is assumed for UL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 4	References

[1] R4-1812489 “Re-evaluation of NR CA H3 MSD assumptions”, Dish Network