3GPP TSG-RAN WG4 Meeting #90	R4-1900700

Athens, Greece, 25 Feb - 1 Mar, 2019

Agenda item:	9.2.2

Source:	Dish Network

Title:	TP for TR38.716-02-00: Requirements for CA_n70A-n71A

Document for:	Approval

# 1	Introduction

This TP captures the requirements for CA_n70A-n71A.

# 2	TP for TR38.716-02-00

## 6.X	CA_n70-n71

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1	Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n66+n70

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n70 | 1695 | – | 1710 | 1995 | – | 2020 | FDD |
| n71 | 663 | – | 698 | 617 | – | 652 | FDD |

#### 6.x.1.2	Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n70+n71

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n70A-n71A | - | n70 | 15 | Yes | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes1 | Yes1 |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3	UE co-existence studies

< Edtor's note: Text will be added, the examples is given as follows. The harmonics and harmonics mixing issues shoule be analysed based on this table. >

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA _ n70-n71.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n70 | 1695 | 1710 | 1995 | 2020 | 3390 | 3420 | 5085 | 5130 |  |  |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 |  |  |

< Edtor's note: Harmonic relation should be captured as far as there is harmonic interference, e.g. n = floor(yy or xx_DL High Band Edge/xx or yy_UL Low Band Edge). >

Based on the table above, the 3rd harmonic of n71 UL lands on top of n70 DL.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n70 | 1695 | 1710 | 1995 | 2020 | 3990 | 4040 | 5985 | 6060 |  |  |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 |  |  |

< Edtor's note: Harmonic mixing relation should be captured as far as there is harmonic mixing interference, e.g. m= floor(yy or xx_UL High Band Edge/xx or yy_DL Low Band Edge). >

Based on the table above, the is no harmonic mixing relation.

#### 6.x.1.4	∆TIB and ∆RIB values

For CA_n70-n71, the TIB,c and RIB,c values are given in the tables below.

Table 6.x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n70-n71 | n70 | 0.3 |
|  | n71 | 0.6 |

Table 6.1.x.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n70-n71 | n70 | 0 |
|  | n71 | 0 |

#### 6.x.1.5	REFSENS requirements

< Editor's note: Text will be added if harmonics and/or harmonic mixing, etc. issues are identified, and only REFSENS numbers for bands have these issues need to be provided in the table. >

Table 6.x.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n71 | n701,2 | 9.9 | 7.1 | 6.7 | 4.9 | 4.1 |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.x.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n71 | n70 | 8 | 16 | 20 | 20 | 20 |  |  |  |  |  |  |  |
| NOTE: 15kHz SCS is assumed for UL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 4	References