3GPP TSG-RAN WG4 Meeting #90	R4-1900701

Athens, Greece, 25 Feb - 1 Mar, 2019

Agenda item:	9.12.1

Source:	Dish Network

Title:	Clarification on n48 bandwidths

Document for:	Approval

# 1	Introduction

n48 WID was approved in Dec 2018 [1]. This contribution recaps the applicable bandwidths for the band.

# 2	Discussion

There was quite a bit of discussion on the applicable maximum UL and DL bandwidths for this band in RAN#82. At the end the following was captured in WID:

------------------------------------------------Excerpt from n48 WID---------------------------------------

The purpose of this work item is to:

- Specify a new NR TDD operating band to include band numbering and core requirements with support of the following SCS and bandwidths:

| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n48 | 15 | Yes | Yes | Yes | Yes | Yes | Yes1 |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes1 | Yes1 | Yes1 | Yes1 | Yes1 |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes1 | Yes1 | Yes1 | Yes1 | Yes1 |
| Note 1: This UE channel bandwidth is applicable only to DL Note 2: Only symmetric UL/DL bandwidths are defined for non-CA operation |  |  |  |  |  |  |  |  |  |  |  |

- These channel bandwidths are to operate in the U.S. by considering:

  - UE transmit: 3550 MHz – 3700 MHz, BS transmit: 3550 MHz – 3700 MHz

- This work-item is for PC3 power class only.

- Intra-band CA is not in the scope of this work-item.

Note: The proposed bandwidths and the corresponding SCS are assuming an availability of wider contiguous bandwidths and are subject to review and revision. The actual available bandwidth allocations are likely to be lower,

Any regulatory updates from the FCC shall be adhered to as part of the specifications associated with this work-item, in terms of performance and compliance requirements

--------------------------------------------------------------------------------------------------------------------------------------

Note 1 states that bandswidths above 40MHz are applicable only to DL and Note 2 states only symmetric bandwidths are defined for non-CA operation. Non-CA operation means single carrier operation in this context.

These all together define the applicable bandwidths for single carrier operation as 5, 10, 15, 20, and 40MHz for both UL and DL.

Observation: Applicable bandwidths for n48 single carrier operation are 5, 10, 15, 20, and 40MHz for both UL and DL

Table 1 Single Carrier bandwidths

| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| n48 | 15 | Yes | Yes | Yes | Yes | Yes |
|  | 30 |  | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes |

# 3	Conclusions

Applicable bandwidths to be defined in n48 WID [1] were discussed with the following observation and proposal.

Observation: Applicable bandwidths for n48 single carrier operation are 5, 10, 15, 20, and 40MHz for both UL and DL

Proposal: Single Carrier bandwidths for n48 are defined in the following table:

Table 1 Single Carrier bandwidths

| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| n48 | 15 | Yes | Yes | Yes | Yes | Yes |
|  | 30 |  | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes |

# 4	References

# [1] RP-182865 “New WID on introduction of n48”, US Cellular