3GPP TSG-RAN WG4 Meeting #90                     	R4-1900868

Athens, Greece, Feb 25th – Mar 1th, 2019

Agenda item:            6.14.1.2.2

Source:	MediaTek Inc.

Title:	NR PDSCH SDR simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation results of PDSCH SDR.

# 2	Discussion

Based on the simulation assumption in [1] and [2], we provide the simulation result of PDSCH SDR test cases. Table A.1 (in Appendix) is the test case list and Table A.2~Table A.5 contain detailed parameter setting in the simulation.

Table 2.1 is the 85% Tput SNR for different MCS in FR1 FDD setting (Case 1~4)

Table 2.1 85% Tput SNR for FR1 FDD

| MCS | Case 1 (Rank2, 64QAM) (dB) | Case 2 (Rank2, 256QAM) (dB) | Case 3 (Rank4, 64QAM) (dB) | Case 4 (Rank4, 256QAM) (dB) |
| --- | --- | --- | --- | --- |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |
| 16 |  |  |  |  |
| 17 |  |  |  |  |
| 18 |  |  |  |  |
| 19 |  |  |  |  |
| 20 |  |  |  |  |
| 21 |  |  |  |  |
| 22 |  |  |  |  |
| 23 |  |  |  |  |
| 24 |  |  |  |  |
| 25 |  |  |  |  |
| 26 |  |  |  |  |

Table 2.2 is the 85% Tput SNR for different MCS in FR1 TDD setting (Case 5~8)

Table 2.2 85% Tput SNR for FR2 TDD

| MCS | Case 5 (Rank2, 64QAM) (dB) | Case 6 (Rank2, 256QAM) (dB) | Case 7 (Rank4, 64QAM) (dB) | Case 8 (Rank4, 256QAM) (dB) |
| --- | --- | --- | --- | --- |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |
| 16 |  |  |  |  |
| 17 |  |  |  |  |
| 18 |  |  |  |  |
| 19 |  |  |  |  |
| 20 |  |  |  |  |
| 21 |  |  |  |  |
| 22 |  |  |  |  |
| 23 |  |  |  |  |
| 24 |  |  |  |  |
| 25 |  |  |  |  |
| 26 |  |  |  |  |

Table 2.3 is the 85% Tput SNR for different MCS in FR2 TDD setting (Case 9~10)

Table 2.3 85% Tput SNR for FR2 TDD

| MCS | Case 9 (60kHz SCS) (dB) | Test 10 (Case SCS) (dB) |
| --- | --- | --- |
| 13 | 8.8224 | 8.9234 |
| 14 | 9.8137 | 9.8064 |
| 15 | 10.8459 | 10.9027 |
| 16 | 11.7782 | 11.7327 |
| 17 | 12.6403 | 12.5604 |
| 18 | 12.9056 | 12.8720 |
| 19 | 13.9640 | 13.8552 |
| 20 | 15.4056 | 15.3091 |
| 21 | 16.3696 | 16.1680 |
| 22 | 17.7724 | 17.6413 |
| 23 | 19.6687 | 19.3129 |
| 24 | 21.5479 | 20.9878 |
| 25 | 25.2734 | 23.7703 |
| 26 |  | 26.2661 |

Table 2.4 is the testable MSC

Table 2.4 Testable MCS for PDSCH SDR tests

| Test case | FR | SCS | Duplex mode | CBW | MIMO config. | Max rank | Max modulation order | Testable MCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | FR1 | 15kHz | FDD | 20MHz | 2x2MIMO | Rank 2 | 64QAM |  |
| Case 2 | FR1 | 15kHz | FDD | 20MHz | 2x2MIMO | Rank 2 | 256QAM |  |
| Case 3 | FR1 | 15kHz | FDD | 20MHz | 4x4MIMO | Rank 4 | 64QAM |  |
| Case 4 | FR1 | 15kHz | FDD | 20MHz | 4x4MIMO | Rank 4 | 256QAM |  |
| Case 5 | FR1 | 30kHz | TDD | 100MHz | 2x2MIMO | Rank 2 | 64QAM |  |
| Case 6 | FR1 | 30kHz | TDD | 100MHz | 2x2MIMO | Rank 2 | 256QAM |  |
| Case 7 | FR1 | 30kHz | TDD | 100MHz | 4x4MIMO | Rank 4 | 64QAM |  |
| Case 8 | FR1 | 30kHz | TDD | 100MHz | 4x4MIMO | Rank 4 | 256QAM |  |
| Case 9 | FR2 | 60kHz | TDD | 100MHz | 2x2MIMO | Rank2 | 64QAM |  |
| Case 10 | FR2 | 120kHz | TDD | 100MHz | 2x2MIMO | Rank2 | 64QAM |  |

# 3	Conclusions

In the contribution, we provide our simulation results of PDSCH SDR tests.

# 4	Reference

# [1] R4-1813929, “WF on SDR requirement for NR”, NTT DOCOMO.

# [2] R4-1816489, “WF on SDR requirement”, NTT DOCOMO.

Appendix

Table A.1 PDSCH SDR test case list

| Test case | FR | SCS | Duplex mode | CBW | MIMO config. | Max rank | Max modulation order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | FR1 | 15kHz | FDD | 20MHz | 2x2MIMO | Rank 2 | 64QAM |
| Case 2 | FR1 | 15kHz | FDD | 20MHz | 2x2MIMO | Rank 2 | 256QAM |
| Case 3 | FR1 | 15kHz | FDD | 20MHz | 4x4MIMO | Rank 4 | 64QAM |
| Case 4 | FR1 | 15kHz | FDD | 20MHz | 4x4MIMO | Rank 4 | 256QAM |
| Case 5 | FR1 | 30kHz | TDD | 100MHz | 2x2MIMO | Rank 2 | 64QAM |
| Case 6 | FR1 | 30kHz | TDD | 100MHz | 2x2MIMO | Rank 2 | 256QAM |
| Case 7 | FR1 | 30kHz | TDD | 100MHz | 4x4MIMO | Rank 4 | 64QAM |
| Case 8 | FR1 | 30kHz | TDD | 100MHz | 4x4MIMO | Rank 4 | 256QAM |
| Case 9 | FR2 | 60kHz | TDD | 100MHz | 2x2MIMO | Rank2 | 64QAM |
| Case 10 | FR2 | 120kHz | TDD | 100MHz | 2x2MIMO | Rank2 | 64QAM |

Table A.2 PDSCH SDR test configuration

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH transmission scheme |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  | dB | N/A |
| Channel bandwidth |  | MHz | See slide #6 |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | First SSB in Slot #0 |
|  | SSB periodicity | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  | 2 |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | Full allocation |
|  | Subcarrier spacing | kHz | See slide #6 |
|  | Cyclic prefix |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH |  | Symbols #0 |
|  | Number of PRBs in CORESET |  | [TBD] |
|  | Number of PDCCH candidates and aggregation levels |  | [TBD] |
|  | DCI format |  | [TBD] |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | Wideband for FR1 [TBD] for FR2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | [TBD] |
|  | VRB-to-PRB mapping interleaver bundle size |  | [TBD] |

Table A.3 PDSCH SDR test configuration

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
|  | Antenna ports indexes |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 - 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuratioaan |  |  | For FR1: PTRS is not configured For FR2: 1 port, per 2PRB in frequency domain, per symbol in time domain |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 6 |
|  | Number of CSI-RS ports (X) |  | 1 |
|  | CDM Type |  | No |
|  | Density (ρ) |  | 3 |
|  | CSI-RS periodicity |  | 20ms |
|  | CSI-RS offset |  | 10ms from SSB |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | Number of CSI-RS ports (X) |  | Same as Tx ports |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | CSI-RS periodicity |  | 20ms |
|  | CSI-RS offset |  | 0ms from SSB |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| Precoding configuration |  |  | SP Type I, Random per slot with PRB bundling granularity |
| Symbols for all unused REs |  |  | OCNG Annex A.5 |
| Propagation condition |  |  | Static propagation condition for FR1 [TBD] for FR2 |

Table A.4 PDSCH SDR test configuration for FDD

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 4 for simulation purpose |
| K1 value |  |  | 2 |

Table A.5 PDSCH SDR test configuration for TDD

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 8 for simulation purpose |
| K1 value |  |  | TBD |
| TDD UL-DL pattern |  |  | FR1 15kHz: DDDSU FR1 30KHz: 7D1S2U FR2 60 kHz: DDSU FR2 120 kHz: DDDSU for simulation purpose |