3GPP TSG-RAN WG4 RAN4-90	R4-1900975

# Athens, Greece, 25 February - 1 March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR | - | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.113: Exclusion Bands for Radiated Immunity Test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | We propose to extend the exclusion bands for Rx immunity test when spatial exclusion is not considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of exclusion band sizes for BS type 1-O, considering the operating bandwidth, and the possibility of regional requirements not accepting spatial exclusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the value is not defined it won’t be possible to calculate the exclusion bands for Radiated Immunity test. On the other hand, it is important to handle potential limitations coming from regional requirements denying the use of spatial exclusion concept. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Start of Text ------------------------------

### 4.4.2	Receiver exclusion band

The receiver exclusion band for BS is the frequency range over which no tests of radiated immunity of a receiver are made.

The receiver exclusion band is defined as:

FUL_low  – ΔfOOB<f < FUL_high + ΔfOOB

Where:

Values of FUL_low and FUL_high are defined for each operating band in TS 38.104 [2], subclause 5.2.

The value of ΔfOOB is derived considering the width of the operating band, and is defined in TS 38.104 [2], subclause 7.4.2.2. For immunity testing of BS Type 1-O,  ΔfOOB  shall be replaced by 60 MHz (when the width of the operating band is < 100 MHz) or 200 MHz (when width of operating band is ≥ 100 MHz).

For BS capable of multi-band operation, the total receiver exclusion band is a combination of the exclusion bands for each operating band supported by BS.

NOTE 1:	The receiver exclusion bands do not apply for SDL bands.

- NOTE 2:	As the radiated immunity testing is defined in the frequency range 80 MHz to 6 GHz, there is no receiver exclusion band defined for BS type 2-O.

------------------------------ End of proposed text ------------------------------