3GPP TSG-RAN4 WG4 Meeting #90	R4-1901058

# Athens, Greece,  Feb.25 - Mar.01, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5363 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Addition of performance requirements for FeNB-IoT TDD NPDCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 2019-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per agreements R4-1814232, RAN4 agreed to define FeNB-IoT TDD NPDCCH performance requirements with UL/DL configuration#4 and special subframe configuration#4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the performance requirements for FeNB-IoT TDD NPDCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for FeNB-IoT TDD NPDCCH will still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.2.1, 8.12.2.2(New), A.3.13.2(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

8.12.2	NPDCCH

The receiver characteristics of the NPDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

8.12.2.1	Half-duplex FDD

The parameters specified in Table 8.12.2.1-1 and Table 8.12.2.1-2 are valid for all half-duplex FDD tests unless otherwise stated.

Table 8.12.2.1-1: Test Parameters for NPDCCH

| Parameter | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- |
| Narrowband physical layer Cell ID |  | 0 | 0 |
| at antenna port | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  | Normal | Normal |
| Number of CRS ports for in-band deployment mode |  | 4 | 4 |
| NPDCCH starting position (eutraControlRegionSize-r13) (Note 1) |  | 3 | 3 |
| Maximum number of repetitions (npdcch-NumRepetitions-r13) |  | 128 for Test 1; 1024 for Test 2. | 64 for Test 1; 512 for Test 2. |
| NPDCCH start subframe (npdcch-startSF-USS-r13) |  | 2 for test 1, 1.5 for test 2 | 2 for test 1, 1.5 for test 2 |
| NPDCCH fractional period offset of starting subframe (npdcch-Offset-USS-r13) |  | 0 | 0 |
| NB-IoT downlink subframe bitmap for anchor carrier (downlinkBitmap-r13) |  | Not configured | Not configured |
| NB-IoT downlink subframe bitmap for non-anchor carrier (downlinkBitmapNonAnchor-r13) |  | Not configured | Not configured |
| Downlink gap configuration for anchor carrier (dl-Gap-r13) |  | Not configured | Not configured |
| Downlink gap configuration for non-anchor carrier (dl-GapNonAnchor-r13) |  | Not configured | Not configured |
| Unused REs or RBs (Note 1) |  | OCNG | OCNG |
| OCNG pattern |  | NB.OP.1 | NB.OP.1 |
| NOTE 1: Applicable only for in-band deployment mode. |  |  |  |

Table 8.12.2.1-2: Test Parameters of related NPDSCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Scheduling delay field () |  | 0 |
| NPDSCH Repetition number |  | 1 |
| at antenna port for NPDSCH | dBm/15kHz | -98 |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDSCH |  | R.NB.6 FDD and R.NB.6-1 FDD for one NRS antenna port; R.NB.5 FDD and R.NB.5-1 FDD for two NRS antenna ports |

8.12.2.1.1	Single-antenna performance

For the parameters specified in Table 8.12.2.1.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.1.1-1. The downlink physical channel setup is in accordance with Annex C.3.6.

Table 8.12.2.1.1-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | Stand-alone/Guard-band | 128 | Anchor | R.NB.3 FDD | EPA5 | 1 | 1 | -4.9 | NB1, NB2 |
| 2 | Stand-alone/Guard-band | 1024 | Non-anchor | R.NB.3 FDD | ETU1 | 1 | 1 | -11.4 | NB1, NB2 |

8.12.2.1.2	Transmit diversity performance

For the parameters specified in Table 8.12.2.1.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.1.2-1. The downlink physical channel setup is in accordance with Annex C.3.6.

Table 8.12.2.1.2-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | In-band | 64 | Anchor | R.NB.4 FDD | EPA5 | 2 | 1 | -3.9 | NB1, NB2 |
| 2 | In-band | 512 | Non-anchor | R.NB.4 FDD | ETU1 | 2 | 1 | -10.0 | NB1, NB2 |

8.12.2.2	TDD

The parameters specified in Table 8.12.2.2-1 and Table 8.12.2.2-2 are valid for all TDD tests unless otherwise stated.

Table 8.12.2.2-1: Test Parameters for NPDCCH

| Parameter | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 4 | 4 |
| Special subframe configuration (Note 2) |  | 4 | 4 |
| Narrowband physical layer Cell ID |  | 0 | 0 |
| at antenna port | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  | Normal | Normal |
| Number of CRS ports for in-band deployment mode |  | 4 | 4 |
| NPDCCH starting position (eutraControlRegionSize-r13) (Note 1) |  | 2 | 2 |
| Maximum number of repetitions (npdcch-NumRepetitions-r13) |  | 128 for Test 1; 1024 for Test 2. | 64 for Test 1; 512 for Test 2. |
| NPDCCH start subframe (npdcch-startSF-USS-r13) |  | 4 | 4 |
| NPDCCH fractional period offset of starting subframe (npdcch-Offset-USS-r13) |  | 0 | 0 |
| NB-IoT downlink subframe bitmap for anchor carrier (downlinkBitmap-r13) |  | Not configured | Not configured |
| NB-IoT downlink subframe bitmap for non-anchor carrier (downlinkBitmapNonAnchor-r13) |  | Not configured | Not configured |
| Downlink gap configuration for anchor carrier (dl-Gap-r13) |  | Not configured | Not configured |
| Downlink gap configuration for non-anchor carrier (dl-GapNonAnchor-r13) |  | Not configured | Not configured |
| Unused REs or RBs (Note 1) |  | OCNG | OCNG |
| OCNG pattern |  | NB.OP.1 | NB.OP.1 |
| NOTE 1: Applicable only for in-band deployment mode. |  |  |  |

Table 8.12.2.2-2: Test Parameters of related NPDSCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Scheduling delay field () |  | 0 |
| NPDSCH Repetition number |  | 1 |
| at antenna port for NPDSCH | dBm/15kHz | -98 |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDSCH |  | R.NB.6 TDD and R.NB.6-1 TDD for one NRS antenna port; R.NB.5 TDD and R.NB.5-1 TDD for two NRS antenna ports |

8.12.2.2.1	Single-antenna performance

For the parameters specified in Table 8.12.2.2.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.2.1-1. The downlink physical channel setup is in accordance with Annex C.3.6.

Table 8.12.2.2.1-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | Stand-alone/Guard-band | 128 | Anchor | R.NB.3 TDD | EPA5 | 1 | 1 | [-5.7] | NB1, NB2 |
| 2 | Stand-alone/Guard-band | 1024 | Non-anchor | R.NB.3 TDD | ETU1 | 1 | 1 | [-12.2] | NB1, NB2 |

8.12.2.2.2	Transmit diversity performance

For the parameters specified in Table 8.12.2.2.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.2.2-1. The downlink physical channel setup is in accordance with Annex C.3.6.

Table 8.12.2.2.2-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | In-band | 64 | Anchor | R.NB.4 TDD | EPA5 | 2 | 1 | [-4.7] | NB1, NB2 |
| 2 | In-band | 512 | Non-anchor | R.NB.4 TDD | ETU1 | 2 | 1 | [-10.8] | NB1, NB2 |

<Unchanged Sections Skipped>

A.3.13	Reference measurement channels for NPDCCH performance requirements

A.3.13.1	Half-duplex FDD

Table A.3.13.1-1: NPDCCH Reference Channel for Category NB1 UE

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.3 FDD | R.NB.4 FDD |
| Number of NRS ports |  | 1 | 2 |
| Channel bandwidth | MHz | 0.2 | 0.2 |
| Aggregation level | NCCE | 2 | 2 |
| DCI Format |  | N1 | N1 |
| Payload (without CRC) | Bits | 23 | 23 |

A.3.13.2	TDD

Table A.3.13.2-1: NPDCCH Reference Channel for Category NB1 UE

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.3 TDD | R.NB.4 TDD |
| Number of NRS ports |  | 1 | 2 |
| Channel bandwidth | MHz | 0.2 | 0.2 |
| Aggregation level | NCCE | 2 | 2 |
| DCI Format |  | N1 | N1 |
| Payload (without CRC) | Bits | 23 | 23 |