3GPP TSG-RAN4 Meeting #90	R4-1901121

# Athens, Greece, 25th Feb – 1st Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on the release with redirection to NR (section 6.2.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no requirement for the cases where the target SMTC configuration is unknown to the UE. The requirments were derived based on the side condition of SNR > -4dB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The RRC connection release with redirection to NR requirement for the cases where the target SMTC configuration is unknown to the UE is added. Side conditions are clarified that the SNR > -4dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

### 6.2.3	SA: RRC Connection Release with Redirection

#### 6.2.3.1	Introduction

This clause contains requirements on the UE regarding RRC connection release with redirection procedure. RRC connection release with redirection is initiated by the RRCConnectionRelease message with redirection to E-UTRAN or NR from NR specified in TS 38.331 [2]. The RRC connection release with redirection procedure is specified in clause 5.3.8 of TS 38.331 [2].

#### 6.2.3.2	Requirements

##### 6.2.3.2.1	RRC connection release with redirection to NR

The UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR.

The time delay (Tconnection_release_redirect_NR) is the time between the end of the last slot containing the RRC command, “RRCRelease” (TS 38.331 [2]) on the NR PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR) shall be less than:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH

The target NR cell shall be considered detetable when for each relevant SSB, the side conditions should be met that,

- the target SSB Ês/Iot  -4dB for FR1 target NR cell, and

- the target SSB Ês/Iot  [TBD] at the reference point for FR2 target NR cell.

TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCRelease” as defined in clause 6.2.2 of TS 38.331 [2].

Tidentify-NR: It is the time to identify the target NR cell and depend on the frequency range (FR) of the target NR cell. It is defined in table 6.2.3.2.1-1. Note that Tidentify-NR = TPSS/SSS-sync + Tmeas, in which TPSS/SSS-sync is the cell search time and Tmeas is the measurement time due to cell selection criteria evaluation.

TSI-NR: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released. TSI-NR = 0 provided the UE is provided with the SI (including MIB and all relevant SIBs) of the target NR cell before the RRC connection is released by the old NR cell.

TRACH: It is the delay caused due to the random access procedure when sending random access to the target NR cell. This delay depends on the PRACH configuration defined in Table 6.3.3.2-2 [6] or Table 6.3.3.2-3 [6] for FR1 and in Table 6.3.3.2-4 [6] for FR2.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided with SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs = 20 ms assuming the SSB transmission periodicity is no larger than 20 ms. There is no requirement if the SSB transmission periodicity is larger than 20ms. If UE is provided with both SMTC configuration and SSB transmission periodicity, the requirement shall be based on SMTC periodicity.

Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| Frequency range (FR) of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, [11] x Trs) |
| FR2 | MAX (880 ms, 8x[11] x Trs) |
| Note : If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the redirection command, SMTC follows smtc1 or smtc2 according to the physical cell ID of the target cell. |  |

<End of Changes>