3GPP TSG-RAN4 Meeting #90	R4-1901176

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of measurement requirements side condition for FR2 (section 10.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | All side conditions for FR2 measurement requirements are TBD in spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add exact values for Es/Iot and minim Io side conditions for FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Side condition for FR2 is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 10.1.3	Intra-frequency RSRP accuracy requirements for FR2

#### 10.1.3.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.3.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.3.1.1-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.3.1.1-1 are defined with respective to the the input of UE antenna array.

Table 10.1.3.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [6] | [9] | ≥-5 dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | -70 |
|  |  |  | NR_TDD_FR2_B | -111 | -108 | N/A | -70 |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | -70 |
|  |  |  | NR_TDD_FR2_G | -106.5 | -103.5 | N/A | -70 |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | -70 |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | -70 |
| [8] | [11] | ≥-5 dB | TNR_TDD_FR2_A, NR_TDD_FR2_B, NR_TDD_FR2_F, NR_TDD_FR2_G, NR_TDD_FR2_T, NR_TDD_FR2_Y | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

##### 10.1.3.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS RSRP levels measured on the same cell in FR2.

The accuracy requirements in Table 10.1.3.1.2-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.3.1.2-1 are defined with respective to the input of UE antenna array.

Table 10.1.3.1.2-1: SS-RSRP Intra frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [6] | [9] | ≥-2 dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | TBD |
|  |  |  | NR_TDD_FR2_B | -111 | -108 | N/A | TBD |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | TBD |
|  |  |  | NR_TDD_FR2_G | -106.5 | -103.5 | N/A | TBD |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | TBD |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | TBD |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

#### 10.1.3.2	Intra-frequency [CSI-RS RSRP] accuracy requirements

<End of Change 1>

<Start of Change 2>

### 10.1.5	Inter-frequency RSRP accuracy requirementsfor FR2

#### 10.1.5.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1.5.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.5.1.1-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.5.1.1-1 are defined with respective to the input of UE antenna array.

Table 10.1.5.1.1-1: SS-RSRP Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [6] | [9] | ≥-5 dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | -70 |
|  |  |  | NR_TDD_FR2_B | -111 | -108 | N/A | -70 |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | -70 |
|  |  |  | NR_TDD_FR2_G | -106.5 | -103.5 | N/A | -70 |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | -70 |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | -70 |
| [8] | [11] | ≥-5 dB | TNR_TDD_FR2_A, NR_TDD_FR2_B, NR_TDD_FR2_F, NR_TDD_FR2_G, NR_TDD_FR2_T, NR_TDD_FR2_Y | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

##### 10.1.5.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell on a frequency in FR2 compared to the SS-RSRP measured from another cell on another frequency in FR2.

The accuracy requirements in Table 10.1.5.1.2-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.5.1.2-1 are defined with respective to the input of UE antenna array.

Table 10.1.5.1.2-1: SS-RSRP Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| [6] | [9] | ≥-5 dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | TBD |
|  |  |  | NR_TDD_FR2_B | -111 | -108 | N/A | TBD |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | TBD |
|  |  |  | NR_TDD_FR2_G | -106.5 | -103.5 | N/A | TBD |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | TBD |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | TBD |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

#### 10.1.5.2	Inter-frequency [CSI-RS RSRP] accuracy requirements

<End of Change 2>

<Start of Change 3>

### 10.1.8	Intra-frequency RSRQ accuracy requirements for FR2

#### 10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2

##### 10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.8.1.1-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.8.1.1-1 are defined with respective to the input of UE antenna array.

Table 10.1.8.1.1-1: SS-RSRQ Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| [2.5] | [4] | [-2] dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_B |  | -111 |  | -108 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_G |  | -106.5 |  | -103.5 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | -50 |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | -50 |
| [3.5] | [4] | [-5] dB | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

<End of Change 3>

<Start of Change 4>

### 10.1.10	Inter-frequency RSRQ accuracy requirements for FR2

#### 10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2

##### 10.1.10.1.1	Aboslute Accuracy of SS-RSRQ in FR2

The requirements for absolute accuracy of SS-RSRQ in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.10.1.1-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.10.1.1-1 are defined with respective to the input of UE antenna array.

Table 10.1.10.1.1-1: SS-RSRQ Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| [2.5] | [4] | [-2] dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_B |  | -111 |  | -108 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_G |  | -106.5 |  | -103.5 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | -50 |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | -50 |
| [3.5] | [4] | [-5] dB | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

##### 10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2

The relative accuracy of SS-RSRQ in inter frequency case is defined as the RSRQ measured from one cell on a frequency in FR2 compared to the RSRP measured from another cell on a different frequency in FR2.

The accuracy requirements in Table 10.1.10.1.2-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.10.1.2-1 are defined with respective to the input of UE antenna array.

Table 10.1.10.1.2-1: SS-RSRQ Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| [3] | [4] | [-2] dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_B |  | -111 |  | -108 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_G |  | -106.5 |  | -103.5 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | -50 |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | -50 |
| [4] | [4] | [-5] dB | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

<End of Change 4>

<Start of Change 5>

### 10.1.13	Intra-frequency SINR accuracy requirements for FR2

#### 10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2

##### 10.1.13.1.1	Absolute SS-SINR Accuracy in FR2

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.13.1.1-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band.

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.13.1.1-1 are defined with respective to the input of UE antenna array.

Table 10.1.13.1.1-1: SS-SINR Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| [3.0] | [4] | [-2] dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_B |  | -111 |  | -108 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_G |  | -106.5 |  | -103.5 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | -50 |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | -50 |
| [3.5] | [4] | [-5] dB | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ [25] dB. NOTE 4: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

<End of Change 5>

<Start of Change 6>

### 10.1.15	Inter-frequency SINR accuracy requirements for FR2

#### 10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2

##### 10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2

The requirements for absolute accuracy of SS-SINR in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 10.1.15.1.1-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.15.1.1-1 are defined with respective to the input of UE antenna array.

Table 10.1.15.1.1-1: SS-SINR Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| [3.0] | [4] | [-2] dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_B |  | -111 |  | -108 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_G |  | -106.5 |  | -103.5 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | -50 |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | -50 |
| [3.5] | [4] | [-5] dB | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ [25] dB. NOTE 4: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

##### 10.1.15.1.2	Relative Accuracy of SS-SINR in FR2

The relative accuracy of SS-SINR in inter frequency case is defined as the SS-SINR measured from one cell on a frequency in FR2 compared to the SS-SINR measured from another cell on a different frequency in FR2.

The accuracy requirements in Table 10.1.15.1.2-1 are valid under the following conditions:

-	Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

-

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	Other conditions are TBD.

-	The measured signals are in the directions covered by 50% and 20% percentile EIS spherical coverage of the UE, for UE power class 1, 2, 3 and UE power class 4, respectively.

The Ês/Iot and Io conditions in Table 10.1.15.1.2-1 are defined with respective to the input of UE antenna array.

Table 10.1.15.1.2-1: SS-SINR Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2, Note 4 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| [3.5] | [4] | [-3] dB | NR_TDD_FR2_A | -112.5 | -109.5 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_B |  | -111 |  | -108 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_F | -106 | -103 | N/A | -50 |
|  |  |  |  |  |  | NR_TDD_FR2_G |  | -106.5 |  | -103.5 |  | N/A |  | -50 |  |
|  |  |  | NR_TDD_FR2_T | -100.5 | -97.5 | N/A | -50 |
|  |  |  | NR_TDD_FR2_Y | -96 | -93 | N/A | -50 |
| [4] | [4] | [-6] dB | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ [25] dB. NOTE 5: NR operating band groups in FR2 are as defined in Section 3.5.3. |  |  |  |  |  |  |  |

<End of Change 6>