3GPP TSG-RAN4 Meeting #90	R4-1901183

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance of SA SSB based RLM test (section A.6.5.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some issues with the SA SSB based RLM test cases: - For in-sync tests with DRX, the duration of T5 is smaller than D1, which means the test is terminated before the verdict can be made - For all tests, the duration of T2 is smaller than the evaluation period, so it cannot verify if UE has wrongly triggered OOS at SNR2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the duration of T5 for in-sync test with DRX, and T2 for all tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test setup for SA SSB based RLM is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.5.1	Radio link Monitoring

In the following section, any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

-	UE output power higher than Transmit OFF power [-50] dBm (as defined in TS 38.101-1 [TBD]) means uplink signal

-	UE output power equal to or less than Transmit OFF power [-50] dBm (as defined in TS 38.101-1 [TBD]) means no uplink signal.

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

A.6.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

Supported test configurations are shown in table A.6.5.1.1.1-1. The test parameters are given in Tables A.6.5.1.1.1-2, A.6.5.1.1.1-3, and A.6.5.1.1.1-4 below. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Editor note: whether to revise power level to be gradually changed

Editor note: whether to revise the SSB configuration to be 2 SSBs and FFS the corresponding power level

Table A.6.5.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 2 | TDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 3 | TDD, SSB SCS 30 KHz, data SCS 30KHz, BW 40MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.1.1-2: General test parameters for FR1 out-of-sync testing in non-DRX mode

| Parameter |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 | Test 2 |  |
| Active PCell |  |  |  |  | Cell 1 | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 | 1 |  |
| Duplex mode |  |  | Config 1 |  | FDD | FDD |  |
|  |  |  | Config 2, 3 |  | TDD | TDD |  |
| TDD Configuration |  |  | Config 1 |  | Not Applicable | Not Applicable |  |
|  |  |  | Config 2 |  | [TDDConf.1.1] | [TDDConf.1.1] |  |
|  |  |  | Config 3 |  | [TDDConf.1.2] | [TDDConf.1.2] |  |
| CORESET Reference Channel |  |  | Config 1 |  | [CR. 1.1 FDD] | [CR. 1.1 FDD] |  |
|  |  |  | Config 2 |  | [CR. 1.1 TDD] | [CR. 1.1 TDD] |  |
|  |  |  | Config 3 |  | [CR. 2.1 TDD] | [CR. 2.1 TDD] |  |
| SSB Configuration |  |  | Config 1 |  | Table A.3.2.2.1.1-1 | Table A.3.2.2.1.1-1 |  |
|  |  |  | Config 2 |  | Table A.3.2.2.1.1-1 | Table A.3.2.2.1.1-1 |  |
|  |  |  | Config 3 |  | Table A.3.2.2.1.2-1 | Table A.3.2.2.1.2-1 |  |
| SMTC Configuration |  |  | Config 1, 2 |  | Table A.3.2.3.1-1 | Table A.3.2.3.1-1 |  |
|  |  |  | Config 3 |  | Table A.3.2.3.1-1 | Table A.3.2.3.1-1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz | 15 KHz |  |
|  |  |  | Config 3 |  | 30 KHz | 30 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  | TBD | TBD |  |
|  |  |  | Config 3 |  | TBD | TBD |  |
| SSB index assigned as RLM RS |  |  |  |  | [0] | [0] |  |
| OCNG parameters |  |  |  |  | Table A.3.2.1.1-1 | Table A.3.2.1.1-1 |  |
| CP length |  |  |  |  | Normal | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | [2x2 Low] | [2x2 Low] |  |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 | 2 |  |
|  | Aggregation level |  |  | CCE | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 | 4 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 | 4 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 | 6 |  |
| DRX |  |  |  |  | OFF | OFF |  |
| Gap pattern ID |  |  |  |  | [N.A.] | *[gp0] |  |
| Layer 3 filtering |  |  |  |  | Enabled | Enabled |  |
| T310 timer |  |  |  | ms | 0 | 0 |  |
| T311 timer |  |  |  | ms | 1000 | 1000 |  |
| N310 |  |  |  |  | 1 | 1 |  |
| N311 |  |  |  |  | 1 | 1 |  |
| NZP CSI-RS configuration |  |  |  |  | TBD | TBD |  |
| ZP CSI-RS configuation |  |  |  |  | TBD | TBD |  |
| CSI-IM configuration |  |  |  |  | TBD | TBD |  |
| Periodic CSI reporting |  |  |  |  | PUCCH | PUCCH |  |
| CSI reporting periodicity |  | Config 1, 2 |  | slot | [5] |  | [5] |
|  |  | Config 3 |  |  | [10] |  | [10] |
| T1 |  |  |  | s | 1 | 1 |  |
| T2 |  |  |  | s | 0.4 | 0.6 |  |
| T3 |  |  |  | s | [0.6] | [0.6] |  |
| D1 |  |  |  | s | [0.24] | [0.44] |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table A.6.5.1.1.1-3: Cell specific test parameters for FR1 (Cell 1) for out-of-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta |  | dB | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  | 4 |  |  |
| PBCH_beta |  | dB | 0 |  |  | 0 |  |  |
| PSS_beta |  | dB |  |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |  |
| SNR | Config 1 | dB | [1] | [-7] | [-15] | [1] | [-7] | [-15] |
|  | Config 2 |  | [1]TBD | [-7] | [-15] | [1] | [-7] | [-15] |
|  | Config 3 |  | [1]TBD | [-7] | [-15] | [1] | [-7] | [-15] |
|  | Config 1 | dBm/15KHz | [-98] |  |  | [-98] |  |  |
|  | Config 2 |  | [-98] |  |  | [-98] |  |  |
|  | Config 3 |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.6.5.1.1.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |  |  |  |

Table A.6.5.1.1.1-4: Measurement gap configuration for out-of-sync tests in non-DRX mode

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [0] |
| Note: Ensure that RLM RS is partially overlapped with measurement gap |  |

Figure A.6.5.1.1.1-1: SNR variation for out-of-sync testing

A.6.5.1.1.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

A.6.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

Supported test configurations are shown in table A.6.5.1.2.1-1. The test parameters are given in Tables A.6.5.1.2.1-2, A.6.5.1.2.1-3, and A.6.5.1.2.1-4 below. There is one cell (Cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Editor note: whether to revise power level to be gradually changed

Editor note: whether to revise the SSB configuration to be 2 SSBs and FFS the corresponding power level

Table A.6.5.1.2.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 2 | TDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 3 | TDD, SSB SCS 30 KHz, data SCS 30KHz, BW 40MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.2.1-2: General test parameters for FR1 in-sync testing in non-DRX mode

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 | Test 2 |
| Active PCell |  |  |  |  | Cell 1 | Cell 1 |
| RF Channel Number |  |  |  |  | 1 | 1 |
| Duplex mode |  |  | Config 1 |  | FDD | FDD |
|  |  |  | Config 2, 3 |  | TDD | TDD |
| TDD Configuration |  |  | Config 1 |  | Not Applicable | Not Applicable |
|  |  |  | Config 2 |  | [TDDConf.1.1] | [TDDConf.1.1] |
|  |  |  | Config 3 |  | [TDDConf.1.2] | [TDDConf.1.2] |
| CORESET Reference Channel |  |  | Config 1 |  | [CR. 1.1 FDD] | [CR. 1.1 FDD] |
|  |  |  | Config 2 |  | [CR. 1.1 TDD] | [CR. 1.1 TDD] |
|  |  |  | Config 3 |  | [CR. 2.1 TDD] | [CR. 2.1 TDD] |
| SSB Configuration |  |  | Config 1 |  | Table A.3.2.2.1.1-1 | Table A.3.2.2.1.1-1 |
|  |  |  | Config 2 |  | Table A.3.2.2.1.1-1 | Table A.3.2.2.1.1-1 |
|  |  |  | Config 3 |  | Table A.3.2.2.1.2-1 | Table A.3.2.2.1.2-1 |
| SMTC Configuration |  |  | Config 1, 2 |  | Table A.3.2.3.1-1 | Table A.3.2.3.1-1 |
|  |  |  | Config 3 |  | Table A.3.2.3.1-1 | Table A.3.2.3.1-1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz | 15 KHz |
|  |  |  | Config 3 |  | 30 KHz | 30 KHz |
| PRACH Configuration |  |  | Config 1, 2 |  | TBD | TBD |
|  |  |  | Config 3 |  | TBD | TBD |
| SSB index assigned as RLM RS |  |  |  |  | [0] | [0] |
| OCNG parameters |  |  |  |  | Table A.3.2.1.1-1 | Table A.3.2.1.1-1 |
| CP length |  |  |  |  | Normal | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters | DCI format |  |  |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 | 2 |
|  | Aggregation level |  |  | CCE | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size | REG bundle size |
|  | REG bundle size |  |  |  | 6 | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 | 2 |
|  | Aggregation level |  |  | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size | REG bundle size |
|  | REG bundle size |  |  |  | 6 | 6 |
| DRX |  |  |  |  | OFF | OFF |
| Gap pattern ID |  |  |  |  | [N.A.] | *[gp0] |
| Layer 3 filtering |  |  |  |  | Enabled | Enabled |
| T310 timer |  |  |  | ms | 2000 | 2000 |
| T311 timer |  |  |  | ms | 1000 | 1000 |
| N310 |  |  |  |  | 1 | 1 |
| N311 |  |  |  |  | 1 | 1 |
| NZP CSI-RS configuration |  |  |  |  | TBD | TBD |
| ZP CSI-RS configuration |  |  |  |  | TBD | TBD |
| CSI-IM configuration |  |  |  |  | TBD | TBD |
| Periodic CSI reporting |  |  |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | Config 1, 2 |  | slot | [5] | [5] |
|  |  | Config 3 |  |  | [10] | [10] |
| T1 |  |  |  | s | 0.5 | 0.5 |
| T2 |  |  |  | s | 0.4 | 0.6 |
| T3 |  |  |  | s | [1.46] | [1.36] |
| T4 |  |  |  | s | 0.4 | 0.4 |
| T5 |  |  |  | s | [1] | [1] |
| D1 |  |  |  | s | [0.42] | [0.72] |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

Table A.6.5.1.2.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta |  | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | Config 1 | dB | [1] | [-7] | [-15] | [-4.5] | [1] | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 2 |  | [1] | [-7] | [-15] | [-4.5] | [1] | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 3 |  | [1] | [-7] | [-15] | [-4.5] | [1] | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 1 | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 2 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 3 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.6.5.1.2.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section [A.3.6]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.1.2.1-4: Measurement gap configuration for in-sync tests in non-DRX mode

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [TBD] |
| Note 1: Ensure that RLM RS is partially overlapped with measurement gap |  |

Figure A.6.5.1.2.1-1: SNR variation for in-sync testing

A.6.5.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode

A.6.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

Supported test configurations are shown in table A.6.5.1.3.1-1. The test parameters are given in Tables A.6.5.1.3.1-2, A.6.5.1.3.1-3, A.6.5.1.3.1-4, A.6.5.1.3.1-5, and A.6.5.1.3.1-6. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Editor note: whether to revise power level to be gradually changed

Editor note: whether to revise the SSB configuration to be 2 SSBs and FFS the corresponding power level

Table A.6.5.1.3.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 2 | TDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 3 | TDD, SSB SCS 30 KHz, data SCS 30KHz, BW 40MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.3.1-2: General test parameters for FR1 out-of-sync testing in DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2, 3 |  | TDD |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | [TDDConf.1.1] |
|  |  |  | Config 3 |  | [TDDConf.1.2] |
| CORESET Reference Channel |  |  | Config 1 |  | [CR. 1.1 FDD] |
|  |  |  | Config 2 |  | [CR. 1.1 TDD] |
|  |  |  | Config 3 |  | [CR. 2.1 TDD] |
| SSB Configuration |  |  | Config 1 |  | Table A.3.2.2.1.1-1 |
|  |  |  | Config 2 |  | Table A.3.2.2.1.1-1 |
|  |  |  | Config 3 |  | Table A.3.2.2.1.2-1 |
| SMTC Configuration |  |  | Config 1, 2 |  | Table A.3.2.3.1-1 |
|  |  |  | Config 3 |  | Table A.3.2.3.1-1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |
|  |  |  | Config 3 |  | 30 KHz |
| PRACH Configuration |  |  | Config 1, 2 |  | TBD |
|  |  |  | Config 3 |  | TBD |
| SSB index assigned as RLM RS |  |  |  |  | [0] |
| OCNG parameters |  |  |  |  | Table A.3.2.1.1-1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | [2x2 Low] |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX cycle |  |  |  | ms | 640 |
| Gap pattern ID |  |  |  |  | [N.A.] |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 0 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| NZP CSI-RS configuration |  |  |  |  | TBD |
| ZP CSI-RS configuration |  |  |  |  | TBD |
| CSI-IM configuration |  |  |  |  | TBD |
| Periodic CSI reporting |  |  |  |  | PUCCH |
| CSI reporting periodicity |  | Config 1, 2 |  | slot | [5] |
|  |  | Config 3 |  |  | [10] |
| T1 |  |  |  | s | 1 |
| T2 |  |  |  | s | 7 |
| T3 |  |  |  | s | [7] |
| D1 |  |  |  | s | [6.44] |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.1.3.1-3: Cell specific test parameters for FR1 (Cell 1) for out-of-sync radio link monitoring tests in DRX mode.

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| PDCCH_beta |  | dB | 4 |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |
| PBCH_beta |  | dB | 0 |  |  |
| PSS_beta |  | dB |  |  |  |
| SSS_beta |  | dB |  |  |  |
| PDSCH_beta |  | dB |  |  |  |
| OCNG_beta |  | dB |  |  |  |
| SNR | Config 1 | dB | [1] | [-7] | [-15] |
|  | Config 2 |  | [1] | [-7] | [-15] |
|  | Config 3 |  | [1] | [-7] | [-15] |
|  | Config 1 | dBm/15KHz | [-98] |  |  |
|  | Config 2 |  | [-98] |  |  |
|  | Config 3 |  | [-98] |  |  |
| Propagation condition |  |  | [TDL-C 300ns 100Hz] |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.6.5.1.3.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |

Table A.6.5.1.3.1-5: DRX-Configuration for out-of-sync tests.

| Field | Test 1 |
| --- | --- |
|  | Value |
| drx-onDurationTimer | [ms6] |
| drx-InactivityTimer | [ms1] |
| drx-RetransmissionTimerDL | [sl1] |
| drx-RetransmissionTimerUL | [sl1] |
| longDRX-CycleStartOffset | [ms640] |
| shortDRX | disable |

Table A.6.5.1.3.1-6: DRX-Configuration for out-of-sync tests.

| Field |  | Test 1 |
| --- | --- | --- |
|  |  | Value |
| TimeAlignmentTimer |  | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | Config 1, 2 | [sl5] |
|  | Config 3 | [sl10] |

Figure A.6.5.1.3.1-1: SNR variation for out-of-sync testing

A.6.5.1.3.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode

A.6.5.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

Supported test configurations are shown in table A.6.5.1.4.1-1. The test parameters are given in Tables A.6.5.1.4.1-2, A.6.5.1.4.1-3, A.6.5.1.4.1-4, and A.6.5.1.4.1-5. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Editor note: whether to revise power level to be gradually changed

Editor note: whether to revise the SSB configuration to be 2 SSBs and FFS the corresponding power level

Table A.6.5.1.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 2 | TDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 3 | TDD, SSB SCS 30 KHz, data SCS 30KHz, BW 40MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.4.1-2: General test parameters for FR1 in-sync testing in DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2, 3 |  | TDD |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | [TDDConf.1.1] |
|  |  |  | Config 3 |  | [TDDConf.1.2] |
| CORESET Reference Channel |  |  | Config 1 |  | [CR. 1.1 FDD] |
|  |  |  | Config 2 |  | [CR. 1.1 TDD] |
|  |  |  | Config 3 |  | [CR. 2.1 TDD] |
| SSB Configuration |  |  | Config 1 |  | Table A.3.2.2.1.1-1 |
|  |  |  | Config 2 |  | Table A.3.2.2.1.1-1 |
|  |  |  | Config 3 |  | Table A.3.2.2.1.2-1 |
| SMTC Configuration |  |  | Config 1, 2 |  | Table A.3.2.3.1-1 |
|  |  |  | Config 3 |  | Table A.3.2.3.1-1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |
|  |  |  | Config 3 |  | 30 KHz |
| PRACH Configuration |  |  | Config 1, 2 |  | TBD |
|  |  |  | Config 3 |  | TBD |
| SSB index assigned as RLM RS |  |  |  |  | [0] |
| OCNG parameters |  |  |  |  | Table A.3.2.1.1-1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX cycle |  |  |  | ms | 40 |
| Gap pattern ID |  |  |  |  | [N.A.] |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 2000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| NZP CSI-RS configuration |  |  |  |  | TBD |
| ZP CSI-RS Configuration |  |  |  |  | TBD |
| CSI-IM configuration |  |  |  |  | TBD |
| Periodic CSI reporting |  |  |  |  | PUCCH |
| CSI reporting periodicity |  | Config 1, 2 |  | slot | [5] |
|  |  | Config 3 |  |  | [10] |
| T1 |  |  |  | s | 4 |
| T2 |  |  |  | s | 1.6 |
| T3 |  |  |  | s | [1.36] |
| T4 |  |  |  | s | 0.4 |
| T5 |  |  |  | s | 1.4 |
| D1 |  |  |  | s | [1] |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.1.4.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta |  | dB | 4 |  |  |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |  |  |
| PBCH_beta |  | dB | 0 |  |  |  |  |
| PSS_beta |  | dB |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |
| SNR | Config 1, 4 | dB | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 2, 5 |  | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 3, 6 |  | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 1, 4 | dBm/15 kHz | [-98] |  |  |  |  |
|  | Config 2, 5 |  | [-98] |  |  |  |  |
|  | Config 3, 6 |  | [-98] |  |  |  |  |
| Propagation condition |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.6.5.1.4.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section [A.3.6]. |  |  |  |  |  |  |  |

Table A.6.5.1.4.1-4: DRX-Configuration for in-sync tests

| Field | Test 1 |
| --- | --- |
|  | Value |
| drx-onDurationTimer | [ms6] |
| drx-InactivityTimer | [ms1] |
| drx-RetransmissionTimerDL | [sl1] |
| drx-RetransmissionTimerUL | [sl1] |
| longDRX-CycleStartOffset | [ms40] |
| shortDRX | disable |

Table A.6.5.1.4.1-5: TimeAlignmentTimer -Configuration for in-sync testing

| Field |  | Test 1 |
| --- | --- | --- |
|  |  | Value |
| TimeAlignmentTimer |  | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | Config 1, 2 | [sl5] |
|  | Config 3 | [sl10] |

Figure A.6.5.1.4.1-1: SNR variation for in-sync testing.

A.6.5.1.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 1>