3GPP TSG-RAN WG4 Meeting #90	R4-1901210

Athen, Greece, 25th Feb – 1st Mar, 2019

Title: 	Further discussion on SSB based measurement accuracy

Source: 	Huawei, HiSilicon

Agenda item:	6.13.1

Document for:	Discussion

Introduction

In this contribution, we will provide our simulation results of SS-RSRP measurement accuracy and identify some issues for SSB based measurement accuracy tests.

Discussion

According to the definition in TS38.215, SS-RSRP is defined as the linear averaging power measured over SSS-REs. The SSS sequence generation is only determined by cell ID.

Simulation assumption

Table 1 and Table 2 provide the parameters for SS-RSRP measurement evaluations, which involved two cells. Cell 1 is the measured cell, and Cell 2 is the interfering cell.

Table 1: Simulation parameters for SS-RSRP measurement

| Parameters | Value |  |
| --- | --- | --- |
|  | Sub-6GHz | Above 6GHz |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier Spacing for SS block | 15KHz, 30KHz | 120KHz, 240KHz |
| Bandwidth of SS block | 20 RBs |  |
| Number of transmitted SS block within a SS burst set period (K) | K=1 |  |
| Reference signals | NR-SSS |  |
| Transmission period of SS block | 40ms; |  |
| Number of samples in L1 evaluation period | 5 samples |  |
| Transmit antennas | 1 |  |
| Receive antennas | 2 |  |
| Propagation conditions | AWGN |  |
| CP type | Normal CP |  |

Table 2: Cell-specific parameters for SS-RSRP measurement

| Parameters |  | Cell 1 (interfering cell) | Cell 2 (target cell) |
| --- | --- | --- | --- |
| RF Channel number |  | Channel 1 | Channel 1 |
| Cell ID |  | Case 1: Cell ID = 184 Case 2: Cell ID = 550 | Case 1: Cell ID = 0 Case 2: Cell ID = 0 |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS |  | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS |  | 0 | 0 |
| RB Utilization |  | 100% | 100% |
| Data Modulation |  | QPSK | QPSK |
| Frequency Offset relative to UE frequency reference |  | 0 | 0 |
| Relative Delay of 1st Path (synchronous) |  | CP/2 | 0 |
| Es/Ioc | Setting 1 | 10 dB | 4.5 dB |
|  | Setting 2 | 6 dB | 1 dB |
|  | Setting 3 | 3 dB | -1 dB |
| Es/Iot | Setting 1 | N/A | -6 dB |
|  | Setting 2 | N/A | -6 dB |
|  | Setting 3 | N/A | -6 dB |

Evaluation Results

Based on the simulation assumption, the evaluation results of SS-RSRP measurement are summarized in Table 3.

Table 1:  Delta SS-RSRP measurement accuracy in AWGN

| Case | SINR | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| #1 | Setting 1 | -2.97 | -2.55 | -2.12 |
|  | Setting 2 | -2.88 | -2.29 | -1.71 |
|  | Setting 3 | -2.62 | -1.93 | -1.27 |
| #2 | Setting 1 | 1.95 | 2.14 | 2.33 |
|  | Setting 2 | 1.78 | 2.06 | 2.35 |
|  | Setting 3 | 1.52 | 1.88 | 2.25 |

The SINR levels of the measured cell (Cell 2) in three setting scenarios are all set as -6dB, whereas the interference ratios from Cell 1 are different.

It can be observed that the value of SS-RSRP for Cell 2 is always under-estimated in Case 1 and over-estimated in Case 2, due to the cross-correlation between the SSS sequences from two cells. In case 1, the SS-RSRP measurement accuracy is >2.5dB. Considering the RF margin and implementation margin, the total SS-RSRP measurement accuracy would be larger than 4.5dB, which can’t meet the accuracy requirements defined in TS38.133. The SINR Setting 2 and 3 are both used in SS-RSRP measurement accuracy tests. If the cell IDs with Case 1 were selected, then UE would fail pass to the SS-RSRP measurement accuracy tests.

Observation 1: In sync network, the SS-RSRP estimation will suffer the interference from some specific cells, which can’t be eliminated by time-domain averaging.

In SS-RSRP measurement accuracy tests, at least two cells will be configured, one serving cell and one target cell. The cell identity of target cell is randomly generated for each test loop. Once these specific cell ID combination are selected, then the SS-RSRP measurement accuracy performance will decrease and the UE under test may fail the accuracy tests.

Observation 2: The interference due to specific cell ID combinations may result in the UE failing to pass the SS-RSRP measurement accuracy tests.

Conclusions

This contribution provides the simulation results of SS-RSRP measurements, and the followings are provided:

Observation 1: In sync network, the SS-RSRP estimation will suffer the interference from some specific cells, which can’t be eliminated by time-domain averaging.

Observation 2: The interference due to specific cell ID combinations may result in the UE failing to pass the SS-RSRP measurement accuracy tests.