3GPP TSG-RAN4 Meeting #90	R4-1901211

# Athens, Greece, 25th February – 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6281 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on corrections on SFTD measurements accuracy in TS36.133 R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are remaining open issues in the SFTD measurements accuracy requirements in TS36.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the conditions of NR cell used in SFTD measurements accuracy requirements. - Clarify the minimum Io required for NR cell for SFTD measurements accuracy requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SFTD measurements accuracy requirements are imcompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.27. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.1.27	SFN and frame Timing Difference (SFTD)

9.1.27.1	SFTD Accuracy Requirement

The SFN and frame timing difference (SFTD) is measured between PCell and NR PSCell under EN-DC, or between PCell and NR cell for inter-RAT SFTD. The inter-RAT SFTD measurement can only be configured for E-UTRA - NR band combinations that are supported by the UE.

The accuracy requirements in Table 9.1.27.1-4 are appilicable under the following conditions:

For PCell SFN and frame timing measurement:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in TS 36.101 [5] Clause 7.3 for reference sensitivity are fulfilled.

No changes to the uplink transmission timing are applied during the measurement period.

RSRP|dBm according to Annex B.3.5 for a corresponding Band.

Io range deifined in Table 9.1.27.1-1.

Table 9.1.27.1-1: PCell Io range conditions for SFTD measurement accuracy

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | E-UTRA operating band groups Note 4, 5 | Minimum Io | Maximum Io |
| Conditions |  | dBm/15kHz Note 2, 3 | dBm/BWChannel |
|  | FDD_A, TDD_A | -121 | -50 |
|  | FDD_C, TDD_C | -120 | -50 |
|  | FDD_D | -119.5 | -50 |
|  | FDD_E, TDD_E | -119 | -50 |
|  | FDD_F | -118.5 | -50 |
|  | FDD_G | -118 | -50 |
|  | FDD_H | -117.5 | -50 |
|  | FDD_N | -114.5 | -50 |
| NOTE 1: When in dBm/15kHz, the minimum Io condition is expressed as the average Io per RE over all REs in that symbol. Io may be different in different symbols within a subframe. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: The condition level is increased by MSD as defined in Sections 7.3B in TS 25.214 [20], if applicable depending on E-UTRA – NR band combination. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. NOTE 5: Only E-UTRA bands within EN-DC band combinations as specified in section 5.5B in TS 25.214 [20] are applicable. |  |  |  |

For NR PSCell, or NR cell SFN and frame timing measurement:

[TBD]

Conditions defined in 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled for FR1.

Conditions defined in 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled for FR2.

Io range deifined in Table 9.1.27.1-2 for FR1.

Io range deifined in Table 9.1.27.1-3 for FR2.

Table 9.1.27.1-2: NR PSCell, or NR cell Io range conditions for SFTD measurement accuracy in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 4, 5 | Minimum Io Note 2, 3 |  | Maximum Io |
| Conditions |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in Sections 7.3B in TS 25.214 [20], depending on E-UTRA – NR band combination. NOTE 3: The condition level is increased by MSD as defined in Sections 7.3B in TS 25.214 [20], if applicable depending on E-UTRA – NR band combination. NOTE 4: NR operating band groups are as defined in Section 3.5. NOTE 5: Only NR bands within EN-DC band combinations as specified in section 5.5B in TS 25.214 [20] are applicable. |  |  |  |  |

Table 9.1.27.1-3: NR PSCell, or NR cell Io range conditions for SFTD measurement accuracy in FR2

| Parameter | Io Note 1 Note 3 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 4, 5 | Minimum Io Note 2 |  | Maximum Io |
| Conditions |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |
|  | NR_TDD_FR2_A | -112.5 | -109.5 | -50 |
|  |  | NR_TDD_FR2_B |  | -111 |  | -108 |  | -50 |  |
|  | NR_TDD_FR2_F | -106 | -103 | -50 |
|  |  | NR_TDD_FR2_G |  | -106.5 |  | -103.5 |  | -50 |  |
|  | NR_TDD_FR2_T | -100.5 | -97.5 | -50 |
|  | NR_TDD_FR2_Y | -96 | -93 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in Sections 7.3B in TS 25.214 [20], if applicable depending on E-UTRA – NR band combination. NOTE 3: Io conditions are defined with respective to the input of UE antenna array. NOTE 4: NR operating band groups are as defined in Section 3.5. NOTE 5: Only NR bands within EN-DC band combinations as specified in section 5.5B in TS 25.214 [20] are applicable. |  |  |  |  |

Table 9.1.27.1-4: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot | Frequency range |
| Ts Note 1 | dB |  |
| [40] | [-3] dB | FR1 |
| [40] |  | FR2 |
| NOTE 1: Ts is the basic timing unit defined in TS 36.211 [16]. |  |  |

<End of Change 1>