# 3GPP TSG-RAN WG4 Meeting #90				  												R4-1901338

# Athens, Greece, 25 Feb – 1 Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.141-2 clause 4.9.2 Test models – filling PDCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This draft CR introduces to clause 4.9.2 Test models solution for filling remaining RBs of PDCCH. Currently in 38.141-2 specification in section 4.9.2.2 first two symbols of PDSCH are fill with three PDCCH slots each, rest of slots in these symbols are unoccupied. Such design may cause PAPR issues due to power amplitude statistics. Details are discussed in contribution [1]. [1] R4-1901328 On filling remaining RBs of PDCCH in test models |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDCCH is restricted from two symbols to only first symbol. Second PDCCH is used to occupy filler region in first symbol. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not filling unocuppied region in first two symbols with PDCCH would cause PAPR issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.2.2, 4.9.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 4.9.2	Test models

#### 4.9.2.1	General

The following sections will describe the NR test models needed for BS type 2-O. Note the NR FR1 test models described in TS 38.141-1 [3] are also applicable for BS type 1-O conformance testing.

#### 4.9.2.2	NR FR2 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR- FR2-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-2 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For NR FR2 TDD, test models are derived based on the uplink/downlink configuration as shown in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon.

Table 4.9.2.2-1: Configurations of TDD for BS type 2-O test models

| Field name | Value |  |
| --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 60 | 120 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25 | 1.25 |
| nrofDownlinkSlots | 3 | 7 |
| nrofDownlinkSymbols | 10 | 6 |
| nrofUplinkSlots | 1 | 2 |
| nrofUplinkSymbols | 2 | 4 |

Common physical channel parameters for all NR FR2 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 for PDSCH. Specific physical channel parameters for NR FR2 test models are described in sections 4.9.2.2.1 to 4.9.2.2.3.

Table 4.9.2.2-2: Common physical channel parameters for BS type 2-O PDCCH

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 1 |
| Starting symbol number for control channel | 0 |
| Starting PRB for PDCCH 0 | 0 |
| # of available CCEs | NRB6 |
| Aggregation levels (AL) / number of allocated CCEs (NCCE) and number of allocated PRBs (NRB PDCCH) for PDCCH 0, PDCCH 1(Note 1) and PDCCH 2(Note 1) | NCCE PDCCH0= ALPDCCH0 =arg maxAL ϵ {1, 2,4,8,16}AL * 6≤ NRB NRB PDCCH0 = 6*NCCE PDCCH0 |
|  |  | NCCE PDCCH1 =ALPDCCH1 =arg maxAL ϵ {1, 2,4,8,16}AL * 6≤ NRB-NRB PDCCH0 NRB PDCCH1 = 6 * NCCE PDCCH1 |  |
|  |  | NCCE PDCCH1 =ALPDCCH2 =arg maxALϵ1,2,4,8,16AL * 6≤ NRB-NRB PDCCH0-NRB PDCCH1 NRB PDCCH2 = 6 * NCCE PDCCH2 |  |
| Note 1: Depending on NRB and SCS combination, PDCCH 1 and PDCCH 2 might not exist |  |

Table 4.9.2.2-3: Common physical channel parameters for BS type 2-O PDSCH

| Parameter | Value |
| --- | --- |
| mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | ‘pos2’ |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 0 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |
| PTRS configuration and density | LPT-RS 4 KPT-RS 2 = 00 |
| Ratio of PT-RS EPRE to DM-RS EPRE | 0 dB |

<Next modified section>

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 12

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], subclause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: (NCCE PDCCH0+NCCE PDCCH1+NCCE PDCCH2) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in subclause 4.4.9.2.2.

-	Generate this amount of bits according to ‘all 0’ data.

-	CCEs shall be according to TS 38.211 [20], subclause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH should occupy first  symbolwith the number of resource-element groups which varies depending on the bandwidth and subcarrier spacing.One resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], subclause 7.3.2.3.

-	NID=NIDcellin DM-RS sequence generation in TS 38.211 [20], subclause 7.4.1.3.

-	nRNTI = 0 for NCCE PDCCH0; nRNTI = 100 for NCCE PDCCH100; nRNTI = 200 for NCCE PDCCH2 in CRC scrambling in TS 38.212 [x], subclause 7.3.2

-	Perform mapping to REs according to TS 38.211 [20], subclause 7.3.2.5.

<End of modified section>