3GPP TSG-RAN WG4 Meeting #90 	R4-1901352

Athens, Greece, 25th Feb -  1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5386 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for extending HST-SFN tests to CA in Rel-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_high_speed_enh2-Core |  |  |  |  |  | Date: |  |  | 2019-03-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to introduce the new CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New requirement for TM3 TDD test is added for PDSCH UE demod tests for nw-based CRS-IM. FRC index is updated for both FDD and TDD together with the overview table in Annex updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements with network CRS-IM can’t be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.9, 8.2.2.9, 8.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<start of the change>

#### 8.2.1.9	HST-SFN performance

##### 8.2.1.9.1	Minimum Requirement

The purpose of this test is to verify UE performance in the HST-SFN scenario defined in B.3A when highSpeedEnhancedDemodulationFlag [7] is recieved.

For single carrier, the requirements are specified in Table 8.2.9.1.1-2, with the addition of the parameters in Table 8.2.9.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CCs, the requirements are specified in Table 8.2.9.1.1-5, based on single carrier requirement speicified in Table 8.2.9.1.1-4, with the addition of the parameters in Table 8.2.9.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are speicifed in Table 8.2.9.1.1-6, based on single carrier requirement speicified in Table 8.2.9.1.1-4, with the addition of the parameters in Table 8.2.9.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are speicifed in Table 8.2.9.1.1-7, based on single carrier requirement speicified in Table 8.2.9.1.1-4, with the addition of the parameters in Table 8.2.9.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are speicifed in Table 8.2.9.1.1-8, based on single carrier requirement speicified in Table 8.2.9.1.1-4, with the addition of the parameters in Table 8.2.9.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 6 DL CCs, the requirements are speicifed in Table 8.2.9.1.1-9, based on single carrier requirement speicified in Table 8.2.9.1.1-4, with the addition of the parameters in Table 8.2.9.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 7 DL CCs, the requirements are speicifed in Table 8.2.9.1.1-10, based on single carrier requirement speicified in Table 8.2.9.1.1-4, with the addition of the parameters in Table 8.2.9.1.1-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.1.9.1-1: Test Parameters for UE performance in HST-SFN scenario (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.2.1.9.1-2: Minimum performance UE in HST-SFN scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.87 FDD | OP.1 FDD | HST-SFN | 2x2 | 70 | 13.3 | ≥1 |
| NOTE 1: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

Table 8.2.9.1.1-3: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (NOTE 2) |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. NOTE 3: PUCCH format 1b with channel selection is used to feedback ACK/NACK for tests with 2 DL CCs, PUCCH format 3 is used to feedback ACK/NACK for tests with more than 2 DL CCs. NOTE 4: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.9.1.1-4: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propagation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.87-1 FDD | OP.1 FDD | HST-SFN | 2x2 | 70 | TBD |
| 10MHz | R.87 FDD | OP.1 FDD | HST-SFN | 2x2 | 70 | [13.3] |
| 15MHz | R.87-2 FDD | OP.1 FDD | HST-SFN | 2x2 | 70 | TBD |
| 20MHz | R.87-3 FDD | OP.1 FDD | HST-SFN | 2x2 | 70 | TBD |

Table 8.2.9.1.1-5: Minimum performance (FRC) based on single carrier performance for CA with 2DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 2 | 2x20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 3 | 2x5MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 4 | 10MHz+5MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 5 | 15MHz+5MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. NOTE 3: The OCNG pattern applies for each CC. |  |  |  |

Table 8.2.9.1.1-6: Minimum performance (FRC) based on single carrier performance for CA with 3DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥5 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.9.1.1-7: Minimum performance (FRC) based on single carrier performance for CA with 4DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.2.9.1.1-4 per CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.9.1.1-8: Minimum performance (FRC) based on single carrier performance for CA with 5DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.2.9.1.1-4 per CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.2.9.1.1-4 per CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.2.9.1.1-4 per CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.2.9.1.1-4 per CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.2.9.1.1-4 per CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.2.9.1.1-4 per CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.2.9.1.1-4 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.9.1.1-9: Minimum performance (FRC) based on single carrier performance for CA with 6DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 6x20MHz | As specified in Table 8.2.9.1.1-4 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.9.1.1-10: Minimum performance (FRC) based on single carrier performance for CA with 7DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 7x20MHz | As specified in Table 8.2.9.1.1-4 per CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell, which is not within the same band or sub-block as PCell, is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

<next change>

#### 8.2.2.9	HST-SFN performance

##### 8.2.2.9.1	Minimum Requirement

The purpose of this test is to verify UE performance in the HST-SFN scenario defined in B.3A when highSpeedEnhancedDemodulationFlag [7] is recieved.

For single carrier, the requirements are specified in Table 8.2.2.9.1-2, with the addition of the parameters in Table 8.2.2.9.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 2 DL CCs, the requirements are specified in Table 8.2.2.9.1-5, based on single carrier requirement specified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are specified in Table 8.2.2.9.1-7, based on single carrier requirement specified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.2.2.9.1-8, based on single carrier requirement specified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are speicifed in Table 8.2.2.9.1-9, based on single carrier requirement speicified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 6 DL CCs, the requirements are speicifed in Table 8.2.2.9.1-10, based on single carrier requirement speicified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

For CA with 7 DL CCs, the requirements are speicifed in Table 8.2.2.9.1-11, based on single carrier requirement speicified in Table 8.2.2.9.1-4, with the addition of the parameters in Table 8.2.2.9.1-3 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.2.9.1-1: Test Parameters for UE performance in HST-SFN scenario (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.2.2.9.1-2: Minimum performance UE in HST-SFN scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.87 TDD | OP.1 TDD | HST-SFN | 2x2 | 70 | 13.2 | ≥1 |
| NOTE 1: Test case applicability is defined in 8.1.2.1. NOTE 2: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

Table 8.2.2.9.1-3: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| PDSCH transmission mode |  |  | 3 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. Note 4: PUCCH format 1b with channel selection is used to feedback ACK/NACK for tests with 2 DL CCs, PUCCH format 3 is used to feedback ACK/NACK for tests with more than 2 DL CCs. |  |  |  |

Table 8.2.2.9.1-4: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.87-1 TDD | OP.1 TDD | HST-SFN | 2x2 | 70 | TBD |
| 10MHz | R.87 TDD | OP.1 TDD | HST-SFN | 2x2 | 70 | [13.2] |
| 15MHz | R.87-2 TDD | OP.1 TDD | HST-SFN | 2x2 | 70 | TBD |
| 20MHz | R.87-3 TDD | OP.1 TDD | HST-SFN | 2x2 | 70 | TBD |

Table 8.2.2.9.1-5: Minimum performance (FRC) for CA with 2DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. Note 3: The OCNG pattern applies for each CC. |  |  |  |

Table 8.2.2.9.1-6: Void

Table 8.2.2.9.1-7: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

Table 8.2.2.9.1-8: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.2.2.9.1-4 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.9.1-9: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.2.2.9.1-4 per CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.2.2.9.1-4 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.9.1-10: Minimum performance (FRC) based on single carrier performance for CA with 6 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 6x20MHz | As specified in Table 8.2.2.9.1-4 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

Table 8.2.2.9.1-11: Minimum performance (FRC) based on single carrier performance for CA with 7 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 7x20MHz | As specified in Table 8.2.2.9.1-4 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 |  |  |  |

<next change>

#### 8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity

For dual connectivity the requirements are specified in Table 8.2.3.4-4, based on single carrier requirement specified in Table 8.2.3.4-2 and Table 8.2.3.4-3, with the addition of the parameters in Table 8.2.3.4-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding by using dual connectivity transmission.

Table 8.2.3.4-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for TDD-FDD dual connectivity

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, and 8 for 15MHz CCs and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 for FDD CC 10 or 11 for TDD CC |
| Reporting interval |  | ms | 1 for FDD CC 1 or 4 for TDD CC (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 |
| ACK/NACK transmission |  |  | Separate ACK/NACK feedbacks with PUCCH format 1b on the MCG and SCG |
| CSI feedback |  |  | Separate PUSCH feedbacks on the MCG and SCG |
| Time offset between MCG CC and SCG CC |  | μs | 0 for UE under test supporting synchronous dual connectivity; 334 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 5) |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: The same PDSCH transmission mode is applied to each component carrier. Note 5: As defined in TS36.300 [11]. Note 6: If the UE supports both SCG bearer and Split bearer, the SCG bearer is configured. |  |  |  |

Table 8.2.3.4-2: FDD single carrier performance for multiple dual connectivity configurations

| Bandwidth | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.14-4 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.36 |
| 3MHz | R.14-5 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10 MHz | R.14 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.3.4-3: TDD single carrier performance for multiple dual connectivity configurations

| Bandwidth | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10 MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP. 1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.3.4-4: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Test num. | Bandwidth combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.2.3.4-2 and Table 8.2.3.4-3 per CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectvity configurations and bandwidth combination sets is defined in 8.1.2.3A. |  |  |  |

#### 8.2.3.5	HST-SFN performance

The single-antenna performance in a given multi-path fading environments is determined by the SNR for which a certain relative information bit throughput of the reference measurement channels in Annex A.3.3 is achieved. The purpose of these tests is to verify the single-antenna performance with different channel models and MCS.

##### 8.2.3.5.1	Minimum Requirement for FDD PCell

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.1.1-4 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.1.1-5 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.1.1-6 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.1.1-7 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.1.1-8 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.1.1-9 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.1.1-1: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | 64QAM |
| PDSCH transmission mode |  |  | 3 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.1.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width |  | Reference Channel |  | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration |  | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) |  | SNR (dB) |  |
| 5MHz | R.87-1 FDD |  | OP.1 FDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |  |
| 10MHz | R.87 FDD |  | OP.1 FDD |  | HST-SFN |  | 2x2 |  | 70 |  | [13.3] |  |
| 15MHz | R.87-2 FDD |  | OP.1 FDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |  |
| 20MHz | R.87-3 FDD |  | OP.1 FDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |  |

Table 8.2.3.1.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width |  | Reference Channel |  | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration |  | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) |  | SNR (dB) |  |
| 5MHz |  | R.87-1 TDD |  | OP.1 TDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |
| 10MHz |  | R.87 TDD |  | OP.1 TDD |  | HST-SFN |  | 2x2 |  | 70 |  | [13.2] |
| 15MHz |  | R.87-2 TDD |  | OP.1 TDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |
| 20MHz |  | R.87-3 TDD |  | OP.1 TDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |

Table 8.2.3.1.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any FDD CC. |  |  |  |  |  |

Table 8.2.3.1.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any FDD CC. |  |  |  |  |  |

Table 8.2.3.1.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.1-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.1.1-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

##### 8.2.3.5.2	Minimum Requirement for TDD PCell

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.1.2-4 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 3DL CCs, the requirements are specified in Table 8.2.3.1.2-5 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 4DL CCs, the requirements are specified in Table 8.2.3.1.2-6 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 5DL CCs, the requirements are specified in Table 8.2.3.1.2-7 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 6DL CCs, the requirements are specified in Table 8.2.3.1.2-8 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 7DL CCs, the requirements are specified in Table 8.2.3.1.2-9 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.1.2-1: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | 64QAM |
| PDSCH transmission mode |  |  | 3 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.1.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width |  | Reference Channel |  | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration |  | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) |  | SNR (dB) |  |
| 5MHz | R.87-1 FDD |  | OP.1 FDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |  |
| 10MHz | R.87 FDD |  | OP.1 FDD |  | HST-SFN |  | 2x2 |  | 70 |  | [13.3] |  |
| 15MHz | R.87-2 FDD |  | OP.1 FDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |  |
| 20MHz | R.87-3 FDD |  | OP.1 FDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |  |

Table 8.2.3.1.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width |  | Reference Channel |  | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration |  | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) |  | SNR (dB) |  |
| 5MHz |  | R.87-1 TDD |  | OP.1 TDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |
| 10MHz |  | R.87 TDD |  | OP.1 TDD |  | HST-SFN |  | 2x2 |  | 70 |  | [13.2] |
| 15MHz |  | R.87-2 TDD |  | OP.1 TDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |
| 20MHz |  | R.87-3 TDD |  | OP.1 TDD |  | HST-SFN |  | 2x2 |  | 70 |  | TBD |

Table 8.2.3.1.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any TDD CC. |  |  |  |  |  |

Table 8.2.3.1.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 4 |  |  |  |  |  | 3x20 |  | 2x20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 5 |  |  |  |  |  | 20+20+15 |  | 20+15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 6 |  |  |  |  |  | 20+20+10 |  | 20+10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 7 |  |  |  |  |  | 20+10+10 |  | 2x10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 8 |  |  |  |  |  | 10+15+20 |  | 10 | 15+20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 9 |  |  |  |  |  | 10+15+20 |  | 10+15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any TDD CC. |  |  |  |  |  |

Table 8.2.3.1.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.2-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.1.2-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

<next change>

Table A.3.1.1-1F: Overview of DL reference measurement channels (FDD, PDSCH Performance, Multi-antenna transmission (CRS))

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Two antenna ports |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.2.1-1 | R.10 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.11-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.11-2 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11-3 FDD | 10 | 16QAM | 1/2 | 40 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11-4 FDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.30 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.30-1 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35-1 FDD | 20 | 64QAM | 0.39 | 100 |  | 4 |  |
| FDD | Table A.3.3.2.1-1 | R.35-2 FDD | 15 | 64QAM | 0.39 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35-3 FDD | 10 | 64QAM | 0.39 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.35-4 FDD | 10 | 64QAM | 0.47 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.46 FDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.47 FDD | 10 | 16QAM |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-7 | R.47-1 FDD | 5 | 16QAM |  | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-7 | R.47-2 FDD | 15 | 16QAM |  | 75 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-7 | R.47-3 FDD | 20 | 16QAM |  | 100 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-5 FDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-6 FDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-7 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.11-8 FDD | 10 | QPSK | 3/5 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.11-9 FDD | 10 | QPSK | 0.58 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-10 FDD | 10 | QPSK | 0.67 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.10-2 FDD | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.10-3 FDD | 10 | 16QAM | 0.58 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.65 FDD | 10 | 256QAM | 0.55 | 50 |  | 11-15 |  |
| FDD | Table A.3.3.2.1-3 | R.62 FDD | 10 | 16QAM | 1/2 | 3 |  | 0 |  |
| FDD | Table A.3.3.2.1-3 | R.63 FDD | 10 | 64QAM | 1/2 | 1 |  | 0 |  |
| FDD | Table A.3.3.2.1-4 | R.79 FDD | 10 | 16QAM | 1/2 | 3 |  | M1, M2,≥ 0 |  |
| FDD | Table A.3.3.2.1-5 | R.81 FDD | 10 | QPSK | 1/10 | 6 |  | M1, ≥ 0 |  |
| FDD | Table A.3.3.2.1-6 | R.84 FDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-6 | R.aa FDD | 10 | QPSK | 0.6-0.65 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-6 | R.bb FDD | 10 | 16QAM | 0.3-0.32 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-6 | R.87 FDD | 10 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| FDD | Table A.3.3.2.1-6 | R.87-1 FDD | 5 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| FDD | Table A.3.3.2.1-6 | R.87-2 FDD | 15 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| FDD | Table A.3.3.2.1-6 | R.87-3 FDD | 20 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| FDD | Table A.3.3.2.1-8 | R.90 FDD | 10 | QPSK | 1/3 | 18 |  | M2 |  |
| FDD | Table A.3.3.2.1-8 | R.91 FDD | 10 | QPSK | 1/10 | 18 |  | M2 |  |
| FDD | Table A.3.3.2.1-8 | R.92-1 FDD | 10 | QPSK | 1/2 | 36 |  | ≥1 |  |
| FDD | Table A.3.3.2.1-8 | R.92-2 FDD | 10 | QPSK | 1/2 | 36 |  | ≥1 |  |
| FDD | Table A.3.3.2.1-4 | R.xx FDD | 10 | QPSK | 1/3 | 3 |  | M1, M2 |  |
| FDD | Table A.3.3.2.1-4 | R.yy FDD | 10 | 64QAM | 0.4 | 3 |  | M1, M2 |  |
| Four antenna ports |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.2.2-1 | R.12 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.13 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.14-1 FDD | 10 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-2 FDD | 10 | 16QAM | 1/2 | 3 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-3 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.36 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.14-4 FDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-5 FDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-6 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-7 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-2 | R.72 FDD | 10 | 256QAM | 0.62 | 50 |  | ≥ 11 |  |
| FDD | Table A.3.3.2.2-2 | R.72-1 FDD | 5 | 256QAM | 0.62 | 25 |  | ≥ 11 |  |
| FDD | Table A.3.3.2.2-2 | R.72-2 FDD | 15 | 256QAM | 0.62 | 75 |  | ≥ 11 |  |
| FDD | Table A.3.3.2.2-2 | R.72-3 FDD | 20 | 256QAM | 0.62 | 100 |  | ≥ 11 |  |
| FDD | Table A.3.3.2.2-2 | R.73 FDD | 10 | 64QAM | 0.43 | 50 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-2 | R.74 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-3 | R.74-1 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-3 | R.74-2 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-3 | R.74-3 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-2 | R.85 FDD | 10 | 64QAM | 1/2 | 24 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-2 | R.93 FDD | 10 | 64QAM | 0.52 | 24 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-4 | R.95 FDD | 10 | 16QAM | 1/2 | 3 |  | M2 |  |
| FDD | Table A.3.3.2.2-3 | R.xx1 FDD | 10 | 1024QAM |  | 50 |  | 20, ≥ 22 | UE DL Category |

<next change>

Table A.3.1.1-1I: Overview of DL reference measurement channels (TDD, PDSCH Performance, Multi-antenna transmission (CRS))

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Two antenna ports |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.2.1-1 | R.10 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.11-1 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.11-2 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11-3 TDD | 10 | 16QAM | 1/2 | 40 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11-4 TDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.30 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.30-1 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.30-2 TDD | 20 | 16QAM | 1/2 | 100 |  | 3 |  |
| TDD | Table A.3.4.2.1-1 | R.35 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.35-1 TDD | 20 | 64QAM | 0.39 | 100 |  | 4 |  |
| TDD | Table A.3.4.2.1-2 | R.35-2 TDD | 10 | 64QAM | 0.47 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.46 TDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.47 TDD | 10 | 16QAM |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-9 | R.47-1 TDD | 5 | 16QAM |  | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-9 | R.47-2 TDD | 15 | 16QAM |  | 75 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-9 | R.47-3 TDD | 20 | 16QAM |  | 100 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-5 TDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-6 TDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-7 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-8 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-9 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-10 TDD | 10 | QPSK | 3/5 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-11 TDD | 10 | QPSK | 0.48-0.58 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-12 TDD | 10 | QPSK | 0.54-0.66 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.10-3 TDD | 10 | 16QAM | 0.57-0.58 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-3 | R.62 TDD | 10 | 16QAM | 1/2 | 3 |  | 0 |  |
| TDD | Table A.3.4.2.1-3 | R.63 TDD | 10 | 64QAM | 1/2 | 1 |  | 0 |  |
| TDD | Table A.3.4.2.1-4 | R.65 TDD | 20 | 256QAM | 0.6 | 100 |  | 11-15 |  |
| TDD | Table A.3.4.2.1-5 | R.67 TDD | 10 | 16QAM | 0.4 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-6 | R.79 TDD | 10 | 16QAM | 1/2 | 3 |  | M1, M2, ≥ 0 |  |
| TDD | Table A.3.4.2.1-7 | R.81 TDD | 10 | QPSK | 1/10 | 6 |  | M1, ≥ 0 |  |
| TDD | Table A.3.4.2.1-4 | R.84 TDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-8 | R.aa TDD | 10 | QPSK | 0.54-0.64 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-8 | R.bb TDD | 10 | 16QAM | 0.27-0.32 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-8 | R.87 TDD | 10 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| TDD | Table A.3.4.2.1-8 | R.87-1 TDD | 5 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| TDD | Table A.3.4.2.1-8 | R.87-2 TDD | 15 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| TDD | Table A.3.4.2.1-8 | R.87-3 TDD | 20 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| TDD | Table A.3.4.2.1-10 | R.90 TDD | 10 | QPSK | 1/3 | 18 |  | M2 |  |
| TDD | Table A.3.4.2.1-10 | R.91 TDD | 10 | QPSK | 1/10 | 18 |  | M2 |  |
| TDD | Table A.3.4.2.1-10 | R.92-1 TDD | 10 | QPSK | 1/2 | 36 |  | ≥1 |  |
| TDD | Table A.3.4.2.1-10 | R.92-2 TDD | 10 | QPSK | 1/2 | 36 |  | ≥1 |  |
| TDD | Table A.3.4.2.1-6 | R.xx TDD | 10 | QPSK | 1/3 | 3 |  | M1, M2 |  |
| TDD | Table A.3.4.2.1-6 | R.yy TDD | 10 | 64QAM | 0.4 | 3 |  | M1, M2 |  |
| Four antenna ports |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.2.2-1 | R.12 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.13 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.14 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.14-1 TDD | 10 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.14-2 TDD | 10 | 16QAM | 1/2 | 3 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥2 |  |
| TDD | Table A.3.4.2.2-1 | R.36 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.43-1 TDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-2 TDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-3 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-4 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.43-5 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-2 | R.72 TDD | 10 | 256QAM | 0.62 | 50 |  | ≥ 11 |  |
| TDD | Table A.3.4.2.2-2 | R.72-1 TDD | 5 | 256QAM | 0.62 | 25 |  | ≥ 11 |  |
| TDD | Table A.3.4.2.2-2 | R.72-2 TDD | 15 | 256QAM | 0.62 | 75 |  | ≥ 11 |  |
| TDD | Table A.3.4.2.2-2 | R.72-3 TDD | 20 | 256QAM | 0.62 | 100 |  | ≥ 11 |  |
| TDD | Table A.3.4.2.2-2 | R.73 TDD | 10 | 64QAM | 0.44 | 50 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-2 | R.74 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-3 | R.74-1 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-3 | R.74-2 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-3 | R.74-3 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-2 | R.85 TDD | 10 | 64QAM | 1/2 | 24 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-2 | R.93 TDD | 10 | 64QAM | 0.50 | 24 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-4 | R.95 TDD | 10 | 16QAM | 1/2 | 3 |  | M2 |  |

<next change>

Table A.3.3.2.1-6: Fixed Reference Channel two antenna ports

| Parameter | Unit |  | Values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  | R.87 FDD | R.84 FDD | R.aa FDD | R.bb FDD | R.87-1 FDD |  | R.87-2 FDD | R.87-3 FDD |  |
| Channel bandwidth |  | MHz |  | 10 | 10 | 10 | 10 | 5 |  | 15 | 20 |  |
| Allocated resource blocks (Note 4) |  |  |  | 50 | 39 | 50 | 50 | 25 |  | 75 | 100 |  |
| Allocated number of PDCCH symbols |  |  |  | 2 | 2 | 2 | 2 | 2 |  | 2 | 2 |  |
| Allocated subframes per Radio subframes |  |  |  | 8 | 9 | 9 | 9 | 8 |  | 8 | 8 |  |
| Modulation |  |  |  | 64QAM | 16QAM | QPSK | 16QAM | 64QAM |  | 64QAM | 64QAM |  |
| Target Coding Rate |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |  | 0.39 | 1/2 | 0.6 | 0.3 | 0.39 |  | 0.39 | 0.39 |  |
| For Sub-Frames 5 |  |  |  | N/A | N/A | N/A | N/A | N/A |  | N/A | N/A |  |
| For Sub-Frames 0 |  |  |  | N/A | N/A | 0.65 | 0.32 | N/A |  | N/A | N/A |  |
| Information Bit Payload (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  | Bits |  | 15264 | 9912 | 7992 | 7992 | 7736 |  | 22920 | 30576 |  |
| For Sub-Frames 5 |  | Bits |  | N/A | N/A | N/A | N/A | N/A |  | N/A | N/A |  |
| For Sub-Frames 0 |  |  |  | N/A | 9912 | 7992 | 7992 | N/A |  | N/A | N/A |  |
| Number of Code Blocks(Notes 3 and 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  |  |  | 3 | 2 | 2 | 2 | 2 |  | 4 | 5 |  |
| For Sub-Frames 5 |  |  |  | N/A | N/A | N/A | N/A | N/A |  | N/A | N/A |  |
| For Sub-Frames 0 |  |  |  | N/A | 2 | 2 | 2 | N/A |  | N/A | N/A |  |
| Binary Channel Bits (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  | Bits |  | 39600 | 20592 | 13200 | 26400 | 19800 |  | 59400 | 79200 |  |
| For Sub-Frames 5 |  |  |  | N/A | N/A | N/A | N/A | N/A |  | N/A | N/A |  |
| For Sub-Frames 0 |  |  |  | N/A | 20592 | 12384 | 24768 | N/A |  | N/A | N/A |  |
| Max. Throughput averaged over 1 frame (Note 4) |  | Mbps |  | 12.211 | 8.9208 | 7.1928 | 7.1928 | 6.1888 |  | 18.336 | 24.461 |  |
| UE Category |  |  |  | ≥ 1 | 1bis | ≥ 1 | ≥ 2 | ≥ 1 |  | ≥ 1 | ≥ 1 |  |
| UE DL Category |  |  |  | ≥6 | N/A | ≥6 | ≥6 | ≥6 |  | ≥6 | ≥6 |  |
| Note 1: Void Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Given per component carrier per codeword. Note 5: For R.84 FDD, 39 RBs are allocated on RB 0-20 and 30-47. |  |  |  |  |  |  |  |  |

<next change>

Table A.3.4.2.1-8: Fixed Reference Channel two antenna ports

| Parameter | Unit | Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  | R.87 TDD | R.aa TDD | R.bb TDD | R.87-1 TDD |  | R.87-2 TDD | R.87-3 TDD |
| Channel bandwidth |  |  | MHz | 10 | 10 | 10 | 5 |  | 15 | 20 |
| Allocated resource blocks (Note 4) |  |  |  | 50 | 50 | 50 | 25 |  | 75 | 100 |
| Uplink-Downlink Configuration (Note 2) |  |  |  | 1 | 1 | 1 | 1 |  | 1 | 1 |
| Allocated number of PDCCH symbols |  |  |  | 2 | 2 | 2 | 2 |  | 2 | 2 |
| Allocated subframes per Radio Frame (D+S) |  |  |  | 2+2 | 3+2 | 3+2 | 2+2 |  | 2+2 | 2+2 |
| Modulation |  |  |  | 64QAM | QPSK | 16QAM | 64QAM |  | 64QAM | 64QAM |
| Target Coding Rate |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  |  |  | 0.39 | 0.61 | 0.3 | 0.39 |  | 0.39 | 0.39 |
| For Sub-Frames 1,6 |  |  |  | 0.36 | 0.54 | 0.27 | 0.39 |  | 0.39 | 0.39 |
| For Sub-Frames 0 |  |  |  | N/A | 0.64 | 0.32 | N/A |  | N/A | N/A |
| For Sub-Frames 5 |  |  |  | N/A | N/A | N/A | N/A |  | N/A | N/A |
| Information Bit Payload (Note 4) |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  |  | Bits | 15264 | 7992 | 7992 | 7736 |  | 22920 | 30576 |
| For Sub-Frames 1,6 |  |  | Bits | 11448 | 5736 | 5736 | 6456 |  | 19080 | 25456 |
| For Sub-Frames 0 |  |  | Bits | N/A | 7992 | 7992 | N/A |  | N/A | N/A |
| For Sub-Frames 5 |  |  |  | N/A | N/A | N/A | N/A |  | N/A | N/A |
| Number of Code Blocks(Notes 3 and 4) |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  |  |  | 3 | 2 | 2 | 2 |  | 4 | 5 |
| For Sub-Frames 1,6 |  |  |  | 2 | 2 | 2 | 2 |  | 2 | 2 |
| For Sub-Frames 0 |  |  |  | N/A | 2 | 2 | N/A |  | N/A | N/A |
| For Sub-Frames 5 |  |  |  | N/A | N/A | N/A | N/A |  | N/A | N/A |
| Binary Channel Bits (Note 4) |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  |  | Bits | 39600 | 13200 | 26400 | 19800 |  | 59400 | 79200 |
| For Sub-Frames 1,6 |  |  |  | 31968 | 10656 | 21312 | 16200 |  | 48168 | ## 64368 |
| For Sub-Frames 0 |  |  |  | N/A | 12528 | 25056 | N/A |  | N/A | N/A |
| For Sub-Frames 5 |  |  |  | N/A | N/A | N/A | N/A |  | N/A | N/A |
| Max. Throughput averaged over 1 frame (Note 4) |  |  | Mbps | 5.342 | 3.5448 | 3.5448 | 2.8384 |  | 8.4 | 11.206 |
| UE Category |  |  |  | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 1 |  | ≥ 1 | ≥ 1 |
| Note 1: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4] Note 2: As per Table 4.2-2 in TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Given per component carrier per codeword. |  |  |  |  |  |  |  |

<end of change>