3GPP TSG-RAN WG4 Meeting #90	R4-1901439

Athens, Greece, 25 February – 1 March 2019

Source:	Ericsson

Title:	TP to TR 37.716-21-21: Addition of EN-DC configuration DC_3A_n20A-n78A

Agenda item:	9.8.2

Document for:	Approval

# Background

This contribution provides text proposal on DC_3A_n20A-n78A as defined in the revised WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_3_n20-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| DC_3_n20-n78 | 3 | 1710 MHz -– 1785 MHz | 1805 MHz –1880 MHz | FDD |
|  | n20 | 832 MHz -– 862 MHz | 791 MHz – 821 MHz | FDD |
|  | n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3_n20-n78 | DC_3_n20 DC_3_n78 | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 140 |
|  |  | n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |

### 6.x.3	Co-existence studies

Co-existence studies of Band 3 + Band n20 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

### 6.x.4	∆TIB and ∆RIB values

For DC_ the TIB,c and RIB,c values are derived from CA_3-20-42 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n20-n78 | 3 | 0.6 |
|  | n20 | 0.3 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n20-n78 | 3 | 0.2 |
|  | n20 | 0 |
|  | n78 | 0.5 |

### 6.x.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_3_n20-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3_n20-n78 | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3420 | 10 | 52 | 3420 | 16.1 | TDD | IMD3 |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3500 | 10 | 52 | 3500 | 4.5 | TDD | IMD5 |

##### ---End of changes---

# Reference

[1] 		RP-182302, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]		R4-1814769,	”TR 37.716-21-21 v0.2.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16”, LG Electronics